| Set 1: Blink | Student Edition |
| --- | --- |

After completing the activities, you should be able to …

- build a closed circuit

- switch on and off devices through digital output via microprocessors

- write simple programs by sequence and iteration structures

- solve problems through programming

Required Components

Each student should have:

- Arduino UNO board × 1

- USB AB cable × 1

- Breadboard × 1

- Red LED × 1

- Blue LED × 1 (for extension task)

- Resistor × 1 (2 for extension task)

- DuPont cables × 4

Think about…

- Question 1: How to light up a bulb or lamp?

- Question 2: In you daily life, list some examples of blinking lights?

- Question 3: How to make a light blink?

- Question 4: Have you ever seen the street lights switched on and off by workers? If not, how do these street lights being switched on and off?

List of Components

| Arduino UNO board | Breadboard |
| --- | --- |
| Resistor | DuPont cables (Electric cables) |
| LED | USB cable |

Notice

- The components are not toys, place them on the table and follow teacher’s instructions.

- Some components are sharp, beware of being punctured.

Activity 1: Light up LED

Objective

Building a closed circuit to light up a LED.

Simulation Activity

Use Circuits on Tinkercad (https://www.tinkercad.com/circuits) to simulate the following activity.

Required components:

| LED | Resistor | Breadboard |
| --- | --- | --- |
| Arduino UNO R3 board |  |  |

- Drag the Arduino UNO R3 board from the component panel to the stage.

Question: Study the Arduino board, find pins for anode and cathode.

Answer:

- Using the components to construct the following connections. 
Select existing wire, its colour of the wire can be modified from the dialog box on the right.

- Start the simulation, check if the LED lights up.

USB cable is plugged into USB port of Arduino UNO board

LED lights up

Arduino UNO ON LED lights up

- Stop the simulation.

How Electricity Lights Up the LED?

|  | Electricity flows into the _________________ through the ____________. |  | Electricity flows into the ____________ through the _____ pin of the Arduino UNO board. |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Electricity flows into the ____________ through the ______________, then flows into the _________ of the LED, then flows into the ____________ through the _________ of the LED. The LED lights up when electricity passes through. |  |  |  | Electricity flows into ______ pin of the Arduino UNO board through the ______________. |

Challenging Question Why resistor is required? Try to simulate on Circuits on Tinkercad as follows.

Answer:

Physical Connection

Try to establish the connection according to the simulation, beware of the anode and cathode of the LED.

Anode (Longer)      Cathode

Checkpoint 2

Checkpoint 4

Checkpoint 3

Checkpoint 1

Can the LED light up?	  Yes      No  Please check the followings.

Does the USB cable connect to the Arduino UNO board? (Checkpoint 1)

Does the LED connect to the breadboard correctly? (Checkpoint 2)

Do two electric wires connect correctly? (Checkpoint 3 & 4)

Activity 2: Blinking LED

Objective

Making LED blink (turn on and off) through programming.

Simulation Activity

Use Circuits on Tinkercad to simulate the following activity.

- Connection

Detach the red wire from 5V pin of Arduino UNO board and connect to digital pin 7.

- Algorithm Design

Target: Make the LED blink

How to make the LED blink once? _________________________ What is the time interval? _________________________ Complete the algorithm (steps for solving the problem). How to light up the LED? Refer to the diagram in Step (1), the electricity passes the LED through the “digital pin 7”. How to turn on and off the LED by programming? - Turn on the LED: Set Digital Pin 7 to LOW/HIGH - Turn off the LED: Set Digital Pin 7 to LOW/HIGH Try to modify the above algorithm.

- Coding

- Start the Code panel.

- Write the program according to the algorithm in Step (2).

- Start the simulation, check if the LED blinks.

- End the simulation.

Physical Connection

- Modify the physical connection in Activity 1, i.e. detach the red wire from 5V pin of Arduino UNO board and connect to digital pin 7, and make sure the USB cable is connected to the computer.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Drag the following programming blocks into the scripts area.

- Using the programming blocks to construct the program, then test the program by clicking the green flag icon.

- Try to modify the program to increase the blinking speed of the LED.

(Extended Activity)

Activity 3: Blinking Two LEDs

Objective

Making two LEDs blink through programming.

Simulation Activity

Use Circuits on Tinkercad to simulate the following activity extended from Activity 2.

- Using similar approach of the red LED connection, connect another blue LED () with its anode and cathode connected to the digital pin 4 () and another GND pin () respectively.

- Try to write the programs according to the blinking pattern of two LEDs as follows.

| Requirement | Blinking pattern of two LEDs | Looping Procedures |
| --- | --- | --- |
| 1 | Two LEDs blinking simultaneously (On and Off at the same time) |  |
| 2 | Two LEDs blinking alternately (One LED on while another off) |  |
| 3 | Each LED blinking three times alternately |  |

Question: Try to simplify the procedures of Requirement 3 by using looping structure.

Answer:

Physical Connection

- Modify the physical connection in Activity 2, add the blue LED and make sure the USB cable is connected to the computer.

- Use mBlock to construct the program and test the result.

Autodesk screen shots reprinted courtesy of Autodesk, Inc.

mBlock is a graphical programming environment based on Scratch Open Source Code by Makeblock Co., Ltd.