| Set 1: Blink | Teacher Edition |
| --- | --- |

Key Points

- Students should be able to build a closed circuit

- Students should be able to switch on and off devices through digital output via microprocessors

- Students should be able to write simple programs by sequence and iteration structures

- Students should be able to solve problems through programming

Required Components

Each student should have:

- Arduino UNO board × 1

- USB AB cable × 1

- Breadboard × 1

- Red LED × 1

- Blue LED × 1 (for extension task)

- Resistor × 1 (2 for extension task)

- DuPont cables × 4

Think about…

Teacher Recommendation Teachers are encouraged to discuss with students to introduce the lesson objectives.

- Question 1: How to light up a bulb or lamp?

(Suggested answers) connect with power / provide power / supply electricity

- Question 2: In you daily life, list some examples of blinking lights?

(Suggested answers) Traffic light / Xmas decorations / ambulance

- Question 3: How to make a light blink?

(Suggested answers) Consecutive on and off

- Question 4: Have you ever seen the street lights switched on and off by workers? If not, how do these street lights being switched on and off?

(Free answers)

List of Components

Teacher Recommendation Teacher can introduce the names of components, including their purposes if necessary.

| Arduino UNO board | Breadboard |
| --- | --- |
| Resistor | DuPont cables (Electric cables) |
| LED | USB cable |

Notice

- The components are not toys, place them on the table and follow teacher’s instructions.

- Some components are sharp, beware of being punctured.

Activity 1: Light up LED

Objective

Building a closed circuit to light up a LED.

Simulation Activity

Use Circuits on Tinkercad (https://www.tinkercad.com/circuits) to simulate the following activity.

Required components:

| LED | Resistor | Breadboard |
| --- | --- | --- |
| Arduino UNO R3 board |  |  |

- Drag the Arduino UNO R3 board from the component panel to the stage.

Question: Study the Arduino board, find pins for anode and cathode.

Answer: Anode - 3.3V、5V, Cathode - GND

- Using the components to construct the following connections. 
Select existing wire, its colour of the wire can be modified from the dialog box on the right.

- Start the simulation, check if the LED lights up.

USB cable is plugged into USB port of Arduino UNO board

LED lights up

Arduino UNO ON LED lights up

- Stop the simulation.

How Electricity Lights Up the LED?

|  |  | Electricity flows into the Arduino UNO board through the USB wire. |  | Electricity flows into the red wire through the 5V pin of the Arduino UNO board. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Electricity flows into the resistor through the red wire, then flows into the anode of the LED, then flows into the black wire through the cathode of the LED. The LED lights up when electricity passes through. |  |  |  | Electricity flows into GND pin of the Arduino UNO board through the black wire. |  |
| Challenging Question Why resistor is required? Try to simulate on Circuits on Tinkercad as follows. |
| Answer: If no resistor is connected, the current will be too large and damage the LED. |  |  |  |  |  |

Physical Connection

Try to establish the connection according to the simulation, beware of the anode and cathode of the LED.

Anode (Longer)      Cathode

Checkpoint 2

Checkpoint 4

Checkpoint 3

Checkpoint 1

Can the LED light up?	  Yes      No  Please check the followings.

Does the USB cable connect to the Arduino UNO board? (Checkpoint 1)

Does the LED connect to the breadboard correctly? (Checkpoint 2)

Do two electric wires connect correctly? (Checkpoint 3 & 4)

Activity 2: Blinking LED

Objective

Making LED blink (turn on and off) through programming.

Simulation Activity

Use Circuits on Tinkercad to simulate the following activity.

- Connection

Detach the red wire from 5V pin of Arduino UNO board and connect to digital pin 7.

- Algorithm Design

Target: Make the LED blink

How to make the LED blink once? Turn on LED Turn off LED What is the time interval? 1 second (Accept other suggestions) Complete the algorithm (steps for solving the problem). Turn on the LED Wait for 1 second Turn off the LED Wait for 1 second How to light up the LED? Refer to the diagram in Step (1), the electricity passes the LED through the “digital pin 7”. How to turn on and off the LED by programming? - Turn on the LED: Set Digital Pin 7 to LOW/HIGH - Turn off the LED: Set Digital Pin 7 to LOW/HIGH Try to modify the above algorithm. Set Digital Pin 7 to HIGH Wait for 1 second Set Digital Pin 7 to LOW Wait for 1 second

- Coding

- Start the Code panel.

Teacher Recommendation If students do not have experience on visual programming (such as Scratch), teacher can introduce the basic operations briefly.

- Write the program according to the algorithm in Step (2).

- Start the simulation, check if the LED blinks.

- End the simulation.

Physical Connection

- Modify the physical connection in Activity 1, i.e. detach the red wire from 5V pin of Arduino UNO board and connect to digital pin 7, and make sure the USB cable is connected to the computer.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Drag the following programming blocks into the scripts area.

- Using the programming blocks to construct the program, then test the program by clicking the green flag icon.

Suggested answer

- Try to modify the program to increase the blinking speed of the LED.

Suggested answer

Teacher Recommendation Teacher can use the above program to introduce Sequence and Looping programming structures. - Sequence: The lighting order of LED (Off and then on vs. On and then off) - Looping: The cycle of on and off to create blinking effect

(Extended Activity)

Activity 3: Blinking Two LEDs

Objective

Making two LEDs blink through programming.

Simulation Activity

Use Circuits on Tinkercad to simulate the following activity extended from Activity 2.

- Using similar approach of the red LED connection, connect another blue LED () with its anode and cathode connected to the digital pin 4 () and another GND pin () respectively.

- Try to write the programs according to the blinking pattern of two LEDs as follows.

| Requirement | Blinking pattern of two LEDs | Looping Procedures |
| --- | --- | --- |
| 1 | Two LEDs blinking simultaneously (On and Off at the same time) | Turn on the red LED (Set Digital Pin 7 to HIGH) Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second |
| 2 | Two LEDs blinking alternately (One LED on while another off) | Turn on the red LED (Set Digital Pin 7 to HIGH) Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second |
| 3 | Each LED blinking three times alternately | Red LED blinking once Turn on the red LED (Set Digital Pin 7 to HIGH) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Wait for 1 second Turn on the red LED (Set Digital Pin 7 to HIGH) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Wait for 1 second Turn on the red LED (Set Digital Pin 7 to HIGH) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Wait for 1 second Blue LED blinking once Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second |

Suggested answer

| Requirement | Blinking pattern of two LEDs | Looping Procedures |
| --- | --- | --- |
| 1 | Two LEDs blinking simultaneously (On and Off at the same time) |  |
| 2 | Two LEDs blinking alternately (One LED on while another off) |  |
| 3 | Each LED blinking three times alternately |  |

Question: Try to simplify the procedures of Requirement 3 by using looping structure.

Answer: Repeat 3 times Turn on the red LED (Set Digital Pin 7 to HIGH) Wait for 1 second Turn off the red LED (Set Digital Pin 7 to LOW) Wait for 1 second Repeat 3 times Turn on the blue LED (Set Digital Pin 4 to HIGH) Wait for 1 second Turn off the blue LED (Set Digital Pin 4 to LOW) Wait for 1 second.

Physical Connection

- Modify the physical connection in Activity 2, add the blue LED and make sure the USB cable is connected to the computer.

- Use mBlock to construct the program and test the result.

Teacher Recommendation Teacher can provide extended activities according to students’ level, e.g. - Increase the number of LEDs - Change the blinking pattern - Task-based assignment, such as traffic light for pedestrians

Autodesk screen shots reprinted courtesy of Autodesk, Inc.

mBlock is a graphical programming environment based on Scratch Open Source Code by Makeblock Co., Ltd.