| Set 2: Application of Sensors | Student Edition |
| --- | --- |

(The connection diagrams of this activity set are created by Fritzing)

After completing the activities, you should be able to …

- build circuit with sensor(s)

- use the data captured from the sensor(s)

- switch on and off devices through digital output via microprocessors

- write simple programs by sequence, selection and iteration structures

- to control different objects through programming by using the collected data from the sensor(s)

- to solve problems through programming

Required Components

Students should be provided with the following items.

- Arduino UNO board × 1

- USB AB cable × 1

- Breadboard × 1

- Ultrasonic Sensor × 1

- Water Sensor × 1 (for extension task)

- Red LED × 1

- Resistor × 1

- DuPont cables × 4

- A tissue and little water (for extension task)

Think about…

- Question 1: Have you ever seen electric lamps without switches? How to turn them on and off?

- Question 2: Have you ever seen electric appliances with automatic tuning according to the environment? What kind of data do they collect?

List of Components

| Arduino UNO board | Breadboard |
| --- | --- |
| Resistor | DuPont cables (Electric cables) |
| LED | USB cable |
| Ultrasonic Sensor | Water Sensor |

Notice

- The components are not toys, place them on the table and follow teacher’s instructions.

- Some components are sharp, beware of being punctured.

Activity 1: Collect Data from Ultrasonic Sensor

Objective

Collect and output the reading from the ultrasonic sensor.

Physical Connection

- Construct the following circuit.

| Ultrasonic Sensor | Arduino UNO board |
| --- | --- |
| Vcc | 5V |
| Trig | 7 |
| Echo | 6 |
| Gnd | Gnd |

Question: The eyesight of bats is poor. How do they know the distance of the obstacles at the front?

Answer:

- The physical connection is shown below.

- Connect computer and Arduino UNO board by the USB cable.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Drag the following programming blocks into the scripts area. Try to construct a program to let mBlock sprite to speak out the reading from the ultrasonic sensor.

- Try to place an object in front of the ultrasonic sensor and execute the program.

Question: What is the reading? Why?

Answer:

- Try to modify the program and test it again.

Question: What is the reading? Try to move the object, does the reading change? Why?

Answer:

- Try to modify the program and test it again.

Question: What is the reading? What is the unit of the reading?

Answer:

Question: Suggest a scenario that ultrasonic sensor is suitable for measurement of distance.

Answer:

Activity 2: Simple Collision Alert

Objective

Through the collected data from the ultrasonic sensor, write a simple program for collision alert.

Physical Connection

- Using the connection in Activity 1, add a red LED in the circuit according to the learning from Activity Set 1. (Anode at digital pin 12, cathode at GND pin)

- Design the algorithm according to the requirement (problem to be solved).

Target: Collision Alert

How to identify collision will occur soon? _____________________________________________________________________________________ _____________________________________________________________________________________ According to previous activities, how to present the alert? ___________________________________ Complete the algorithm (steps for solving the problem) accordingly. If _______________________________ (Condition) ______________________ (Instruction(s) to be executed). Does the above algorithm require “forever” loop? Why? _____________________________________________________________________________________ _____________________________________________________________________________________ Try to modify and complete the algorithm.

- Use mBlock to construct the program and test the result.

When the object moves closer to the front of the ultrasound sensor to a distance less than the 			specified value, does the LED blink?

When the object moves beyond the specified distance in front of the ultrasonic sensor, does the 		LED stop blinking?

This program _____________________________ to control the _____________________________.

Question: According to this system, list out its input, process and output.

Answer:

Question: Suggest an application for this system.

Answer:

(Extended Activity)

Activity 3: Collect Data from Water Sensor

Objective

Collect and output the reading from the water sensor.

Physical Connection

- Construct the following circuit.

| Water sensor | Arduino UNO board |
| --- | --- |
| + / V | 5V |
| – / G | Gnd |
| S | A0 |

- The physical connection is shown below.

- Connect computer and Arduino UNO board by the USB cable.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Modify Activity 1 so as to let mBlock sprite to speak out the reading from the water sensor.

- Execute the program, test it according to water level and record the corresponding reading.

| Water level | Water sensor |  | Reading |
| --- | --- | --- | --- |
| Nil | No water is contacted |  |  |
| Low | A drop of water on the detection area of the water sensor |  |  |
| Medium | Few drops of water on the detection area of the water sensor |  |  |
| High | The detection area of water sensor is immersed into a bottle of water. |  |  |

(Extended Activity)

Activity 4: Humidity Alert

Objective

Through the collected data from the water sensor, write a simple program for humidity alert.

Physical Connection

- Using the connection of Activity 3 (or Activity 2), adding a red LED in the circuit and make sure the USB cable is connected to the computer.

- Use mBlock to construct the program and test the result by different water levels.

This program uses _____________________________ to control the _____________________________.

Question: According to this system, list out its input, process and output.

Answer:

Question: Suggest an application for this system.

Answer:

Fritzing (http://fritzing.org) screen shots reprinted courtesy of FH Potsdam and Friends-of-Fritzing foundation.

mBlock is a graphical programming environment based on Scratch Open Source Code by Makeblock Co., Ltd.