| Set 2: Application of Sensors | Teacher Edition |
| --- | --- |

(The connection diagrams of this activity set are created by Fritzing)

Key Points

- Students should be able to build circuit with sensor(s)

- Students should be able to use the data captured from the sensor(s)

- Students should be able to switch on and off devices through digital output via microprocessors

- Students should be able to write simple programs by sequence, selection and iteration structures

- Students should be able to control different objects through programming by using the collected data from the sensor(s)

- Students should be able to solve problems through programming

Required Components

Students should be provided with the following items.

- Arduino UNO board × 1

- USB AB cable × 1

- Breadboard × 1

- Ultrasonic Sensor × 1

- Water Sensor × 1 (for extension task)

- Red LED × 1

- Resistor × 1

- DuPont cables × 4

- A tissue and little water (for extension task)

Think about…

Teacher Recommendation Teachers are encouraged to discuss with students to introduce the lesson objectives.

- Question 1: Have you ever seen electric lamps without switches? How to turn them on and off?

(Suggested answers) Clap activated / Sound activated

- Question 2: Have you ever seen electric appliances with automatic tuning according to the environment? What kind of data do they collect?

(Suggested answers)

Lamp with intensity tuning according to indoor brightness level (Collected data: Brightness)

Air conditioner with temperature tuning according to room temperature (Collected data: Temperature)

(Free answers)

List of Components

Teacher Recommendation Teacher can introduce the names of components, including their purposes if necessary.

| Arduino UNO board | Breadboard |
| --- | --- |
| Resistor | DuPont cables (Electric cables) |
| LED | USB cable |
| Ultrasonic Sensor | Water Sensor |

Notice

- The components are not toys, place them on the table and follow teacher’s instructions.

- Some components are sharp, beware of being punctured.

Activity 1: Collect Data from Ultrasonic Sensor

Objective

Collect and output the reading from the ultrasonic sensor.

Teacher Recommendation As the ultrasonic sensor in Circuits on Tinkercad is different from those available in the markets, it is not suggested to conduct simulation activity. To let students familiarising with Arduino connection, it is suggested to complete Activity Set 1 first.

Physical Connection

- Construct the following circuit.

| Ultrasonic Sensor | Arduino UNO board |
| --- | --- |
| Vcc | 5V |
| Trig | 7 |
| Echo | 6 |
| Gnd | Gnd |

Question: The eyesight of bats is poor. How do they know the distance of the obstacles at the front?

Answer: Through emission of ultrasonic wave. Echo is generated when the wave hits the obstacle. The distance can be estimated according to the travelling time of the echo.

Teacher Recommendation Teacher can discuss the use of ultrasonic sensor with students, according to the above question related to bat. There are four pins in the ultrasonic sensor (): Vcc and Gnd are used as anode and cathode for electricity transmission. Trig (Trigger) is used to send signal in form of high-frequency sound and Echo is used to receive the reflected signal, this is similar to the way of distance estimation of bat. Transmitter locates on the left side of the ultrasonic sensor, marked by T (). Receiver locates on the right side of the ultrasonic sensor, marked by R (). They can be regarded as the mouth (vocal cord) and ear of bats.

- The physical connection is shown below.

- Connect computer and Arduino UNO board by the USB cable.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Drag the following programming blocks into the scripts area. Try to construct a program to let mBlock sprite to speak out the reading from the ultrasonic sensor.

Suggested answer

- Try to place an object in front of the ultrasonic sensor and execute the program.

Question: What is the reading? Why?

Answer: Reading is 0 or No reading Because trig pin should be 7 and echo pin should be 6

- Try to modify the program and test it again.

Question: What is the reading? Try to move the object, does the reading change? Why?

Answer: (Any positive number) The reading does not change Because the program request to output the reading once. It does not request to output the reading continuously, so it does not reflect the change of distance.

- Try to modify the program and test it again.

Suggested answer

Question: What is the reading? What is the unit of the reading?

Answer: (Any positive number) Centimetre (CM)

Teacher Recommendation If time is allowed, teacher can let the students to justify the unit of the reading.

Question: Suggest a scenario that ultrasonic sensor is suitable for measurement of distance.

Answer: Long distance to measure / Distance is not easily measured by ruler (or other possible answers)

Activity 2: Simple Collision Alert

Objective

Through the collected data from the ultrasonic sensor, write a simple program for collision alert.

Physical Connection

- Connection

Using the connection in Activity 1, add a red LED in the circuit according to the learning from Activity Set 1. (Anode at digital pin 12, cathode at GND pin)

Suggested answer

- Algorithm Design

Target: Collision Alert

How to identify collision will occur soon? When the ultrasonic sensor is close to the object in the front, e.g. 5 cm According to previous activities, how to present the alert? Blink (Activity Set 1) Complete the algorithm (steps for solving the problem) accordingly. If Reading from ultrasonic sensor is less than 5 (Condition) blink the LED (Instruction(s) to be executed). Does the above algorithm require “forever” loop? Why? It is required, because the distance between the ultrasonic sensor and object ahead is changing, the program needs to handle the changes. Try to modify and complete the algorithm. Suggested answer Forever If Reading from ultrasonic sensor is less than 5 Set Digital Pin 12 to HIGH (Pin number according to connection) Wait for 0.2 second (Self-defined duration) Set Digital Pin 12 to LOW Wait for 0.2 second.

Teacher Recommendation Teacher can introduce the concept of reuse through copying the program in Activity Set 1. - Reuse: Using the program develop in Activity Set 1 on blinking LED

- Coding

Use mBlock to construct the program and test the result.

When the object moves closer to the front of the ultrasound sensor to a distance less than the 			specified value, does the LED blink?

When the object moves beyond the specified distance in front of the ultrasonic sensor, does the 		LED stop blinking?

Suggested answer

This program uses the distance of the front object to control the blinking of the LED lamp.

Question: According to this system, list out its input, process and output.

Answer: Input: Reading collected from the ultrasound sensor Process: Compare the reading and defined value (5 cm) Output: Blink LED

Question: Suggest an application for this system.

Answer: Parking alert (or other reasonable answers)

(Extended Activity)

Activity 3: Collect Data from Water Sensor

Objective

Collect and output the reading from the water sensor.

Teacher Recommendation As the water sensor is absent in Circuits on Tinkercad, it is not suggested to conduct simulation activity. To let students familiarising with Arduino connection, it is suggested to complete Activity Set 1 first.

Physical Connection

- Construct the following circuit.

| Water sensor | Arduino UNO board |
| --- | --- |
| + / V | 5V |
| – / G | Gnd |
| S | A0 |

- The physical connection is shown below.

- Connect computer and Arduino UNO board by the USB cable.

- Run mBlock application in the computer, connect the computer to Arduino UNO board (Connect > Serial Port > Select COM port (usually the largest COM port)), and make sure it is connected.

- Modify Activity 1 so as to let mBlock sprite to speak out the reading from the water sensor.

Suggested answer

Teacher Recommendation Teacher can introduce the concept of reuse and remix by modifying the program in Activity 1. - Reuse: Use the program in Activity 1 - Remix: Modify the program for the new requirement (From reading of ultrasonic sensor to water sensor)

- Execute the program, test it according to water level and record the corresponding reading.

| Water level | Water sensor |  | Reading |
| --- | --- | --- | --- |
| Nil | No water is contacted |  |  |
| Low | A drop of water on the detection area of the water sensor |  |  |
| Medium | Few drops of water on the detection area of the water sensor |  |  |
| High | The detection area of water sensor is immersed into a bottle of water. |  |  |

Teacher Recommendation Teacher can introduce the water sensor briefly, the signal is generated as water increases the electric current.

(Extended Activity)

Activity 4: Humidity Alert

Objective

Through the collected data from the water sensor, write a simple program for humidity alert.

Physical Connection

- Using the connection of Activity 3 (or Activity 2), adding a red LED in the circuit and make sure the USB cable is connected to the computer.

Teacher Recommendation Students make use of Activity 2 and Activity 3, reuse and remix the connection. - Using Activity 2: Change from ultrasonic sensor to water sensor - Using Activity 3: Add a red LED

Suggested answer

- Use mBlock to construct the program and test the result by different water levels.

Suggested answer

This program uses the water level or amount of water to control the blinking of the LED lights.

Question: According to this system, list out its input, process and output.

Answer: Input: Reading collected from the water sensor Process: Compare the reading and defined value (400) Output: Blink LED

Question: Suggest an application for this system.

Answer: Rain alarm / Automatic watering (or other reasonable answers)

Teacher Recommendation Teacher can design other extended tasks according to students’ level, e.g. - Compare the changes in programs under different requirements (e.g. rain alarm vs. automatic watering) - Extended tasks - Different levels of alert, LED blinks slower when the water level is lower and blinks faster when the water level is higher - Connect other hardware as an alarm tool, such as a buzzer

Fritzing (http://fritzing.org) screen shots reprinted courtesy of FH Potsdam and Friends-of-Fritzing foundation.

mBlock is a graphical programming environment based on Scratch Open Source Code by Makeblock Co., Ltd.