Technology Education Section

Curriculum Development Institute

Education Bureau

The Government of the HKSAR

May 2019

All comments and suggestions related to the resource materials may be sent to:

Chief Curriculum Development Officer (Technology Education)

Technology Education Section

Curriculum Development Institute

Education Bureau

Room W101, West Block, 19 Suffolk Road

Kowloon Tong

Hong Kong

The copyright of the materials in this package, other than those listed in the Acknowledgments section and the photographs mentioned there, belongs to the Education Bureau of the Government of the Hong Kong Special Administrative Region.

Schools and educational organisations are welcome to use the content of this package for non-profit making educational purposes.  In all cases, proper acknowledgements should be made.

Otherwise, all rights are reserved, and no part of these materials may be used for publication or other purposes in any form without the prior permission of the Education Bureau.

© Copyright 2019

The learning resources is developed by the Robot Institute of Hong Kong.

Content

| Chapter 1: Basics of Robotics | P.3 |
| --- | --- |
| Chapter 2: What Is Arduino? | P.7 |
| Chapter 3: Arduino IDE (Software) | P.11 |
| Chapter 4: Control (I) – LED Control | P.17 |
| Chapter 5: Control (II) – Buzzer | P.26 |
| Chapter 6: Control (III) – Motor | P.33 |
| Chapter 7: Unit Project – Lie Detector | P.39 |

Basic Unit 1

Basic control of electronic components

Chapter 1: Basics of Robotics

What is a robot?

When it comes to robots, many people will think of humanoid machines with hands and feet.  However, this kind of machines tends to appear only in science fiction movies, entertainment venues, exhibitions and toy stores. They are very different from industrial robots.

Industrial robots (IRs), sometimes called robotic arms, can perform simple actions such as up-and-down motions, and grab and pick out components from machines. However, industrial robots can also perform more complicated tasks such as transportation, gripping, targeting, assembly and inspection.

The definition of a robot

A robot is officially defined by the Robot Institute of America in 1979 as 'a re-programmable, multi-functional manipulator designed to move materials, parts, tools, or special devices through variable programmed motions for the performance of a variety of tasks'.  As long as a machine satisfies the criteria, it can be called a robot even if it does not have a 
human form.

An industrial robot, which is usually referring to a robotic arm, consists of several links connected in series by linear, rotary or prismatic joints.  At one end, the robot is fixed to a supporting base, and the other end is equipped with a tool and manipulated into position to perform tasks.

Computer science

Autonomous judgment

Electronic engineering

Sensing, driving and calculating

Mechanical engineering

Interaction with the environment

Nowadays, robots are becoming popular for entertainment purposes and even for innovative technologies, such as those related to human life, animals and military purposes.

Therefore, a robot can also be defined as 'a human made semi- or fully autonomous (self-controlled) object or cooperating objects (with common objectives) with intelligence that is programmable'.

Design of industrial robots

Industrial robots are made up of six basic constituent elements, which are: the dynamic system, end-of-arm tools, computerised digital controller, actuators, feedback devices and sensors.

The following diagram shows the inter-relationship between these six elements:

Dynamic system

Microcontroller

Micro Junior Secondary 
Robotics Learning and Teaching Resources

Basic Unit 1: Basic control of electronic components

Targets

Program

Sensor

Feedback

Robotic arm

Linkages

(Equivalent to the human arm)

End-of-arm tool

(Equivalent to the human hand)

Rotational base

(Equivalent to the human body)

Joints

(Equivalent to the human joint)

Initially we upload the program to the microcontroller, which will then react to give corresponding outputs, for example to the robotic arm and the end effector. The microcontroller controls the various joints and the rotational base flexibly to perform high-precision actions and specific work goals. Then, the sensors of the robot will return some readings to the microcontroller. Finally, the microcontroller acts accordingly to achieve the work goals.

Development of robots and advancement of robotics

The beginning of modern robots.  Artificial intelligence laboratories were set up by advanced international universities and the first computer-controlled robotic arm was born.

The era of mass production of robots.  Different countries began to design robots themselves and microprocessors were invented.

The intelligence of robot has been developed to a level where robots can perform intelligent activities such as playing chess and interacting with animals.

Robots were no longer limited to the industrial area and have penetrated into all aspects of society, such as military use, entertainment, and imitation of humans and animals.

Robots have become popular, and advanced technologies have been developed, such as the da Vinci surgical system, the robots sent to Mars by NASA for exploration, and the open source project Arduino.

Before 1950, robots were only characters in science fictions. 1960 marks the beginning of modern robots.  Artificial intelligence laboratories were set up by advanced international universities such as the Massachusetts Institute of Technology. The first computer-controlled robotic arm was born during this period of time. From 1970, the world officially entered the era of mass production of robots. Different countries, especially those in Europe, began to design robots themselves. Microprocessors were also invented during this period of time. In 1980, scientists made a breakthrough to the bottleneck of robot development, where robots were developed to possess their own intelligence such that they can play chess or interact with animals. In 1990, robots were no longer limited to the industrial area and penetrated into all aspects of society, such as military use, entertainment, and imitation of humans and animals. In 2000, robots have become popular, and advanced technologies have been developed, such as the da Vinci surgical system, the robots sent to Mars by NASA for exploration, and the birth of the open source project Arduino.

Review questions

- What is a robot?

- What elements does robotics include?

- What are the basic components of the design of a robot?

- Can you describe the entire operation of a robot?

- Can you give an overview of the development of robots?

Chapter 2: What Is Arduino?

- Understanding microcontrollers

Simply speaking, a microcontroller, MCU, is indeed a tiny computer containing a processor, memory, input/output units and other peripheral devices that is installed in a single integrated circuit. However, a microcontroller does not have a built-in operating system and thus it cannot operate like a computer. Programs need to be developed on other computer systems before they can be input to it.

- Various microcontrollers

The core of most early microcontrollers adopts Atmel AVR series developed by Atmel in 1996, which is a family of 8-bit to 32-bit automatic control reduced instruction set microcontrollers. AVR uses flash memory as the data storage medium for its single-chip microcontrollers, as opposed to one-time programmable ROM, EPROM, or EEPROM used by other microcontrollers at the time. The following are a few common microcontrollers.

| Arduino UNO The microcontroller that will be used in this teaching resource. The entry barrier for this development board is low, and is easy for non-professional people to learn and use. |  |
| --- | --- |
| Intel Galileo Famous computer processor manufacturer Intel has also developed microcontrollers. The characteristic of this development board is that it has pin positions compatible with Arduino UNO, and can be programmed with Arduino IDE. |  |
| Nucleo 64 The Nucleo development board consists of a 32-bit STMicroelectronics microprocessor, which has a higher processing power than Arduino, but is harder to use in comparison with Arduino. |  |
| Udoo Dual This is a development board that can load Windows 10 IoT. It is more powerful than Arduino UNO but more difficult to program. |  |
| BeagleBone Black This is a microcontroller development board produced by Texas Instruments in association with Digi-Key and Newark element14, which was released in 2011. |  |

What is Arduino?

Arduino was an assignment given to students of the Ivrea Interaction Design Institute in Italy in 2005, aiming to provide a low-cost, easy-to-use interface for students, amateurs or professionals to design interactive devices that consist of sensors and drivers.

Arduino is a computer control board that uses an Atmel AVR single-chip microcontroller. It can be connected to various input and output devices, and can be connected to a variety of communication modules. With suitable control programs, a wide range of automatic control applications can be made, for example, controlling the rotation speed of a fan according to the temperature, controlling the brightness of a light or the speed of a motor with a variable resistor, remote controlling home appliances with infrared or Bluetooth, controlling a robotic arm or robot with servo motors, and producing automatic vehicles and aircraft.

The most prominent feature of Arduino is that both its software and hardware are open-source. Traditionally, it is a prerequisite for the developer to possess a background in disciplines such as electronic or electrical engineering in order to develop a microcontroller program, which is not at all easy for ordinary people to take part in. Arduino has a low entry barrier, and is easy for ordinary people to learn and use. Furthermore, there are a lot of Arduino resources on the Internet to which people can refer, and as a result, they can complete their own works in a short time by adjusting the design according to their needs.

Circuit design diagrams of Arduino can be downloaded from the Internet, and there are reasonably priced Arduino control boards available for sale on the Internet.

- Arduino control boards (hardware)

Programming by connecting to a computer via USB

Arduino can be connected to a computer with the universal USB connection to facilitate users to write programs.

Online programming

Arduino uses an online programmer to load the boot loader to the control panel. It guides the operating system to initiate the program, which has the following advantages:

- Simplify the programming process

- Initiate when running the user's program

- Inspect the solution on the development platform

The following diagram shows the various components on an Arduino control board, and the various input and output pins available.

Reset button

ICSP pin

SCL

SDA

Digital output and input pins

ICSP pin

ATmega328

16 MHz quartz crystal

Analogue input pins (can also be used as digital output and input pins)

7 ~ 12 V power input socket

DC voltage regulation module

ATmega16U2

USB connector

- Review question

- What is Arduino UNO?

Chapter 3: Arduino IDE (Software)

- Program installation and connection between the hardware and the software

Arduino IDE is the programming platform and environment for Arduino. A lot of different programming software can be used for Arduino but Arduino IDE that can be downloaded from arduino.cc will be focused in this teaching resource.

Arduino IDE can be downloaded from www.arduino.cc

Arduino installation file

- Arduino IDE layout

1. Layout introduction

Message area

Toolbar

Code writing area

2. Descriptions of the toolbar buttons:

Serial Monitor: Opens the serial monitor for communicating with the Arduino via the serial port.

New: Creates a new sketch (a program written using Arduino IDE).

Save: Saves your sketch.

Verify: Checks your code for errors before compiling it.

Open: Presents a menu of all the sketches in your sketchbook.

Upload: Compiles your code and uploads it.

- Do it yourself – Understanding Arduino & Arduino IDE

This section is for students to familiarise themselves with the operation of the Arduino control board and Arduino IDE. For the computer to communicate with the Arduino control board, we must use the serial port.  However, modern computers generally use USB ports and thus cannot communicate with the Arduino control board directly. To handle this, we need to use a USB-to-serial chip, commonly used types including ATmega8U2, CH340, CP2102, FTDI232R and PL2303. The official version of the Arduino control board contains an ATmega16U2 USB-to-serial chip, which can facilitate us to use the computer to communicate with the Arduino control board and burn programs.

ATmega16U2

This exercise uses the on-board LED of the Arduino, and does not need to connect to external units.

- Open Arduino examples - LED Blink: Click File  Examples  01.Basics  Blink.

- The following program will appear after clicking Blink.

This program consists of two main parts.  In the first part, the code inside the setup( ) function configures the Arduino's on-board LED as an output (OUTPUT).  In the second part, the code inside the loop( ) function turns the on-board LED on and off continuously.

- Then plug in the USB cable to connect the Arduino control board with the computer.

- After connecting to the computer, we must first open the Control Panel of the computer > Device Manager to find the Communications Port (also known as Serial Port) of the Arduino board.  The figure below shows the Communications Port (COM9) of the Arduino UNO found in the Device Manager.

- In Arduino IDE  Tools Board:  select Arduino UNO development board.

- In Arduino IDE, select the Communications Port of the Arduino UNO found in the Device Manager above.

- Press the Upload button. The program will be compiled and uploaded to the Arduino UNO development board.

- When the upload is done, you should see the Arduino's on-board LED begins to blink.

- Review questions

- What is Arduino IDE?

- How to connect the Arduino control board to the computer, and make them communicate?

Chapter 4: Control (I) – LED Control

- Light-emitting diode, LED

A light-emitting diode is the most basic and commonly used output device. Its illuminating property can be used to indicate the status, for instance, the lighting up of a green LED and a red LED can be used to indicate a qualified condition and an unqualified condition respectively. In addition, it can be used for program testing.  As a program is executed from top to bottom, if we would like to test whether a part of commands has been executed, we can add some codes under that part of commands for flashing an LED. If the LED flashes, it means that part of commands has been executed. Furthermore, we can test the strength of a signal. For example, we can use the received signal as the brightness of the LED. Then we can judge the signal strength with the naked eye. We can use LEDs and programming to carry out all of the above functions

Anode (+)

Cathode

( - )

An LED is a kind of unidirectional hardware, i.e. we need to connect it according to the current of a circuit; otherwise, it will not light up.  An LED usually has two leads, one long and one short. The long one is called the anode and the short one is called the cathode. The direction of the current should flow from the anode to the cathode.

Since LEDs have a small size, do not heat up and are more robust, in contrast to traditional light bulbs that are solely used for lighting, LEDs are widely used as the signal indicators of electronic products. In this section, the LEDs we are going to use are commonly seen small LEDs with a range of colour choices such as red, orange, yellow, green and blue.

- Different parts of an Arduino program

Content between /* and */ is the remark written by the programmer which will not be executed.

Setup(): a function that will be executed once. Pin 13 is configured as the output pin here.

Content after // will not be executed also.

Loop(): a function that will be executed over and over again until the power is cut off.

- Different parts of a function

Different parts of a function written in C/C++ programming language.

Output of the function: 'void' means no output

Name of the function

Input of the function: () means no input

All commands of a function need to be placed between { and }

The command that a function needs to complete should end with a ';', e.g. a = a + b;

pinMode(13, OUTPUT);

pinMode – calls the function to define the mode of the pin.  The function first specifies the pin number (1 ~ 13 or A0 ~ A5), followed by the action (INPUT, OUTPUT or INPUT_PULLUP).  INPUT: input; OUTPUT: output; INPUT_PULLUP: enables the pull-up resistor of this pin and provides a stable 5V voltage.

digitalWrite(13, HIGH);

digitalWrite – Calls the function to define the pin as a digital output to be written with a 5V or 0V value.  The function first specifies the pin number (1 ~ 13 or A0 ~ A5), followed by the write action (LOW or HIGH).  HIGH: high electric potential (5V); LOW: low electric potential (0V).

delay(1000);

Delay – Calls the function to define a delay time of 1000 ms (must use milliseconds (ms) as the unit).

Throughout the programming process, we will need to record some data, which may be text or numbers. We record these data at an address in the memory and give them names. These are variables.  The following are some variables commonly used in Arduino programs.

| Type | Memory size | Value |
| --- | --- | --- |
| boolean | 8 bit (1 byte) | 1 or 0 |
| byte | 8 bit (1 byte) | 0 ~ 255 |
| char | 8 bit (1 byte) | -128 ~ 127 |
| int | 16 bit (2 bytes) | -32768 ~ 32767 |
| long | 32 bit (4 bytes) | -2147483648 ~ 2147483647 |
| float | 32 bit (4 bytes) | ±3.4 e±38 |

- Do it yourself – LED blink

The first task of this chapter is to try to control the blinking of an LED with the Arduino. There are two parts in this task, which are to connect the Arduino with the LED and to program the Arduino to control the LED.

1  Connecting the Arduino and the LED

We connect the LED, resistor and the Arduino as follows.

220 ohm resistor

Not many electronic parts are required for this task and the connection method is very simple. First, we attach a red LED to a breadboard.  Note that the leads of an LED are different in lengths. The longer one is the anode and the shorter one is the cathode. To light up an LED, the current must flow from the anode to the cathode.  Therefore, when the LED is on, the anode must be at a high potential and the cathode must be at a low potential.   In this task, the anode and cathode of the LED are connected to pin 13 and GND (ground) of the Arduino board respectively. A resistor is connected between the LED and the Arduino board for limiting the current flowing through the LED, so that there will not be overcurrent affecting the operation of the LED.

5 connected nodes in a vertical column

25 connected nodes in a horizontal row

Understanding a breadboard:

A breadboard allows connection of electronic components used in a circuit with no soldering required.

1. Power rails: The breadboard shown in the above figure has two rows of holes at the top and bottom respectively.  They are generally used for providing power supply. The row marked with '+' at the top has 25 interconnected holes and all of them are positive pole. The second row at the top marked with '–' is for connecting to the ground. The first and second rows at the bottom have the same configuration as those at the top of the breadboard.

2. Terminal strips: The connection holes on the breadboard shown in the above figure are divided into upper and lower parts, which are the main working area for connecting the electronic components and jumper wires.  The five holes in the same vertical column (i.e. a-b-c-d-e or f-g-h-i-j) are interconnected. However, there is no connection between the vertical columns (i.e. 1-30), and between the upper and lower parts of the ravine (i.e. e-f).

2  Programming the Arduino to control the LED

Open Arduino IDE and connect the Arduino to the computer.  Select Arduino UNO as the development board in the Tools menu and check whether the Communications Port is connected to the Arduino already.

After connecting the Arduino, we need to input the following program and upload it to the Arduino.

The complete program codes.  Refer to file 1_4_1_led_flash.

Define the variable

Define the pin for the LED

Set the blinking frequency of the LED

3  Program content explanation

- Define the variable

The program is divided into three parts.  In the first part, we define variable 'ledPin' to be an integer 13.  An integer is one of the data types in a program.  To define an integer variable, we need to add 'int' before the variable.

- Define the pin for the LED

In the second part, we define the pin of the Arduino that is connected to the LED. In the previous part, variable 'ledPin' is defined as integer 13. In this part, we use 'pinMode ()' to define pin 13 as an output pin.

- Set the blinking frequency of the LED

Assuming that the LED blinks once every two seconds, that means the time the LED is set to turn on and off is one second each. We use 'digitalWrite()' in the program to control the LED: when 'digitalWrite' is set as HIGH, the LED turns on; when 'digitalWrite' is set as LOW, the LED turns off.  'delay()' is used to pause the program for a specific amount of time (in milliseconds). There are 1000 milliseconds in a second.

- When the program is uploaded to the Arduino UNO, the red LED on the breadboard will turn on for one second and off for the next, and repeat over and over.

- Do it yourself – Traffic light

The second task of this chapter is very similar to the first one. In this task, we will try to control three LEDs of different colours to light up one after another, imitating the traffic lights we often see in our daily life.

1  Connecting the Arduino and the LEDs

We connect the LED, resistor and the Arduino as follows.

The way to connect the LEDs in this task is the same as that in the first task. The anodes of the LEDs are connected to the Arduino and the cathodes are connected to GND. Since there are three LEDs to be controlled in this task, we need to use three pins on the Arduino to connect the LEDs, which are pins 11, 12 and 13.

2  Programming the Arduino to control the traffic light

Open Arduino IDE and connect the Arduino to the computer. Select Arduino UNO as the development board in the Tools menu and check whether the Communications Port is connected to the Arduino already. After connecting the Arduino, we need to input the following program and upload it to the Arduino.

The complete codes of the traffic light program.  Refer to file 1_4_2_traffic_light.

Define the variables

Define the pins for the LEDs

Set the blinking sequence and frequencies of the LEDs

3  Program content explanation

- Define the variables

We define three variables in this part, namely, 'ledPin_R', 'ledPin_Y' and 'ledPin_G. The three variables store the integers 13, 12 and 11 respectively.

- Define the pins for the LEDs

In the second part, we define the pins of the Arduino to which the LEDs will connect. We use 'pinMode()' to define pins 11, 12 and 13 as the output pins.

- Set the blinking sequence and frequencies of the LEDs

The sequence for the LEDs to blink depends on the colours of the LEDs and is set to be in the order of red, yellow and green. The duration for each LED to light up is one second. When an LED is turned on, the LED that turns on previously will turn off. We use 'delay(1000)' to control the on and off duration of the LEDs, and 'digitalWrite()' to control the order in which the LEDs are turned on and off.

Review questions

- What is an LED?

- What are pinMode, int x = 3;?

Chapter 5: Control (II) – Buzzer

- Sound-producing objects and sound

In addition to emitting light, it is also common for electronic software to generate sounds. Sound-producing objects can generally be categorised into two types: speaker and buzzer. A buzzer is small and flexible.  However, since its sound quality is poorer, it is more suitable to be used in alarm devices or devices that produce sounds for drawing attention. A small speaker is larger than a buzzer. Making use of the vibration of a paper film, the sound quality of speakers is higher than that of buzzers. Therefore, speakers are more suitable for electronic products that need to produce precise pitches such as robots, electronic keyboards and mechanical assistants. The following diagram shows a buzzer.

The main component of a buzzer is the diaphragm. The diaphragm consists of a thin copper plate. When a current is applied, electromagnetic force will be produced to make the copper plate vibrate. Sound is caused by vibration, and the frequency of the vibration is called audio frequency. With a formula, we can find out the notes on conventional music scale that various audio frequencies refer to. Before writing a music program, let's review some fundamental music knowledge.

- Pitch and beat

The level of audio frequency is called pitch. In music, we use Do, Re, Mi and other phonetic names to represent pitches of various frequencies. The keyboard on a piano is arranged according to the audio frequency, in the order of musical scales such as C, D and E. The frequency of a specific pitch is exactly twice of that in the previous octave (note: the 8th white key to the right of the middle C 'Do' is the 'Do' an octave above middle C 'Do').

Frequency is a standard of vibration measurement.  Sound is a waveform and each sound has its specific frequency.  The sharper the sound, the higher the frequency; the deeper the sound, the lower the frequency.

Time

Low frequency

High frequency

Deeper sound

Sharper sound

The task in this chapter is to use new syntax to control a buzzer, including 'for' loop and array.

- 'for' loop

To get started, let's understand the syntax of a 'for' loop:

for（initial variable; conditional statement; count statement）{

statement 1;

statement 2;

}

When setting the loop, the first thing we should decide is exactly how many times we want the loop to repeat.  Suppose we want to repeat three times, example: for（byte i = 0 ; i < 3 ; i++）{  }. In the first loop, the variable i has a value of zero. When the first loop is executed completely, the variable i will increase by one and the second loop will be executed. Finally, the variable i will increase by one again and the third loop will be executed. As i needs to be smaller than 3, the loop stops after the third loop is completed.

From the loop syntax, we know that there are four parts we need to set in a 'for' loop. The first thing is to write 'for'. Then we need to set the initial value of the variable, which is usually set as 0. After representing the variable, we need to set the execution condition. In the example, we set the condition that the variable is smaller than 3, so that the loop can repeat three times. Finally, we need to set the method for determining the number of repetition. We know that if the variable is equal to 3, the loop will stop. If the variable is incremented by one each time, a total of 3 times will be run from 0 to 3. So we set the variable to be incremented by one each time. Eventually, write the program we need to execute repeatedly to finish the loop.

In the previous paragraph, we mention that the initial value is usually set as 0, but there are exceptions, for example, when we want to define the LEDs connected to pin 8 to pin 10 in turn. We can define the pins as output in the program, then we can use variable i to handle the logic. During the process, the pin number to be defined is incremented each time starting from 8 and ending at 10. In the first run, the variable is 8; in the second run, the variable becomes 9; in the third run, the variable becomes 10. In this way, we can reduce the size of the program and avoid repeating the same program codes.

void setup() {

for ( byte i = 8 ; i <= 10 ; i++ ) {

pinMode ( i, OUTPUT );

}

}

- Array

Before discussing array, let's first find out how a variable stores a value. In this part, we can think of variable as a box. To define a variable, we are giving a name to the box. Suppose an undefined variable is an empty box.  After we define it, the box will store a value. In normal circumstances, one of these boxes can store only one value. If we need to store 10 values in a program, we need to write 10 statements; if we need to store 100 variables, we need to declare 100 times. Such a program is very inefficient. To handle such situations, we can use an array, i.e. to define a certain amount of similar boxes in one time. For instance, the following syntax means that the box can store 4 integers.

int Nums[4];（data type array name[array size] ;）

| Nums | [0] | [1] | [2] | [3] |
| --- | --- | --- | --- | --- |

- Do it yourself – Buzzer

In this chapter, we will try to use Arduino to control a buzzer to play music. The task consists of two parts: connecting the Arduino and the buzzer, and programming the Arduino to play music with the buzzer.

1  Connecting the Arduino and the buzzer

We connect the buzzer and the Arduino as follows.

The way to connect the buzzer to the Arduino is very simple and straightforward. As the buzzer has high resistance itself, no extra electronic component is required when using the buzzer. The buzzer also has an anode and a cathode. The anode and cathode are usually marked with a red wire and a black wire respectively. We need to connect the anode and cathode of the buzzer to pin 11 and GND of the Arduino respectively.

2  Programming the Arduino to play music with the buzzer

Open Arduino IDE and connect the Arduino to the computer. Select Arduino UNO as the development board in the Tools menu and check whether the Communications Port is connected to the Arduino already. After connecting the Arduino, we need to input the following program and upload it to the Arduino.

The complete music playing program codes.  Refer to file 1_5_play_music.

Use a loop to play the song

Define the variables and arrays

Define the functions

Define the pin for the buzzer

3  Program content explanation

- Define the variables and arrays

We define three variables and two arrays in this part. The three variables are 'buzzerPin', 'notes_length' and 'tempo'.  'buzzerPin' stores an integer of 11, representing the pin number of the Arduino to which the buzzer is connected.  'notes_length' stores an integer of 25, representing the total number of notes in the song.  'tempo' stores an integer of 300, representing the length of a beat. The two arrays are 'notes[]' and 'beats[]'.  'notes[]' stores different English letters, which represent the notes in the song.  'beats[]' stores the lengths of the notes. When the value of the tempo is multiplied by each item in the array beats[], the result obtained is the time each note is played.

- Define the functions

We define two functions, namely 'playTone()' and 'playNote()', in this part. Function 'playNote()' is responsible for setting the frequency of each note according to the English letters in array 'notes[]'. Function 'playTone()' is responsible for playing the song. Using 'tone()', we can play the song with the buzzer connected to the pin defined in 'buzzerPin' according to the frequencies calculated in function 'playNote()'; using 'noTone()', we can stop the buzzer from playing the song. In the 'for' loop, the number of loops is set according to the length of the song, and the song is played according to the length of the music scales.

- Define the pin for the buzzer

In this part, we use 'pinMode()' to define pin 11 set in 'buzzerPin' as the output pin.

- Use a loop to play the song

In the fourth part, we use a 'for' loop from 0 to 25 to play all the notes defined in 'notes[]' once in the program. There is an if-else condition set in this loop. The if condition is to check whether there are blank spaces in array 'notes[]'. If a blank space occurs, the program will treat it as a rest; if it is not a blank space, the program will treat it as a normal note. At the end of the program, there is a waiting of 'delay(1000)', which functions as a pause between songs, making it easier for us to listen to the song.

- Review questions

- What is a 'for' loop?

- What is an array?

- What is a buzzer?

- What are tone(), noTone()?

Chapter 6: Control (III) – Motor

- Understanding direct current (DC) motors

Coil

Rotational shaft

Permanent magnet

There are many types of motors. The one used in this chapter is the DC motor, also known as the yellow motor.  The indispensable parts of a motor include a permanent magnet and a coil. A magnetic field will be generated when power is supplied to the coil, and the permanent magnet in the middle will rotate according to the magnetic field. The same current will cause the motor to turn in the same direction, allowing the motor to rotate.  Following this logic, if the power supply is connected in reverse, the motor will rotate in the reverse direction.

There are only two connections on a yellow motor or a DC motor, one for power supply and the other for connecting to the ground. However, the servo motor we will user later, which provides high stability, may have four to six connection wires for setting the number of turns or angle of rotation of the motor. Such highly precise operations are typically used for industrial control, such as robotic arms and photocopiers.

Except electrical appliances such as electric fans, hair dryers and power drills that connect the motor to the load (example: fan blades) directly, most power devices use mechanical systems such as gearboxes and pulleys to reduce the motor speed or convert the direction of the power output to increase the torque.

The H-bridge motor control circuit for controlling the forward and backward rotation of a motor

From the introduction of motors above, we understand that if we want to reverse the rotation direction of a motor, we need to reverse the flow of the current. However, in real situations, we cannot control a robot in this way. Therefore, we need to have an additional motor driver attached to the motor for changing the rotation direction of the motor. This motor driver is known as an H-bridge motor controller. With this control method, we can see from the above figure that the motor is surrounded by four switches. The operation method is as follows: in the first condition, when switches 2 and 3 are closed, the motor will have a leftward rotation; in the second condition, when switches 1 and 4 are closed, the motor will have a rightward rotation.

Do it yourself – Motor control

This chapter expects students to complete a task for controlling a motor. Generally speaking, to control a motor, a motor driver is required. A motor has two rotation directions: clockwise and anticlockwise, which correspond to different directions of current flow respectively. Therefore, if we want to program the motor to rotate in a clockwise or anticlockwise direction to achieve the task, we must use a motor driver.

1  Connecting the Arduino, the DC motor and the motor driver

We connect the Arduino, the DC motor and the motor driver as follows.

Motor 2

Motor 1

Enlarged view of the motor driver

2  Programming the Arduino to operate the motor with the motor driver

Open Arduino IDE and connect the Arduino to the computer. Select Arduino UNO as the development board in the Tools menu and check whether the Communications Port is connected to the Arduino already. After connecting the Arduino, we need to input the following program and upload it to the Arduino.

The complete dual DC motor program codes.  Refer to file 1_6_two_dc_motor.

Main program

The control logic on the motor driver

Define the pins as output

Define the pins

3  Program content explanation

- The first part defines the pins, allowing us to write the program promptly.

- The second part sets the working mode of the pins, which in this case is for communicating with the motor driver. The Arduino UNO sends data to the motor driver, so the pin modes are set as output.

- The repeating part instructs the motor to move forwards for two seconds, move backwards for two seconds and then stop for one second. Then the whole action is repeated.

These codes define the actions according to the signals received by the motor driver. For example, when all IN1, IN2, IN3 and IN4 are at low voltage, the motor will be stopped.

Review questions

- What is a DC motor?

- What is an H-bridge motor control?

Chapter 7: Unit Project – Lie Detector

Robotics covers a range of elements, including science, technology and engineering. When conducting the unit project, students need to apply knowledge of various disciplines. For example, firstly, you should start with scientific research, studying the principle of electric current and the characteristics of human skin, and then find out how to use science to judge whether a person is lying. Secondly, you should consider the design of the robot by using the electronic components learned in Unit 1. When the prototype is completed, testing should be carried out. Finally, you should improve your design based on the test results. Throughout the project, students are expected to acquire more knowledge on science and engineering. It is also hoped that students can improve their communication, problem solving and leadership skills through collaboration with the others.

| Stage / step of engineering design | Relevant knowledge | Investigation and design considerations | Applying the relevant knowledge and investigation results |
| --- | --- | --- | --- |
| Define the problem (confirm the requirements and limitations) | Understand the working principle of lie detectors and refer to the standards used by current or previous lie detectors | Determine the design requirements and limitations | / |
| Research | - The principle of electric current and the different conductivity of materials - Conductivity of human skin - Effect of emotions, environments and weather on skin conductivity (e.g. tension causes a decrease in skin conductivity) - Methods for measuring skin conductivity | Select a handling method (for example, if carried out in the class, may choose one or more items, depending on the teaching aims, lesson time and limitations on materials) | / |
| Design consideration | - Choose Arduino as the processor of this experiment - Use analogue signals to measure skin conductivity - Use LEDs and a buzzer to show the responses of the person when he/she is being asked questions | Prediction: When lying, the conductivity of the skin is reduced due to tension. Analogue signals can effectively measure the changes in skin conductivity | Test with the Arduino first |
| Testing model | - Factors that affect skin conductivity include human perspiration, environmental humidity and emotions - Design a circuit diagram to measure the skin conductivity according to the voltage divider rule (potential divider) - Use a red LED, a green LED and a buzzer to show the responses of the subject when being asked questions - Use a blue LED to show the contact condition of the lie detector - Use a positioning tool to reduce the error when the lie detector is used on different people | Assume emotion is the major factor that affects the conductivity of human skin. Conduct fair tests for the remaining variables one by one to verify the impact of each variable on the effect | / |
|  |  | Fair tests: To find out how human perspiration affects skin conductivity - Independent variable: the amount of perspiration of the human body - Dependent variable: skin conductivity - Control variables: exercise volume, body temperature, metabolism, etc. To find out how different environmental humidity affects skin conductivity - Independent variable: environmental humidity - Dependent variable: skin conductivity - Control variables: temperature, weather, ventilation, etc. | / |
| Solve problems encountered during the design / production / testing processes | The testing process shows that to get better results and more accurate measurement: - the subject needs to be in contact with the lie detector constantly when he/she is being asked questions - adjust the position of the lie detector with the positioning tool for each user before use - conduct the test at a place with good ventilation equipment | Regard these findings as the necessary procedures for the current engineering design | / |
| Analyse and evaluate test results and problems occurred | - The linear relationship between voltage and resistance [Ohm's law] (engineering knowledge) - The accuracy of the lie detector | - Analyse data using Ohm's law to assess whether the design has achieved its stated objectives - According to the test results, evaluate whether Ohm's law is the key to success of the lie detector - Measuring the skin conductivity as the only condition for lie detection gives inaccurate results | Based on the analysis and evaluation results, find out ways for improving the design |

1  Circuit making of the lie detector

We set up the circuit of the lie detector as follows:

2  Exterior design of the lie detector

Students can design and make the housings for the lie detectors based on their own ideas to protect the Arduino boards and other electronic components. The following is a reference of the housing for the lie detector made of cardboard.

|  |  | - Reserve a space for the USB port on the housing |
| --- | --- | --- |

3  Programming the Arduino to make it work as a lie detector

Open Arduino IDE and connect the Arduino to the computer. Select Arduino UNO as the development board in the Tools menu and check whether the Communications Port is connected to the Arduino already. After connecting the Arduino, we need to input the following program and upload it to the Arduino.

The complete program codes for the lie detector.  Refer to file 1_7_lie_detector.

Define the variables

Set the variables as output and open the serial communication channel

Use the analogue input to change the error value and detect lies by measuring the amount of water

When failing the test, the buzzer sounds

4  Operating principles

This lie detector assumes that the subject will sweat when lying, and sweating will change the conductivity of the fingers slightly. Making use of this property, the Arduino can be programmed to detect the changes when connected to the circuit, and make corresponding judgment. This is how this lie detector works.

5  Program content explanation

- Define the variables

In the first part, we define the variables. Firstly, the pin settings of the red, green and blue LEDs are 9, 10 and 11 respectively. We define pin 7 for the buzzer, and 14 and 15 for the two analogue pins. Finally, we add the standard definition 25 obtained from testing.

- Set the variables as output and open the serial communication channel

In the second part, we need to set the variables defined in the first part as output and input, such as the red, green and blue LEDs, the buzzer and the two analogue pins. We will also open the serial communication channel and set the transmission rate as 9600.

- Use the analogue input to change the error value and detect lies by measuring the amount of water

In the third part, we first read the values from the two analogue input pins with the function 'analogRead()'. If the values of the two contacts we obtain through the wires are larger than the pre-set standard, we will turn on the red LED, sound the buzzer and print the line 'Lie detector fail.'. If the values do not exceed the pre-set standard, the line 'Lie detector ready' and finally 'Lie test pass.' will be printed.

- When failing the test, the buzzer sounds

In the fourth part, we configure a function called 'buzzer()'. We create a loop that repeats for a thousand times, turning on and off the buzzer continuously to simulate the sound of an alarm.