Junior Secondary

Robotics Learning and Teaching Resources

Extension Unit 1:	Communication and 
     		automation in robotics

Technology Education Section

Curriculum Development Institute

Education Bureau

The Government of the HKSAR

May 2019

All comments and suggestions related to the resource materials may be sent to:

Chief Curriculum Development Officer (Technology Education)

Technology Education Section

Curriculum Development Institute

Education Bureau

Room W101, West Block, 19 Suffolk Road

Kowloon Tong

Hong Kong

The copyright of the materials in this package, other than those listed in the Acknowledgments section and the photographs mentioned there, belongs to the Education Bureau of the Government of the Hong Kong Special Administrative Region.

Schools and educational organisations are welcome to use the content of this package for non-profit making educational purposes.  In all cases, proper acknowledgements should be made.

Otherwise, all rights are reserved, and no part of these materials may be used for publication or other purposes in any form without the prior permission of the Education Bureau.

© Copyright 2019

The learning resources is developed by the Robot Institute of Hong Kong.

Content

| Chapter 1: Basics of Communication and Automation | P.3 |
| --- | --- |
| Chapter 2: Wireless Communication – Bluetooth | P.6 |
| Chapter 3: Wireless Communication – APP Inventor | P.11 |
| Chapter 4: Unit Project – Rescue Robot | P.23 |

Communication and Automation in Robotics

Extension Unit 1

Chapter 1: Basics of Communication and Automation

When setting up a microprocessor, input device, output device and data transmission are always involved.  Taking the temperature and humidity sensor learned in the previous unit as an example, we need to read the data from the sensor and then transfer it to the microprocessor. The microprocessor provides a logical output after analysis.  The whole process involves two sessions of data transmission.  In certain cases, we will use wireless communication hardware to design the robot.  In daily life, robots are closely related to us.  For instance, with careful observation of the mechanical devices in MTR stations, one can find the existence of robot systems.  Another example is the automation applied to the Octopus devices, enabling them to identify different types of Octopus cards, and make records and debiting.  The way the Octopus system operates has produced an immense impact on human living.  In this unit, we will study the basic concepts of various communication interfaces and automation, allowing us to make semi-automatic or fully automatic robots.

- Communication in robots

A communication method refers to the method of transferring data between electronic components such as a sensor and a control board.  We will introduce three common communication interfaces: serial communication, parallel communication and wireless communication.  Serial communication conveys only a single bit of data at a time.  It contrasts with parallel communication, which conveys multiple binary digits simultaneously.  In parallel communication, 8 binary digits (a byte) are transmitted as a whole on a link with multiple parallel channels.

| A sensor Arduino UNO microcontroller board | A sensor Arduino UNO microcontroller board |
| --- | --- |
| Serial communication | Parallel communication |

| Serial communication | If the transfer of files employs serial communication, we can imagine as if there is only one pipe for transmitting four numbers at the same time. Based on this characteristic, we can conclude that the speed of serial communication is relatively slower. Since there is only one pipe for data transmission, the setting is relatively simple. Serial communication is widely used in daily life. For example, in addition to the pins for connecting to the power supply and ground, the temperature and humidity sensor we used has a pin for collecting data. |
| --- | --- |
| Parallel communication | If the transfer of file employs parallel communication, we can imagine as if there are four pipes for transmitting the four numbers at the same time. Based on this characteristic, we can conclude that the speed of parallel communication is faster. However, since there are several pipes for data transmission, the setting is more complex. Parallel communication is also widely used in daily life. For example, the joystick for a game console has five pins. In addition to the ones for connecting to the power supply and ground, there are three other pins for collecting data. Early Random Access Memory (RAM), a narrow strip of object full of pins, also made use of the principle of parallel communication. |
| Wireless communication | In general, wireless communication uses wireless signals, which are electromagnetic waves of different frequency bands, for data transmission. Radios, wireless TVs, Bluetooth, mobile phones and wireless networks are examples that use the wireless communication technology. Depending on the frequency, wireless communication has different usage characteristics and applications. For instance, very high frequency (VHF) is commonly used in civil aircraft. |

Automation

Automation can be defined as the technology by which a process or procedure is performed without human assistance.  In other words, automation or automatic control is the use of various control systems for operating equipment such as machinery, processes in factories, boilers and heat treating ovens.  These processes have been completely automated.

The major mechanism used in automatic control systems is the feedback mechanism.  We can use a constant temperature air conditioner as an example.  In the operation of a constant temperature air conditioner, we first preset a specific temperature, then the sensor will continuously monitor whether the temperature index is consistent with the temperature we set.  If the temperature is lower than the preset one, heating will be released and vice versa.  Automation is an indispensable part of the design of robots.  In high-level robot competitions, the automated system is often the part that decides the winner.  For example, in the World Robot Olympiad, robots often need to walk automatically and detect specific objects, and then pick up a particular item and put it in the proper position.

Constant temperature air conditioner

Automation	Semi-automation

Apart from automation, there is also semi-automation.  Semi-automation generally refers to the situation that after a machine has completed a work cycle automatically, a human decides whether or not to start a new cycle.  Semi-automation is also common in robot design.  The project in the previous unit – robot cleaner – is a good example.  Initially, the robot moves around randomly and cleans the floor, but then needs to be reset manually and restarted when needed, which is an example of a semi-automatic robot.  In this unit, we will discuss and use different communication methods to create automatic and semi-automatic robots.

Review questions

Can you answer the following questions after completing this chapter?

- What communication methods are available in hardware such as Arduino?

- What is automation?

Chapter 2: Wireless Communication – Bluetooth

- Bluetooth basics

Bluetooth is a wireless communication technology standard that is commonly used in mobile devices such as smartphones or tablets for exchanging data within a certain distance.  In general, wireless communication technology uses radio waves of specific frequencies to transmit data, and Bluetooth uses UHF (ultra high frequency) radio waves (2.4 GHz).  Several electronics industry leaders such as Sony Ericsson, Nokia and Toshiba founded the Bluetooth Special Interest Group (Bluetooth SIG) to develop the technology, hoping to develop a standard that is low-cost, highly efficient and wirelessly connectable over short distances.

Master device

Slave device

Slave device

Slave device

Bluetooth technology has a wide range of applications, from wearable devices, computers, mobile phones, heart rate monitors, smart homes, to cars.  Bluetooth devices are usually categorised into two types: master and slave.  Bluetooth devices such as computers and mobile phones that can explore and pair with other Bluetooth devices are called master devices.  Slave devices are mostly devices that are passively waiting to be connected, such as mice, keyboards, headphones and robots.  There are many profile specifications set by the Bluetooth SIG, such as Human Interface Device Profile, which is generally used for mice, keyboards, etc., and Hands-Free Profile, which is generally used for mobile devices like smartphones to support voice dialling and redialling.

Human interface device

Hands-free device

- Do it yourself – Basic operation of the HC05 Bluetooth module

In this chapter, students need to use a Bluetooth module to let an Android mobile phone communicate with an Arduino control board.  Even though this task may seem complicated, the codes are actually similar to those we used to connect to the computer with serial communication.  We have tried to use serial communication for the connection between the Arduino control board and the computer.  Instead of a computer, the current task is to establish connection between the Arduino control board and the Bluetooth module with serial communication.  Data of a temperature and humidity sensor will be sent to the mobile phone and displayed on it.

Be reminded to remove the Bluetooth module from the Arduino board when burning the program since the Bluetooth module will occupy the only serial communication channel on the Arduino UNO when the module is connected to the control board.  The computer thus will not be able to communicate with the Arduino board, leading to errors when burning the program.  We unplug the power cable or the transmit (TX) / receive (RX) pins of the Bluetooth module.  The TX pin of the HC05 is connected to the RX pin of the Arduino board; and the RX pin of the HC05 is connected to the TX pin of the Arduino board.

- Search for 'Bluetooth spp tools pro' on Google Play.  This tool allows the mobile phone to become a serial monitor via Bluetooth, thus achieving the purpose of controlling the Arduino control board.

- Connecting the Arduino control board to the power supply, you should see the LED on the HC05 flash rapidly, which means the HC05 has entered the pairing mode.  At this time, students can turn on the Bluetooth function of the Android phone, and then open the Bluetooth list on the phone.  Look for the name of the HC05 Bluetooth module and try to pair with it.  If a connection password is required, the password is typically 1234; try 0000 if unsuccessful.

- The following are the program codes:

The complete program codes for monitoring the temperature and relative humidity via Bluetooth communication.  Refer to file 3_2_bluetooth_dht11.

Define the variables

Open the Bluetooth serial communication channel

Use the DHT11 sensor to take the readings and display them on the Bluetooth serial communication channel

Include the function libraries

Program content explanation

- This part of the program mainly defines the names of the variables and includes the function libraries.  #include <dht.h> means that the program will include all the function definitions and variable definitions in the dht.h file.  Without a library, we will be unable to use some quick methods to extract values.  Note that this command line does not require a semicolon.  The other line dht DHT is a command for turning on the temperature and humidity sensor.  Without executing this line, DHT.readll (dht11_pin) will not work.  Finally, as the value returned by the program contains decimal places, be reminded to use the data type 'float' to store the variable.

- Open the serial communication for sending messages to the Bluetooth module

- Read the temperature value and send the value retrieved by the DHT11 to the serial communication device, which is the Bluetooth module in this case.

| DHT.readll(dht11_pin) | DHT.temperature |
| --- | --- |
| Update the value of DHT.temperature | Update the temperature value after readll() is executed |

- Open the serial monitor to read the temperature values.

Review questions

Can you answer the following questions after completing this chapter?

- What is Bluetooth?

- Can you list some specifications of Bluetooth devices and suggest the characteristics set by the specifications?

Chapter 3: Wireless Communication – APP Inventor

The task of this project is to use APP Inventor to control the rotation of a servo motor connected to an Arduino board through an HC-05 Bluetooth module.  Students should note that this task involves three parts: first, the interface and program settings of APP Inventor; second, using the HC-05 Bluetooth module to connect to the mobile phone and communicate with APP Inventor; third, use APP Inventor to control the rotation of the servo motor.

- Interface and program settings of APP Inventor

- Enter the URL http://appinventor.mit.edu/explore into the browser to open the website of APP Inventor.

- Choose 'Create apps!' in the top right corner of the APP Inventor website to create a new mobile app.

- Sign in with a Google account before using APP Inventor.

- After logging in, the My Projects page of App Inventor will be shown.

- At the top left of the My Projects page, press Start New Project and give a name to the new project.  The project name can be set as 'Bluetooth_Servo' but students can also use other names.  Note that the project name must start with a letter.

- After setting the name and pressing OK, a new Bluetooth_Servo project will be added to My Projects.  Open the Bluetooth_Servo project and the work panel will be shown, where the display interface (Viewer) of the mobile app can be seen.

- Students can personalise their mobile app by selecting their favourite settings in the Component Properties panel on the right.  For the sake of convenience, this demonstration will only choose 'Center' in the 'AlignHorizontal' and 'AlignVertical' options.  Students can set other properties by themselves.

- In the User Interface on the Palette on the left-hand side, select ListPicker.  Drag the ListPicker to the Viewer area in the middle.  The ListPicker will appear in the middle of the tablet preview screen and ListPicker1 will be added to the component list on the right.

- We can add an image to ListPicker1 to make it easier for users to understand the various parts of the app.  To add an image, we can press Image in the Properties panel on the right and select the image you want.

- After adding the image, you can adjust its size and add text.

- Then, in the User Interface on the Palette on the left-hand side, select Button.  Drag the Button to the Viewer area in the middle.  We can resize the button and input text for the button in the Properties panel on the right.  We have to make a total of three buttons, namely Disconnect, Servo motor rotates 90° and Servo motor rotates 180°.  Note that the visibility of the Disconnect button is preset as no, i.e. we should uncheck the Visible checkbox.

- When the setup for the button is completed, we need to configure the Bluetooth settings in the app.  Select Connectivity on the Palette on the left-hand side, and then select BluetoothClient.  Drag the BluetoothClient to the Viewer area in the middle.  We will see the BluetoothClient appearing at the bottom of the preview screen, under Non-visible components.

- When the setup of the user interface is completed, we need to write the program of the app.  Press the Blocks button at the top right to convert to the programming view.

- Drag the program blocks from the Blocks area on the left to the Viewer area to write the codes.  Based on the type and colour of the program blocks, we create the following program.

Make the Bluetooth connection

Set the Bluetooth functions

Configure the Bluetooth settings

The program can be divided into three parts, which are for configuring the Bluetooth settings, making the Bluetooth connection and setting the Bluetooth functions.  Firstly, in the configuring the Bluetooth settings part, we define the address and name that the Bluetooth needs to connect in the ListPicker.  When we have selected the Bluetooth address we need to connect to in the app, the app will connect the specified Bluetooth via the BluetoothClient.  If the Bluetooth connection is successful, we can send messages to the Arduino by pressing the Servo motor rotates 90° button and the Servo motor rotates 180° button.  In the above program, when we press the Servo motor rotates 90° button and the Servo motor rotates 180° button, the phone will send the letter 'a' and the letter 'b' to the Arduino via Bluetooth, respectively.

When all the operations are complete, we can interrupt the Bluetooth connection by pressing the Disconnect button.

Connecting the HC-05 Bluetooth module and the servo motor to the Arduino

First, connect the Arduino, the HC-05 Bluetooth module and the servo motor as follows.

The way to connect the HC-05 Bluetooth module and the servo motor to the Arduino is simple.  There are four pins to connect for the Bluetooth module.  The VCC pin, GND pin, TXD pin and RXD pin of the Bluetooth module are connected to the 5V pin, GND pin, RXD pin and TXD pin of the Arduino, respectively.  For the servo motor, the red and black power wires are connected to the 5V pin and GND pin of the Arduino respectively, and the yellow or white signal wire is connected to pin 11 of the Arduino.

Using APP Inventor to control the rotation of the servo motor

Firstly, we need to input the following program in the Arduino IDE and then upload it to the Arduino UNO board.

The complete program codes for controlling the servo motor via Bluetooth.  Refer to file 3_3_bluetooth_servo_motor.

Open the serial communication channel and define the pin for the servo motor

Control the servo motor according to the signal received via Bluetooth

Include the function library and define the variable

Program content explanation

- Include the function library and define the variable

In the first part, we include the servo motor library and define a variable called servoPin.  In the above program, we first add the library named Servo downloaded to the Library Manager before.  Then we store the number 11 in the variable servoPin.  When completed, we use 'Servo Servo1' to initiate the servo motor library included above.

- Set the serial port connection rate and define the pin for the servo motor

In the second part, we set the serial port connection rate as 9600.  In this task, the Arduino UNO board needs to communicate with the HC-05 Bluetooth module, therefore the two units need to have the same serial port connection rate so that they can connect and communicate with each other.  Then,  we define the signal pin of the Arduino UNO board to which the servo motor is connected with 'Servo1.attach', which has been defined as pin 11 of the Arudino UNO board in the first part.

- Control the servo motor according to the signal received via Bluetooth

The third part is the key to the entire Arduino program.  This part of the program controls the rotation of the servo motor according to the signal received by the Bluetooth module.  In APP Inventor, we have configured the phone to deliver the letter 'a' and the letter 'b' when the Servo motor rotates 90° button and the Servo motor rotates 180° button are pressed, respectively.  As a result, in the Arduino program, we can determine whether the servo motor should rotate 90° or 180° by analysing whether the letter 'a' or 'b' is received by the Bluetooth module.  In the program, we use 'Servo1.write' to control the angle of rotation of the servo motor.  At the beginning, we reset the angle of the motor to ensure that the motor starts from the same position before every rotation.  Then, we use 'Serial.available' to check if the Bluetooth connection with the device has been established.  If the connection is successful, we use 'Serial.read()' to read the message received by the Bluetooth module.  If the received message is the letter 'a', we use 'Servo1.write' to control the servo motor to rotate 90°; if the received message is the letter 'b', we use 'Servo1.write' to control the servo motor to rotate 180°.  When the Arduino program is completed, we can turn on the Bluetooth function of the phone and connect it to the HC-05 Bluetooth module using the App Inventor on the phone.  Once connected, we can use the APP Inventor to control the rotation of the servo motor.

Review questions

Can you answer the following questions after completing this chapter?

- What is App Inventor?

- Which type of communication means does Bluetooth belong to?

- Can you set up the Arduino for it to interact with the App Inventor?

Chapter 4: Unit Project – Rescue Robot

| Stage / step of engineering design | Relevant knowledge | Investigation and design considerations | Applying the relevant knowledge and investigation results |
| --- | --- | --- | --- |
| Define the problem (confirm the requirements and limitations) | Understand that a disaster scene is very dangerous. If the rescue job is done by humans, not only that the rescue team may risk their lives, but it is also not quick enough. Therefore, we need the help of a robot |  | Clearly set the design requirements and limitations (see the example in data file 1) |
| Research | - Performance and transmission distance of various wireless connection - What tasks need to be completed during a rescue - What sensors can be used to find the signs of life - What sensors can complete the rescue mission |  | Choose a handling method (if conducted in class, one or more options can be chosen, depending on the teaching objectives, lesson time and material restrictions) |
| Design consideration | Research data show that Bluetooth transmission has the highest efficiency when operating in short distances | Prediction: Automatic robots can improve rescue efficiency | Test the operation of the sensor and the robot with simple programs first |
| Test the model | Use two robots to find signs of life and rescue lives separately. The two robots communicate via Bluetooth to speed up the rescue operation | How to speed up the life rescuing operation effectively |  |
|  |  | Fair test: Find the program that can make the robots walk the fastest - Independent variable: different programs - Dependent variable: length of time - Control variables: venue, motor power, battery voltage, etc. |  |
| Solve problems encountered during the design / production / testing processes |  | The program must allow the action robot to effectively send useful information to the rescue robot, so that the rescue robot can rescue lives successfully | Take these findings as the necessary procedures of this engineering design |
| Analyse and evaluate test results and problems occurred |  | Use a sensor and a program to allow the robots to move close to a black line and assess whether the design speed is up to standard | Based on the analysis and evaluation results, find out ways to improve the design |
| Improve | In addition to installing a single infrared sensor, multiple different sensors can be installed to help the robots act quickly and effectively | Use computer vision and artificial intelligence to improve the efficiency of the robots, including the ability to move along a black line effectively | Explore together how a rescue robot is made today with the current technology |

1  Preparing the circuit of the robot

We connect the Bluetooth module, motors, motor control module, 9V battery and Arduino as follows.  When completed, we will install the electronic components on the chassis of the rescue robot.  In order to drive the motors, it is recommended to use a higher voltage.  Since the Arduino UNO has a voltage regulator module, the 9V battery can be connected directly to the Arduino to drive the control board.

2  Making the body structure of the rescue robot

Students can design and make their own outer casing for the rescue robot, in which the Arduino board and other electronic components will be placed, based on their own ideas.  The following is a reference for making the chassis of the rescue robot:

| - Get 8 craft sticks and put them together | - Put hot melt glue on a craft stick with a hot melt glue gun |
| --- | --- |
| - The craft stick covered with hot melt glue | - Press it on the whole row of craft sticks to fix them |
| - Finally, add a craft stick diagonally for fixing | - Use hot melt glue to fix the castor on the craft sticks |
| - Add 2 craft sticks at the rear to increase the height | - Attach a yellow motor at the rear of the robot |
| - Repeat the above action | - Apply hot melt glue to the craft sticks beside the motor on the left side of the robot |
| - Apply hot melt glue to the craft sticks beside the motor on the right side of the robot | - First cut up the plastic cover and apply hot melt glue |
| - Finally, add a rear cover for the robot |  |

3  Writing the program

Mobile app

Send 'f', 'r', 'l', 'b' and 's' commands to the HC-05 Bluetooth module

Send 'f', 'r', 'l', 'b' and 's' commands to the Arduino control board via serial communication

Send the control commands to the motor driver to control the motors

Connect the motors

In the above example, we use 'f', 'b', 'r', 'l' and 's' to represent the commands forward, backward, right, left and stop, respectively.  When the Arduino control board receives the commands from the HC-05 Bluetooth module, it will send control signals to the motor driver.  The motor driver will react when it receives the signals, and drive the motor to rotate.

When the design of the project procedures is finished, we can begin to design and write the interface and logic of the Android mobile app.  The following are the steps to write the Android mobile app.  Like the previous Do It Yourself activity, we have to design the interface first and then write the logic operation.

- First, start a new project and name the project.

- When the interface is loaded, drag the TableArrangement under Layout on the Palette on the left-hand side to the Viewer area.  This table is used to locate the arrow keys.  We have a total of five buttons for forward, backward, left, right and stop, so we need to enter ‘3’ for both the Columns and Rows in the Properties panel to fit in all these buttons.  The TableArrangment can be seen on the Components panel.  To make it easier for students to understand and develop the app faster, it is usually recommended to change the names of the components.

- Once the interface configuration is defined, we can add more components to complete the mobile app.  Students can drag five buttons from the Palette to the table and arrange the forward, backward, left, right and stop buttons accordingly.  Same as other general Bluetooth apps, we need a ListPicker to display the Bluetooth devices found, a non-visible BluetoothClient component and a Disconnect button.

- The properties of the buttons such as size and text can be found in the Properties panel.  To change the text on the buttons, we need to find a field named Text in the Properties panel and enter the name you want in the text box.  To make the app look tidier, students can set the AlignHorizontal in the Properties panel of Screen1 as Center, so that the various components will be placed in the middle of the screen.  The Disconnect button should only be shown when the connection is successful.  Students can uncheck the Visible checkbox in the Properties panel of the button and change the status of the visibility when doing the logic programming.  The interface of the app can be considered done by now.

- The program logic can be divided into two parts.  The first part deals with the program blocks for the Bluetooth connection.

The three sets of codes above allow the mobile phone to communicate with the Bluetooth module.  The first set of program blocks means that when the user presses the list, each element in the list will change to the address and name of the Bluetooth receiving end.  The second set means that when the user finishes the selection, the program will establish the Bluetooth connection with the address and name chosen.  Moreover, the program will make the Disconnect button visible, so that the user can interrupt the connection between the mobile phone and the Bluetooth module when appropriate.

- These five buttons are the same in nature, so their codes are almost identical.  When one of the buttons is pressed, a letter will be sent to the Bluetooth module.

- The mobile app part is complete.  Then we will focus on the program part of the Arduino control board.

The complete program codes for the rescue robot.  Refer to file 3_4_rescue_robot.

Define the operation modes of the pins and open the serial communication channel

Define the variables for the pins

Update the command from the mobile phone at each execution

Perform the action after receiving the command

- The first part of the program defines the constants for the program.  This part of the program turns the pin numbers of the connected pins to text, making them easier to use and to understand when writing the program.  Pins 9 and 10 are selected to connect to pins ENB and ENA of the motor driver in the program, respectively, because the two pins can output analogue signals.  In addition, a variable called incomingByte is defined to store the message received by the Bluetooth module for decision making in the later part of the program.

- This part of the program will only be executed once to set the transmission rate of the serial communication, and the use and configuration of the pins.  Since we are sending messages to the motor driver, the mode of the pins must be set as output to transmit the control signals.  The last two digitalWrite() are used to indicate to the motor driver that both motors are being used.  That is why the pins motorENA and motorENB are always outputting a high voltage.

- This part of the program is placed in the loop() function to keep it running.  This is a simple if-then logic sentence, meaning that if there is an incoming message sent through the serial communication, the program will read the message and store the message in the variable incomingByte, and then wait for further actions.

- The following part of the program may seem complex, but with a closer look, it can be noted that there are many repeated parts.  Each command in the program is written in accordance with the following table.  For example, if we want the robot to stop, students should output a low voltage to the four pins IN1, IN2, IN3 and IN4 to trigger a stop command.

- The programming part is finished.  The final action is to connect the hardware.