ＩSafe Use of Tools and Equipment

Relevant Knowledge

In addition to the tools and equipment mentioned in the previous two books, we will also use instruments for measuring. In this section, we will introduce multimeters and data-logging devices.

1. Multimeters

A multimeter is a multipurpose electronic measuring instrument. It usually can function as an ammeter, a voltmeter and an ohmmeter. Multimeters can be categorised into analogue type and digital type, which display the measured values by means of a pointer over a scale and digital numerals respectively.

A digital multimeter

An analogue multimeter

a. Analogue Multimeters

N

S

Inside an analogue multimeter, there is a pair of magnetic poles and within the poles is a rotatable coil of wire  surrounding a cylindrical iron core. A pointer is attached to the coil and underneath it is a scale to indicate the measured values. When an electric current passes through the coil, a force will be generated. This force is directly proportional to the electric current. The pointer can thus indicate the magnitude of the electric current. Inside an analogue multimeter, there are also resistors connected in series or in parallel with the coil for measuring larger electric currents and voltages. With the addition of batteries to provide the power, an analogue multimeter can also be used to measure resistance.

b. Digital Multimeters

A digital multimeter utilises a circuit to create various impedance for different ways of use.

c. Comparison

An analogue multimeter has a fast response and does not require power for measuring electric currents and voltages. Calibration is required to adjust the zero position of its pointer before using. For a beginner, the major difficulty for using an analogue multimeter is the way to take a reading from it. Comparatively, everyone can read with ease the values shown on a digital multimeter.

When a digital multimeter functions as a voltmeter, it has very high input impedance and is thus advantageous in measuring the voltages of electronic circuits because the components in these circuits often have high resistance.

d. Safe Use

When using a multimeter on a low-voltage circuit, it is relatively safe for the users. However, if it is used to measure the 220V mains circuit or circuits with even higher voltages, it is dangerous to be carried out by secondary students. They should not try to do this at all; if they have to, they must do it under teacher's guidance.

Even though a low-voltage circuit should not pose danger to the users, if one uses the probes to make contact here and there randomly while the circuit is energised, short circuits may occur and the parts of the circuit may get burnt and be damaged. Therefore, measurement with a multimeter should be conducted according to teacher’s instructions.

2. Data-logging Devices

In most systems, more than one set of data is used. This will require the use of a computer to collect data through sensors, analyse data, and store and output the results being collected and analysed. Data recording involves the control of a computer to collect and analyse data.

Employing appropriate data-logging devices, we can monitor and quickly collect a large amount of accurate data. Examples of data that can be collected include temperature, light intensity, sound frequency and intensity, vibration time and intensity, electric currents, pressure, and status of material change.

A small wind speed and temperature logging device that stores records in a memory card

A wind speed and humidity logging device that sends data to terminals directly

With these data-logging devices, we can collect data under suitable circumstances, clearly displaying the data, analysing the data, realising the situations, making adjustments, predicting the future, ….

Analogue sensor

Digital sensor

Computer

Quantity or event to be monitored

Simple LCD display

Computer screen

a. Block Diagrams

A block diagram on logging and displaying functions

Parts for testing temperature, light intensity, sound intensity, pressure, etc. have been discussed in previous materials and will not be repeated here.

Some of the sensors, e.g. limit switches and temperature switches, are digital sensors. Their output signals are digital signals that consist of only two states – high and low electric potentials.  Other sensors, e.g. displacement sensors (rotary variable resistors) and temperature sensors (thermistors), are analogue sensors. Their output signals are analogue voltage signals, which are directly proportional to physical quantities such as degree of displacement and change of temperature.

A limit switch logging device that outputs digital voltage signals

A pressure logging device that outputs analogue voltage signals

b. Application Examples

The following figure shows a website of a campus weather station of a school. Making use of various data-logging devices and a computer for data processing, the weather information of the surrounding region of the school is presented to the public through the website.

II Application of a Range of Machines to Realise Solutions for Design Problems

When realising design solutions, in addition to hand tools and small power tools, some larger machines can help save production time, add precision and improve product appearance. In Secondary 2 Material 1C, we have introduced lathes. In this section, we will introduce vacuum formers and laser cutters.

- 1. Lathes

Headstock

Machine bed

Leadscrew

Chuck

Adjustable tool post

Tailstock

Apron

Finished products made by metalwork lathes	A metalwork lathe

Finished products made by woodwork lathes	A handle made by a woodwork lathe

From the finished products shown in the above figures, it can easily be seen that all of them are round shaped.

Mould

Finished product made by vacuum forming

2. Vacuum formers

Vacuum forming is a thermoforming process. During the process, a sheet of plastic is heated to a specific temperature, ‘a forming temperature’, and stretched onto a single-surface mould. Then, air between the surface of the mould and the plastic sheet will be extracted to form a vacuum.

Vacuum forming is usually applied to plastic parts that are shallower in depth. For instance, a thin sheet can be formed into a rigid cavity for storing medicines. On the other hand, by using thicker sheets, permanent objects such as traffic signs and protective covers can also be formed by vacuum forming.

Thermoplastics are materials used in vacuum forming conventionally. A common choice of which is high impact polystyrene sheet. It can be deformed to various shapes according to the shapes of the moulds.

Finished products made by vacuum forming

A vacuum former

When using a vacuum former, the following safety measures must be taken to minimise the risks of accidents:

(1)	Ensure all fixed and interlocked guards are ready;

(2)	Do not use an operating machine when the interlocked guards are open;

(3)	Return the heater to its rest or rear position when the interlocked guards are open.

- 3. Laser cutters

Laser gun emitting a laser beam

High-energy laser beam is directed at material to achieve effects such as cutting and engraving

Laser beam reflected and focused by a mirror

a. Principle

A laser is a high-energy light ray. When a laser beam is emitted from a laser tube and focused on a very tiny spot of a material (such as plastics, metals and wood), the light energy will heat up this area of the material to a very high temperature, causing it to melt and vaporise rapidly. This method can be used to pierce, cut and weld metals together.

b. Laser cutting

A laser cutter directs the output of a high power laser beam at the target material. The material will melt, burn or vaporise away, leaving an edge with a high quality surface finish. Since semiconductor lasers have higher efficiency, they are becoming more popular. For cutting materials, carbon dioxide (CO2) lasers are still the most commonly used type.

A CO2 Laser Cutter / Machine

Finished products made by laser cutting

c. Other Uses of Lasers

In addition to cutting, welding, rapid prototyping, engraving, drilling and etching workpieces, lasers can also be used for e.g. heat treatment and surface treatment of metals.

Scientists are also studying their uses on medical applications. For example, they can be used for cutting during surgery to reduce the areas of wounds and speed up recovery processes. They can also be used to improve vision correction procedures. Besides, scientists are studying the application of laser technology in energy and military projects as well.

d. Safety

When using a laser cutter, the following safety measures must be taken to minimise the risks of accidents:

- Wear approved protective goggles to protect eyes from burning by laser reflection;

- Do not cut highly reflective surfaces, especially mirrors, to avoid damages or injuries caused by laser reflection;

- Keep flammable substances away from the cutting area to avoid burning by the laser cutter;

- Ensure sufficient ventilation to prevent respiratory hazards induced by fumes and smoke from the laser cutter;

- Follow the safety procedures set by the manufacturer, e.g. closing the cover and preventing electric shock.

III Glossary of Terms

| Multimeter | 萬用表 | Anemometer | 風速計 |
| --- | --- | --- | --- |
| Ammeter | 電流表/安培計 | Lathe | 車床 |
| Voltmeter | 電壓表/伏特計 | Vacuum forming | 真空成形 |
| Data logging | 數據擷取 | LASER | 鐳射/激光 |

IV Interactive Information

|  | Website | Content |
| --- | --- | --- |
| 1. | https://www.youtube.com/watch?v=dnUcLSkfFD8&list=PLTZ4awCLR8qcmSbpIWWeT-FIdpO6zsNXr | Making a bowl using a woodwork lathe |
| 2 | https://www.youtube.com/watch?v=sWDM7Ho3us8 | A homemade woodwork lathe |
| 3 | http://www.teepr.com/53899/ | Using a woodwork lathe to make a small lovely Japanese wooden doll |
| 4 | http://zh.wikipedia.org/w/index.php?title=%E6%BF%80%E5%85%89&variant=zh-tw | Laser |
| 5 | http://www.oshc.org.hk/download/publishings/1/302/28.htm | Occupational Safety and Health Council |
| 6 | http://zqfenglei.com.cn/CNf/383/385/index.html | A Chinese laser marking machine company |
| 7 | www.ckylaser.com | Types of laser machines |

After-school Activity

【XX News】	X-X-2014

This morning at about 10 am, a janitor of XX secondary school found smoke coming out of a special room on the Xth floor when he was passing by. He reported the incident to the principal immediately and the school activated the alarm to notify the Fire Services Department. No people were in the room at that time but for the sake of safety, the entire hundreds of teachers and students of the school were evacuated to the playground immediately.

A fire engine standing by at the scene

Firemen arrived at the scene and put out the fire.  Initial investigations revealed no suspicious circumstances. A laser cutter was placed in the special room and was suspected to catch fire due to a mechanical failure.

The school principal said that the laser cutter had been used for over 10 years and was mainly used for cutting acrylic sheets by students during lessons and extracurricular activities.

The above is a piece of news about a laser cutter accident happened in 2014. Form groups of two students, discuss the following and write down your answers.

1. 	What had the person who first found the incident done correctly?

____________________________________________________________________

2. 	The school evacuated the entire hundreds of teachers and students of the school to the playground, affecting their normal classes. On what basis do you think the school did this?

____________________________________________________________________

3. 	Prior to the breakout of the fire, what could have been done to prevent it from happening?

____________________________________________________________________

4. 	What material was the laser cutter of the school mainly used to cut? What precautions should be taken when cutting this kind of material?

____________________________________________________________________

____________________________________________________________________

Appendix: Using vacuum forming to make a package box

| - First, prepare a set of moulds for the products: - Mould for the book strap buckle: 5 mm plastic sheet or plywood board - Mould for the book strap: wood block |  |
| --- | --- |
| - Put the moulds into the vacuum former. |  |
| - Fix the transparent plastic sheet on the machine and lower the heater to heat the plastic sheet. |  |
| - When the plastic sheet softens, lift up the heater. Pull the handle to push the moulds up. |  |
| - Start the vacuuming device to let the plastic sheet form into the shapes of the moulds. Press the button to blow in some air will make it easier to separate the moulds and the plastic sheet. |  |

Remarks on vacuum forming:

1.	The upper side of the mould should not be wider than the lower side, or else the mould cannot be taken out.

2.	The edges of the moulds should not be too sharp, or else the plastic sheet may be ruptured.