Relevant Knowledge

- I Material joining

Joining method

Usually, an article comprises a number of different components. The components have to be joined to form the article. Methods of joining components can be categorised into permanent and semi-permanent.

- Permanent joining

Permanent joining is to join components firmly in a fixed and rigid way. Damages will occur if the joined parts are separated. There are various types of permanent joining such as joining of metals. Examples of permanent joining methods include electric arc welding, gas welding, soft soldering, hard soldering, riveting, pop riveting, seaming and gluing

(a) Soft soldering

Soldering bit

Melted solder

Melted flux

Workpiece

(b) Soft soldering

- Soft solder: a wire form of solder

Soft soldering is a process to produce soldering joints by melting solder (such as tin) onto a workpiece under a high temperature. This is a commonly used permanent joining method for electronic circuits, where solder is melted and placed at the junction points of wires with the use of a high-temperature electric solder iron. During the process of soft soldering, flux, such as pine resin, is often applied. It helps dissolve oxides on the metal surface, allows solder to flow and cleans the workpiece surface.

The following precautions should be strictly followed during the process of soft soldering:

- Wear safety gloves.

- Do not touch flux that is corrosive.

- Be careful of the soldering bits to avoid getting burnt.

- Keep the workplace well ventilated.

- Do not put flammable substances in the workplace.

An electronic circuit board

- (b)	Hard soldering

# Hard soldering is a method for joining metals with the use of an alloy. This method forms joints that are stronger than soft soldering and is generally applied to steel workpieces. Hard soldering can be divided into two types, silver soldering and brazing. Flux, such as borax, is often applied to facilitate the soldering process by preventing the formation of oxides and breaking them down if they exist.

The high temperature required for the hard soldering process is supplied by the brazing torch of a brazing hearth. The brazing torch burns a combination of a flammable gas mixture and compressed air to provide the required heat.

| # Type of hard soldering | # Solder | # Melting point of the solder | # Characteristics |
| --- | --- | --- | --- |
| # Silver soldering | # Silver solder (copper + zinc + silver | # Lower melting point # (600℃ – 630℃) | # Firm soldering effect; temperature needed is lower for application on meticulous structures of ornaments. |

| # Brazing | # Brass solder (copper + zinc) | # Higher melting point # (850℃ – 950℃) | # Commonly applied to steel workpieces in industry, e.g. the carbonised tips connecting to the front of cutting tools. |
| --- | --- | --- | --- |

| - (c) Electric arc welding Electric arc welding is a permanent metal-joining method, in which electrodes and metallic materials are melted using the energy generated during the discharge of an electric arc. The phenomenon of strong and persistent electric discharge in a gas medium between two electrodes is called an electric arc. An electric arc can generate strong heat (about 6000℃) and ultraviolet light at the same time. |  | A product made by welding |
| --- | --- | --- |
| - (d) Gas welding # In gas welding, a flame produced by burning a mixture of oxygen and acetylene is used to melt the workpiece and welding material together. When cooled and solidified, the workpiece and welding material will be joined. |  |  |

- (e)	Riveting

Riveting is the technique of joining two or more workpieces together by rivets. There are various types of rivet for a wide range of use, including the joining of the metallic bodies of cars, aircraft and ships.

Pan head rivet

Round head rivet

Countersink rivet

Flat head rivet

- Different types of rivet

# (b) An application example of rivets – the outer shell of an aircraft

# The procedure of riveting is simple. Take the installation of a round head rivet as an example.  First, clamp two workpieces together and drill a hole through them. Then use a flat file to remove the sharp burrs around the edge of the hole. Put a rivet of an appropriate length into the hole. Then press the two workpieces together using a rivet set and hammer the workpieces together. Finally, hammer the edges of the tail of the rivet until a small round top is formed.

Rivet set

Round head rivet

Rivet mould

- (f) Pop riveting

### The principle of pop riveting is similar to that of riveting, but is even more convenient in operation. The only tool you need are a pop rivet gun and pop rivets. Release the pop rivet gun and fit an appropriate chuck. Insert a pop rivet into the tool and put the other end of the pop rivet into the hole of a workpiece. Squeeze the handles together until the steel pin in the rivet breaks off. Release the handles and remove the pin from the chuck.

Using pop rivets on aluminium windows frames

Pop rivets

A pop rivet gun

The procedure of pop riveting

Workpiece 1

Workpiece 2

Rivet gun bit

Rivet head that remained

Pin that breaks off

Pop rivet

Pop rivet

- (g) Seaming

# Seaming is a process to join materials by their edges, for example, by folding up metallic sheets and then joining the edges by soldering. Seaming can be used to join two pieces of metallic sheets, as at the bottom of a can.

- (h) Gluing

# Gluing is the technique of using adhesives to stick materials together. Most solid materials such as wood, plastics and metals can be joined by gluing. There are various types of adhesives, including contact glues, all-purpose adhesives and chloroform. Adhesives usually have greater shear strength but poorer tensile strength. Therefore, materials separate more easily when being pulled.

A hot melt glue gun

An all-purpose adhesive

- Semi-permanent joining

Semi-permanent joining refers to joining the parts of an assembly to a fixed shape but separation of the parts is still easy after joining. Methods of semi-permanent joining include screw thread joining and knock-down fitting.

- (a)	Screw Thread Joining

# (a) Machine screws		              (b) Set screws

# (c) Steel screws					(d) An anchor screw

Examples of screw components

Application examples of screws

After fixing with a bolt and a nut, the middle part can still move

An electric plug that uses screws for fixing the casing; when maintenance is required, it can be opened up by unscrewing

# Screw thread joining is to use screws to join materials. When the thread on a screw turns, the bevel surface of the screw generates a huge force which can tightly fasten two or more workpieces together. This joining method can be used on most materials, such as metals, wood and plastics. It can also be applied to join objects made of different materials, for example, to attach a plastic sheet to a wooden object. There are many different types of screw components used for this joining method. The following table lists some examples.

| # Screw component | # Uses |
| --- | --- |
| # Wood screw | # To fasten two wooden objects. |
| # Machine screw | # A widely used screw which has threads throughout its entire body. |
| # Bolt | # Mainly used for fastening thicker workpieces of larger scale. When employed, a bolt is inserted through the holes of two or more workpieces, which is then tightened up with a nut. |
| # Set screw | # To fix two workpieces, e.g. components of gears, propellers or other similar rotary shafts. |
| # Stud | # A cylindrical rod with threads on both of its ends or on its whole body. |
| # Self-tapping screw | # A screw that has strong and sharp threads, which can create threads on the workpiece when pressed into it. It is suitable for joining thin metals. |
| # Anchor screw | # A screw set used for hanging a heavy object onto the ceiling. Two commonly used types are stud anchors and expansion anchors. |

Some supporting components for screws

# (a) Washers          (b A wing nut    (c) A cap nut   (d) A bolt and nut

Some supporting components for screws

Anchor screws hold the brackets to the external wall of a building

Bolts and nuts tighten the brackets, fixing the pipes

# Some supporting components may be needed when using screws. The following table lists their functions.

| # Supporting # Component | # Uses |
| --- | --- |
| # Washer | # To avoid causing marks of abrasion on workpiece surfaces when fastening the screws or nuts |
| # Nut | # Working together with a bolt for a tightening action when being turned |
| # Cap nut | # Mainly for aesthetic and safety purposes |
| # Wing nut | # To be applied on parts which need frequent knock-down actions or adjustments |

Some supporting components for screws

- (b)	Knock-down Fitting

A knock-down screw fitting

A knock-down fitting is a specially designed joining device, which is particularly suitable for joining parts of flat pack furniture such as coffee tables, hi-fi cabinets, wardrobes, beds, chairs, shoe cabinets and dining tables. Using knock-down fittings allows easy assembling and dismantling of components. It also facilitates transportation and storage. In addition, damaged parts can be replaced readily, so that production costs can be reduced. Examples of knock-down fittings include cam locks, interlocking fittings and screw-type fittings.

A knock-down fitting used on flat pack furniture

| # Type of fitting | # Characteristics |
| --- | --- |
| # Cam lock | # When assembling and dismantling, simply use a screwdriver to turn the eccentric outer shell; usually applied on quality furniture. |
| # Interlocking fitting | # Suitable for internal structures only and not supposed to be exposed. |
| # Screw-type fitting | # Usually applied on cheaper furniture made of chipboards. |

Characteristics of various types of knock-down fitting

A cam lock used on flat pack furniture

- (c)	Other Fasteners

In addition to those mentioned above, there are other fasteners designed and used based upon the needs. The following are some examples.

1.	With a hook on one side and a ring on the other side of each barricade, interlocked mills barriers can be formed when needed.

2	Hooking the belt of one stand to the receiver on the other stand, barrier stands can be inter-connected to keep people in line.

3.	A hook-and-loop fastener, or Velcro, consists of two components, one with tiny hooks and the other with even smaller loops. The two surfaces of the two components can be attached and detached repeatedly, facilitating the adjustment of tightness and position.

4. 	Ties and straps with different designs and strengths.

- II Equipment Used in Manufacturing Processes

Unless we are making a single or a small amount of products, product manufacturing usually needs to use production equipment with specific functions. The following are some common methods and equipment used in making products.

| Manufacturing process | Concept / Principle | Products |
| --- | --- | --- |
| Sand casting |  |  |
| Plastic injection moulding |  |  |
| Vacuum forming and blow moulding |  |  |
| Bending |  |  |

| Extrusion, drawing and rolling | Material Mould Extruded shape |  |
| --- | --- | --- |
| Compression moulding |  |  |
| Wood lamination and steam bending | Mould Bending along the grain of the wood |  |

- III Material Surface Treatment

Material surface treatment refers to the processing of the surfaces of materials for aesthetic, durability and safety purposes. For example, surface polishing of wooden objects should be carried out after cutting so as to prevent the splinters from hurting our hands. In general, when processing a material surface, the surface will be smoothed first and then a protective coating material will be applied to it.

1.	Surface Smoothing

The process of smoothing and flattening rough and uneven material surfaces is called surface smoothing. Different materials usually require different surface smoothing methods.

- (a) Metals

Burr

Flat file

After cutting processes (e.g. drilling) taking place on metals, uneven surfaces or burrs will always be formed which can be smoothed by using a hand file. When casting metal workpieces, there will always be excessive materials left at the gates. We can use a saw to remove the excessive parts and then use a hand file to smooth the edges. Emery cloth is also often employed to remove burrs and smooth a small area of surface.

- (b)	Wood

The surface of solid wood is always rough and contains knots. We can use a smoothing plane to plane the wood surface and then use a blade to scrap the knots.  Finally use sandpaper or a sander to polish the surface until it is smooth. The surface of plywood is usually covered with a veneer, so it only needs to be smoothed by sandpaper.  After cutting processes (e.g. sawing) taking place on wood, uneven surfaces or splinters will always be formed. We can use a smoothing plane or a hand file to carry out the smoothing process.

If there are gaps or small holes on a wood surface, we can use fillers to fill them and then smooth them. This method is also applicable to gaps caused by cracks, nail holes and joints.

- (c) Plastics

Excessive materials and rough edges will always be left on plastics after moulding and cutting respectively. These undesirable parts can be removed by using a blade or a file. Silicon carbide paper (from coarse to fine) can be used for smoothing the scratch marks left on the surface of a plastic material during processing. If a polishing wheel will be used for smoothing plastic surfaces, it is better to choose a softer material.

2.	Surface Coating

After smoothing, material surfaces may still be susceptible to damage. For example, iron surfaces will rust and wood surfaces will be scratched. In order to make the surfaces of materials more appealing and more durable, appropriate protective materials can be coated on the surfaces. There are a variety of surface coating methods, including electroplating, dip coating, painting, plastic coating, wax and protective coating, veneering and enamelling.

- (a)	Electroplating

Electroplating is a commonly used industrial metal coating method. Its principle is to employ electrolysis to deposit a layer of metal onto the surface of a workpiece made of another metal. Hard and shiny metals that are resistant to abrasion and oxidation are suitable plating materials. Common plating materials include nickel, copper, chromium, cadmium, silver and gold. After electroplating, the surface of the workpiece will become shinier and stronger.

An electroplating production line

Industrially, mass electroplating is usually carried out by using an electroplating production line. To ensure that the protective coatings can adhere onto the workpieces firmly, grease, dust and oxides on the workpiece surfaces are removed using water and chemicals before conducting the electroplating process

(b)	Dip Coating

Dip coating is another method to apply a metal coating to a workpiece made of a different metal. It differs from electroplating in the way that no electrolysis takes places.  One of the uses of dip coating is to plate a layer of zinc on steel plates (i.e. galvanisation) to prevent oxidation and rusting.

Galvanised steel sheets

In dip coating, metals with lower melting points are usually used as the plating materials, for example, tin, zinc, lead, aluminium and some alloys. Generally, the metal layers formed on the objects by dip coating are thicker, thus the objects can be used for a prolonged period even under corrosive conditions. Examples of application of dip coating include galvanised steel sheets and pipes.

Advance preparation: In addition to surface smoothing, before electroplating and dip coating, the following should be done on the objects to be plated.

- Remove the dust and debris

- Remove the grease and oxides on the workpiece surfaces by chemicals

- Rinse with water

- Dehumidify

- (c) Painting

Painting is a common method of surface coating to decorate and protect materials such as metals and wood. Paints will form a protective layer on material surfaces when dried. They are effective in preventing air from coming into contact with the material surfaces, so rusting of metals and rotting of wood will occur less easily. Paints can be categorised into natural (e.g. raw lacquer), artificial (e.g. enamel paints, lacquer and spray) and special paints (e.g. marine paints and rubber paints).

There are various ways of painting, including brushing, dip painting, spraying and steaming.  In schools and at home, brushing and spraying are often used.

(a) Painting tools

(b) Hand spray painting

Paints can be categorised into primers and topcoats according to their functions. Primers, e.g. shellac, do not have an attractive appearance generally but they provide better protection and adhesion. Topcoats, e.g. cobal varnish, are usually more appealing but they provide less protection and are less adhesive. Therefore, to maintain the beauty and durability of an object, paint the object with a primer before applying a topcoat.

As most of the thinners in paints are poisonous and flammable, for safety reasons, the following precautions should be strictly adhered to when painting.

- Keep the workplace well ventilated.

- Never light a fire or store flammable substances in the workplace.

- Put on a mask during painting.

- A large-scale painting work should be carried out in a workplace equipped with ventilating equipment.

- (d) Plastic Coating

Plastic coating refers to covering of metal products with plastics so that the products will become insulated and more resistant to rusting and wear. Moreover, using brightly coloured plastics can improve the appearance of the metal products. This is a simple and inexpensive method, and is thus often applied to household products such as wire grids in refrigerators, coat hangers and handles of hand pliers. Commonly used coating materials are thermoplastics such as polyvinyl chloride (PVC), nylon and polyethylene.

A coat hanger

Hand pliers

- (e)	Wax and Protective Films

Waxing the shell of a car with a waxing machines

Simply coating a protective layer on the surface of a material can achieve aesthetic and caring purposes. Wax, for example, is a commonly used protective layer material. Waxing is a simple and quick surface processing method, which can enhance the lustre of an object while keeping its original colour and texture. Resistance to heat and moisture will also result when waxing is applied on top of layers of various paints. For instance, the coatings of cars and the surfaces of wooden furniture finished with lacquer and varnish can both be waxed.

- (f) Veneering

Veneering is a process to protect and decorate wooden materials by covering them with a thin slice of wood. This process is often used to cover edges or small areas of wood surface. Wood with clear grain pattern is the mostly used raw material for producing veneer. While having a better outlook, the grains, however, may easily cause cracking when applying.

A veneer door

Before applying a piece of veneer, clean the wood surface. Apply contact glue and wait until it is slightly dried. Then, paste the veneer, which is a bit larger than the wood surface, and apply pressure with a wooden board. When the veneer is fixed, trim the excessive part with a file and sandpaper, and then a nice smooth surface will be resulted.

A beautiful enamelled vase

- (g) Enamelling

Clay pots with an enamel coating applied can reduce the accumulation of dirt

In the production of pottery and porcelain, a blank is first formed by firing and then a coat of enamel is applied on it. After which, the object is fired again. There are a wide variety of enamels. They are made from raw materials including quartz, borax and clay. Pottery and porcelain with an enamel coating applied have a vitreous appearance (glass-like and glossy), improving their aesthetic level and making them into truly decorative articles. Additionally, since the enamel surface is a glass layer that is almost non-porous, dirt can hardly gather and the strength of the products can be improved.

3. Surface Polishing and Buffing

- (a) Polishing

# Polishing is the process of turning rough surfaces of objects smooth. This process can decorate and make correction to the surfaces. For example, polishing can be applied when an object has an uneven thickness or when the surface of an object has deep scratch marks.  Besides, polishing the surface of a workpiece is a prerequisite procedure for creating an even coating.
Polishing can be done by means of hands or machines. Hand polishing is suitable for polishing small metal or plastic workpieces. First, use a rough file to smooth the workpiece surface; then, use a finer file to continue the filing process. File grains selected can start from ‘cross-filing’ and finish by ‘draw-filing’. Finally, use emery cloth with extra fine grains to achieve an even smoother finish.  Mechanical polishing, on the other hand, is to use a grinder to smooth the surface of a workpiece rapidly.

A grinder

- (b) Buffing

# Although a material surface is smoother after surface treatment, the colour of the material may appear dull due to the grinding traces all over the material surface. Buffing is to smooth a surface so delicately, to the extent that the surface becomes shining. This method is often applied to materials such as plastic, copper and aluminium products.

- A hand-buffing compound

# (b) A buffing machine

# Buffing can be done by means of hands or machines. Hand buffing refers to the use of fabric (cotton) and a gentle abrasive (a buffing compound) to rub the surface of a workpiece repeatedly. Buffing can only make a surface become shinier and smoother but it is not a smoothing process that makes a surface flat. Mechanical buffing, on the other hand, makes use of a buffing machine to buff the surface of a workpiece rapidly. When using a buffing machine, force applied should not be too hard, or else the workpiece may get too hot under excessive pressure and its surface may be damaged.

# For safety reasons, the following precautions should be strictly adhered to when using a polishing or buffing machine:

- Install safety shields on all rotating parts of a grinder or buffing machine.

- Wear safety goggles.

- Never buff sharp edges or corners.

- Allow only one user to use a grinder or buffing machine at one time.

IV Glossary of Terms

| Permanent joining | 永久接合 |  | Casting | 鑄造 |
| --- | --- | --- | --- | --- |
| Electric arc welding | 電弧焊接 |  | Injection moulding | 注射成形 |
| Soft soldering | 軟焊 |  | Vacuum forming | 真空成形 |
| Hard soldering | 硬焊 |  | Extrusion | 擠壓 |
| Silver soldering | 銀焊 |  | Compression moulding | 壓縮鑄造法 |
| Brazing | 銅焊 |  | Electroplating | 電鍍 |
| Flux | 焊劑 |  | Galvanised sheet | 鍍鋅片 |
| Rivet | 鉚釘 |  | Paint | 油漆 |
| Pop rivet | 拉釘 |  | Cobal varnish | 谷巴凡立水 |
| Seaming | 接縫法 |  | Plastic coating | 塑膠覆蓋 |
| Adhesive | 黏合劑 |  | Veneer | 薄木皮 |
| Semi-permanent joining | 半永久接合 |  | Enamel | 瓷釉；搪瓷；琺瑯 |
| Screw | 螺絲 |  | Polishing | 磨光 |
| Anchor screw | 錨式螺絲 |  | Buffing | 拋光 |
| Bolt and nut | 螺栓及螺帽 |  | Knock-down fastener | 裝拆扣件 |
| Washer | 墊圈 |  | Velcro | 魔術貼 |
| Wing nut | 翼形螺帽 |  |  |  |

V Interactive Information

|  | Website | Content |
| --- | --- | --- |
| 1 | https://www.youtube.com/results?search_query=%E9%9B%BB%E5%BC%A7%E7%84%8A%E6%8E%A5 | Electric arc welding |
| 2 | https://www.youtube.com/watch?v=nTu-n_JYjf4 | Soft soldering tutorial |
| 3 | https://www.youtube.com/watch?v=d7ZDymNemHU | Injection moulding |
| 4 | https://www.youtube.com/watch?v=CIQAKgTdDoE | A teaching video on electroplating |

After-class Activity

- Chairs and desks in a classroom are made by processing a variety of materials.

- Observe the chairs and desks carefully and then note down in detail every part or component (e.g. a screw) that is used to make up these furniture.

- Analyse and note down the production and processing methods of each part of the chairs and desks.

- Refer to the following four types of material processing methods when carrying out the analysis.

- Material cutting

- Material forming

- Material joining

- Material surface treatment

- Suggest ways to improve the processing methods so as to enhance the quality of the chairs and desks.

- Find 5 places at home where screws are used. Put down the name, shape and use of each of the screws.

| Name of the screw | Shape of the screw | Use of the screw |
| --- | --- | --- |

# Project Activity

# Design and Production

Open Day Souvenirs

- Design Brief

Your school principal or technological subject teacher asked you to design and make a number of refrigerator magnets as the open day souvenirs for visitors for the coming school year. These magnets should be made by injection moulding, similar to the in-moulded magnet products shown in the following figures.

These open day souvenirs will be dispensed from a toy capsule vending machine installed in the D&T room.

- Design Considerations

Your products should not exceed the size of 30 mm × 50 mm and not be thicker than 5 mm.  There is no limitation on the design of the products, but their shape must be kept simple, and their design should be related to a school or D&T activity.

First, you need to draw up a list of design specifications for your moulded products, which should be written on the worksheet provided by the teacher. This list of specifications should describe in full the style of your products, their uses and their target users. The following questions may help you set out your specifications.

- Whom are the souvenirs for?

- What theme will you use for the design?

- What will be the overall size of the souvenirs?

- What should be the colours of the souvenirs?

Before you start the design work, you should know what materials and equipment are available. To manufacture the products, you need to create either wire outline or laminated moulds. Information in Appendix 3 will guide you through each of these two options. You are going to use hot melt glue as the moulding material and a glue gun as the injection moulding device / equipment.

- Design Proposal

Having considered what materials are available for making the products, you need to start to propose the designs of the souvenirs and the moulds. The suggested steps are as follows:

- note your ideas down on the worksheet;

- together with your classmates and / or technological subject teacher, decide which ones are the best designs;

- prepare detailed full-scale working drawings of your designs on drawing paper; and

- finally, draw the working drawings, on which the making of the moulds will be based.

When developing design ideas, you should also consider the feasibility of mass production of your designs in an industrial environment.

- Mass Production

If you are about to produce more than one moulding, for instance, as a part of a mini-enterprise activity (an extended activity), you need to consider the manufacturing process. If making one moulding takes you 4 hours, it is likely that making 10 identical mouldings will take far less than 40 hours. This is because that once the mould is ready, repeating the injection moulding process will save production time.

Injection moulding is capital intensive. It is relatively costly and time consuming to make moulds.  However, once the mould is completed, producing thousands of mouldings can soon pay for itself!  Consider setting up a production line for your designs with the help of your classmates. You can use a block diagram or a flow chart to illustrate your production line on the worksheet.

- Quality Control

Checking for quality is very important when products are manufactured in quantity because a mistake in the manufacturing process will be repeated many times. You need to examine your first moulding meticulously for any defects. If you are producing a batch of identical mouldings, check one or two at random to ensure they remain consistent. Repair the mould and / or modify the working procedure if the quality of the moulding is unsatisfactory.  Write down the defects found and the suggested improvements on the worksheet.

Evaluating your work (mould and moulding)

There are a number of things to consider when evaluating the success of your work. Here are some questions to help you evaluate your work:

- Is the mould successful?  Does it enable you to produce what you intend to?

- Does the moulding meet the requirements of your brief and specifications?

- What is your estimated cost of the moulding?  How much would it cost to make ten identical mouldings as opposed to just one?

- Compare your product against the ones by your competitors in terms of quality, features, proposed selling price, etc.

。

# Appendix 1

Injection Moulding (School internal)

Large industrial injection moulding machines are expensive and not common for use in schools. Hot melt glue guns are sometimes used to simulate an injection moulding 'machine' in a school environment. When the glue stick is pressed through the barrel of a glue gun, the glue will be heated, and sticky hot melt adhesive will be squeezed out from the nozzle at the front of the glue gun.

A hot melt glue gun

Hot melt glue guns are not designed for performing injection moulding. However, they can be used as an ordinary injection moulding device to make your desired product (i.e. the souvenirs) if you find the glue material suitable. Hot melt glue sticks are now available in a range of bright colours. Although it is a flexible material, this property is very suitable for some moulded products.

# Appendix 2

Casting (School internal)

We can make a decorative magnet using simple tools and materials in schools. The following are the main steps in making a fish-shaped magnet.

| 1. Use modelling clay and simple tools to make the model of the object to be moulded. |  |
| --- | --- |
| 2. Use paper to make the mould box. |  |
| 3. Prepare the yellow plaster for making the mould. |  |
| 4. Pour the yellow plaster into the box until it covers the model. |  |

| 5. When the yellow plaster dries, remove the paper box. |  |
| --- | --- |
| 6. Remove the modelling clay model. |  |
| 7. Prepare the metal for casting (tin is used here). |  |
| 8. Pour the casting metal into the mould. |  |
| 9. After cooling, turn the set of materials upside down to demould. |  |

# Appendix 3

Industrial Injection Moulding

The surface of the moulding is a ‘mirror image’ of the mould surface. Therefore, the mould for use in injection moulding has to be made of steel with great precision so that every part of the mould matches accurately and has to be well finished. Any defects or tool marks on the mould will be reflected on the moulded product. The production costs of large moulds for products such as rubbish bins and television sets are very high. The major part of the costs is devoted to obtaining a good finish for the mould (see the figure below).

Steel moulds with a good surface finish

Sprue entry

Sprue

Part

A mould for making toy car wheels

- The moulds you can make in a school

Two types of moulds that can be made rather easily in a D&T workshop:

- Wire outline mould (for simple 2D shapes)

- Laminated mould (for 3D forms)

(I) Wire Outline Mould

A wire outline mould makes use of a metal wire (e.g. 3 mm wide by 1 mm thick) trapped between two metal plates, forming a cavity. The molten plastic (i.e. the melted glue) is injected through an injection hole on one side of the plate. A smaller hole, the indicator hole, near the injection hole is used to show when the mould is filled completely (see the figure below).

Indicator hole

Injection hole

Shape of the car body

Exploded view drawing of a wire outline mould

A wire outline mould can be made within a matter of minutes but it is limited to moulding objects with a flat shape. Besides, since plastic shrinks on cooling, a sunken spot will result where the molten plastic is injected. If this is not a preferred result, the wire shape should be adjusted to leave space for a sprue to form, which can be cut off from the finished product.

(II) Laminated Mould

A laminated mould consists of three or more plates clamped together. In a three-plate mould, a pattern will be cut out from the centre plate so that a cavity will be formed when this plate is sandwiched between the other two plates. In laminated moulds, patterns will be cut out from more than one plate such that a complex shape can be formed. The following figure shows a laminated mould for making a car-like key fob. One of the middle plates has two holes drilled on it for moulding the wheels of the car.

Wheels

Exploded view drawing of a laminated mould

# Appendix 4

# Simplified Insert Moulding

One way to allow your product to be attached to a refrigerator door is incorporating a small magnet into it. The process of incorporating an object, an insert, into an item after moulding is completed is known as insert moulding.

Magnet

Galvanised iron plate

鍍鋅鐵片

Insert moulding

You can buy ready-made magnetic plastic sheet as your raw material / component. However, the plastic-covered strip used in magnetic wall planners can also be used for insert moulding.  The strip can be cut into segments of length about 15 mm. Put one of the segments into the mould before adding the plastic. The molten plastic will form around the magnet and will hold the magnet securely upon hardening.

Working Procedure

When the mould is ready, follow the procedures listed below to mould your product.

- Spray a thin layer of silicon release agent on the two inner sides of the mould to prevent the molten plastic sticking to the surface of the mould.

- Tighten the mould with a jig and fixture after it is put into position.

- Place the nozzle of the hot melt glue gun at the injection hole and squeeze out the melted glue by pressing the trigger repeatedly.

- When the cavity is full, a small spot of glue will appear at the indicator hole. Stop injecting any more glue then.

- The mould conducts heat away from the plastic inside very quickly, so the mould can be opened a few seconds later. However, if a large quantity of plastic is injected, the mould will get very hot and may need to be cooled by water before opening.