Design and Applied Technology

Assessment Exercises

Elective Part

Module 1 - Automation

Technology Education Section

Education Bureau

(Trial version - Aug 2024)

Preface

This learning resource package was developed to provide Design and Applied Technology assessment exercises for teachers’ reference, so as to support the implementation of the DAT curriculum (S4-6) in schools and promotion of assessment for learning. This learning resource package includes: (i) Tips for Answering Questions (ii) Practice Questions (iii) Marking Scheme and Answering Guide

Suggestions for use

- Teachers may share the Tips for Answering Questions with students to enable them to become familiar with the common question style, as well as gain confidence in interpreting the questions and planning the completion time. - This learning resource package should not be regarded as the mock exam paper or related reference materials for the DAT public exams. Teachers may guide students to complete related practice questions after teaching a topic to help students master what they have learned. Students may also practise answering questions within the suggested time duration. Teachers are advised to adapt the materials according to the diverse learning needs of students if deemed necessary. - Marking schemes with suggested answers are provided for each question. For some ‘open-ended’ questions, example answers have been included. Students could refer to the marking schemes to understand about the answering requirements in each question. - After completion of the practice question, students could refer to the Answering Guide to further understand the points to note for the question concerned.

*This is the trial version of the learning resource package. Teachers are welcome to provide comments and feedback by sending email to: te_team4@edb.gov.hk

Table Of Content (i) Tips for Answering Questions 4 (ii) Question Paper 6 Elective Module 1: Automation (5 questions in total) 6 (iii) Marking Scheme and Answering Guide 21 Marking Scheme 21 Answering Guide 38 (iv) Sources and Acknowledgment 41

(i) Tips for Answering Questions

- General advice on answering questions

- Read the question carefully. Pay attention to all information provided in the question. It is a good practice to read the question twice to ensure you fully understand the answer requirements. - Look at the number of marks allocated to each item in the question and determine whether marks would be awarded point by point based on the number of answers required, or according to the level criteria of the answer. The former may only require the provision of the corresponding number of correct answers; while the latter may require determining the implicit requirements in the question carefully and the provision of answers in response to each requirement with more details to obtain marks at a higher level. - Underline the ‘command words’ and ‘key terms’ in the question before start. - For longer questions, students should spend some time to think and plan for the answers. - For questions to be answered in the Question & Answer booklets, students could sometimes make reference to the space given when assessing how much to write.

Example question:

Give two mechanical properties of mild steel that make it suitable for the construction industry. (2 marks) Specific situation connected to the major information Major information Command word Key term in this question As deduced from the total mark, it is likely that one mark would be awarded for each property correctly named

- Where most of the marks lost when answering a question?

- Misunderstand (some) requirements of the question - Ignoring (some) clues in the question - Missing bits of a question - Poor representation of information and/or annotations in diagrams - Inaccurate diagrams

- Understanding command words

- The command words in questions instruct you to provide the answers required. Therefore, you need to understand the meaning of the command words to prepare or the answers. The following are the commonly used command words in DAT:

| Command word | What does it mean |
| --- | --- |
| - Analyse | Examine in detail to show meaning, identify related elements and the relationship between them. |
| - Apply | To use previous learning and understanding in another familiar situation. |
| - Annotate | Add brief notes to a drawing, diagram or graph. |
| - Calculate | Work out from given facts, figures or information. Obtain a numerical answer showing the relevant stages in the working. |
| - Compare | Identify/comment on similarities and/or differences. |
| - Describe | State the points of a topic / give characteristics and main features. |
| - Develop | Take forward to a more advanced stage or build upon given information. |
| - Draw / Produce | Represent by means of accurate diagrams or graphs using drawing equipment. A ruler should be used for straight lines. Diagrams and graphs should be drawn to scale. |
| - Distinguish | Make clear the differences between two or more concepts or items. |
| - Explain | Set out purposes or reasons / make the relationships between things clear / say why and/or how and support with relevant evidence. |
| - Give / State | Provide an answer from a given source or recall from memory. Express in clear terms. |
| - Illustrate / (Using sketches to) Explain | Include examples or a diagram to show what you mean or demonstrate understanding of issues or concepts. |
| - Indicate | Point out the existence of something and provide its name. |
| - Justify | Support a case with reasons/evidence. |
| - List | Give a number of features or points without further elaboration. |
| - Name | Identify using a recognised technical term. |
| - Outline | A general description showing essential features. |
| - Sketch | Make a simple freehand and roughly proportional drawing showing the key features. |
| - Suggest | Apply knowledge and understanding to situations and propose a solution or other possible answers. |
| - Put forward | Provide an idea for consideration or discussion. |

(ii) Question Paper

Elective Module 1: Automation (5 questions in total)

| This question is about “Basics of Control Systems” and “Robotics”, and has a total of 25 marks. Suggested completion time is 25 minutes. |  |
| --- | --- |
| 1. | Figure 1 shows the control system of an obstacle avoiding toy car. The system is controlled only by an analog circuit without using any integrated circuits. |
|  | Figure 1 |
|  | The working principle of the obstacle avoiding toy car: - Initially, both motors receive the signal to move forward. The bump sensor will continuously send binary signals to the adjustable timers, (0) for no obstacle is encountered, (1) for an obstacle is encountered. - After encountering an obstacle, the timers will send a binary signal to the motors, (0) for forward rotation, (1) fors reverse rotation. - The timers are pre-set in such a way that after hitting an obstacle, the toy car will pause and move backward for 2 seconds; then the left motor will resume forward rotation, and the right motor will continue to maintain reverse rotation for 2 seconds, making the toy car turn right . - The toy car will repeat the above operation sequence until it clears the obstacle and then moves forward. |

|  | Figure 2 illustrates the operational sequence when the toy car hits an obstacle. |  |
| --- | --- | --- |
|  | Figure 2 |  |
|  | (a) | According to the operational sequence of the toy car shown in Figure 2, - indicate whether the control system of the toy car is an open-loop or closed-loop system, and give a reason. (2 marks) - complete the “timing diagram” in Figure 3 (6 marks) |
|  |  | Figure 3 |

|  | Figure 4 shows another toy car equipped with a microcontroller. |  |
| --- | --- | --- |
|  |  | Figure 4 |
|  | (b) | - Fill in the appropriate boxes with the names of the three appropriate components to complete the system control diagram of the toy car shown in Figure 5. (3 marks) |
|  |  | Figure 5 |
|  |  | Figure 6 |
|  |  | - Using annotated sketches, illustrate the working principle of ultrasonic sensors, and explain how the toy car can navigate around the obstacle shown in Figure 6 and continue on its way with the help of an ultrasonic sensor. (6 marks) |
|  |  | Figure 7 |
|  |  | - Using annotated sketches, illustrate the working principle of infrared sensors, and explain how the toy car can travel along the black line shown in Figure 7 with the help of infrared sensors. (6 marks) |
|  | (c) | Give an example on the application of each of the following robots that were equipped with microcontrollers: - Obstacle avoiding robot - Line following robot (2 marks) |

| This question is about “Basics of Control Systems” and has a total of 25 marks. Suggested completion time is 25 minutes. |  |  |
| --- | --- | --- |
| 2. | Figure 1 shows a system that prompts car drivers to fasten the seat belt before driving. The system is equipped with three sets of sensors. Table 1 shows the output of the system in different states. |  |
|  | Figure 1 |  |
|  | Table 1 |  |
|  | Input/Ouput; State; Output. Driver seat sensor; No driver on the seat; 0. Driver on the seat; 1. Engine on/off sensor; Engine is switched off; 0. Engine is switched on; 1. Seat belt sensor; Seat belt unfastened; 0. Seat belt fastened; 1. Warning device; Off; 0. On; 1. |
|  | If the driver has not fastened his/her seat belt before driving, the warning device will be activated. |  |

| (a) | (i) | Complete the following truth table to express the relationship between each input and output. (8 marks) |
| --- | --- | --- |
|  |  | Input; Output. Driver seat sensor; Engine switch sensor; Seat belt sensor; Warning device. 0; 0; 0. 1; 1; 1. |
|  | (ii) | Using logic gate symbols provided, connect the three sensor blocks shown in the figure to the warning device block to build a logic circuit that provides the required outputs in Table 1. The logic gate symbols can be used once, more than once, or not at all. Copy and complete the diagram onto your answer book to show your answers. (3 marks) |
|  | Figure 2 |  |

|  | Figure 3 shows a control system of an automatic door. |  |  |
| --- | --- | --- | --- |
|  | Figure 3 |  |  |
|  | Working sequence of the automatic door: - Door opening sequence: When the infrared sensor detects that someone enters the sensing area, door opening motor starts to open the doors. Once reaching the door opening limit switch, the door opening motor stops. - Strating timer: When the door touches the open limit switch and no one is detected in the sensing area, the timer starts counting. - Door closing sequence: After 8 seconds, if no one enters the sensing area, the door closing motor will start and the door will close automatically until reaching the door closing limit switch and the door closing motor will stop. If someone enters the sensing area during the door closing process, the system will immediately stop the door closing action and restart the door opening sequence. |  |  |
|  | (b) | - Complete the automatic door control system diagram in Figure 4. (4 marks) |  |
|  |  | Figure 4 |  |
|  |  | - The system utilises a programmable logic controller (PLC) to control the opening and closing of the doors. Write the names of the input/output components to of the following incomplete “ladder diagram”, and complete the state of each contact. (8 marks) |  |
|  |  | Door opening sequence |  |
|  |  | Strating timer |  |
|  |  | Door closing sequence |  |
|  |  | Figure 5 |  |
|  |  | - State two disadvantages of using infrared sensors to control automatic door opening and closing. (2 marks) |  |

| This question is about “Pneumatics” and “Programmable Control Systems”, and has a total of 25 marks. Suggested completion time is 25 minutes. |  |
| --- | --- |
| 3. | Figure 1 shows the bottle filling and full bottle detection parts of an automatic beverage filling production line. |
|  | Figure 1 |
|  | Working process of bottle filling and full bottle detection: - When the start button (SB1) is pressed, the conveyor belt is driven by the drive motor to transport empty bottles. - When the bottle detection sensor (P0) detects and locates an empty bottle, the conveyor belt stops. 1 second later, the filling equipment starts working. - After 5 seconds, the full bottle detection sensor (P1) checks whether the bottle is fully filled. If the bottle is full, the filling is complete; if not, filling continues until the full bottle detection sensor (P1) detects that the bottle is full. - When filling is complete, the conveyor belt resumes running until the bottle detection sensor (P0) detects an empty bottle again. The conveyor belt will then stop and start filling again. - When the stop button (SB2) is pressed, the production line stops. |

|  | (a) | The flow chart below shows the operation of the production line. Fill in steps to to complete the flow chart. (5 marks) |
| --- | --- | --- |
|  | (b) | An inspection system is required for the automatic beverage filling line in Figure 1 to check that the empty bottles are clean and undamaged before filling. The inspection system can output an electric signal to activate a pneumatic bottle rejection system that will separate the stained and damaged bottles from the conveyor belt. Using annotated sketches, show the following systems, list the major components and explain the working principles: - Clean and damage inspection (6 marks) - Bottle rejection (6 marks) |

|  | (c) | Using appropriate pneumatic symbols shown in Figure 2, draw annotated pneumatic circuit diagram to illustrate the bottle rejection system you have suggested in (b)(ii). Each symbol can be used once, more than once or not at all. (6 marks) |
| --- | --- | --- |
|  |  | Figure 2 |
|  | (d) | Compare the performance of the following two actuators in performing linear actuation: - Pneumatic actuator - Electric actuator (2 marks) |

| This question is about “Pneumatics” and has a total of 25 marks. Suggested completion time is 25 minutes. |  |  |  |
| --- | --- | --- | --- |
| 4. | Figure 1 shows the control system of the barrier for the entrance/exit of a car park. The barrier is raised and lowered by a single-acting cylinder. Figure 2 shows the pneumatic circuit diagram for controlling the single-acting cylinder. The car park attendant controls the raising and lowering of the barrier with a push button. |  |  |
|  | Figure 1 |  | Figure 2 |
|  | (a) | Refer to Figures 1 and 2, use annotated pneumatic circuit diagrams to show and explain the state of the valve, piston, and barrier under each of the following conditions: - When the button is pressed. (4 marks) - When the button is released. (4 marks) |  |
|  | The control system of the barrier will be replaced by a double-acting cylinder. As shown in Figure 3, the double-acting cylinder has two air inlets/outlets, and the pressure of the air is used to actuate the outstroke (forward stroke) and the instroke (retract stroke). |  |  |
|  | Outstroke Instroke |  |  |
|  | Figure 3 |  |  |
|  | Figure 4 shows the modified pneumatic circuit diagrams of the control system of the barrier. The piston of the double-acting cylinder cannot return to its original position by itself after actuation, and needs to be controlled by adding a 5-way 2-position directional control valve. |  |  |
|  | Figure 4 |  |  |
|  | (b) | Add a 5-way 2-position directional control valve in Figure 4 so that the piston can be pushed back to its original position after actuation. Complete the diagram and show all components in the figure in correct positions and conditions. (7 marks) |  |
|  | (c) | Using annotated sketches, explain how the 5-way 2-position directional control valve would work under each of the following conditions: - During the outstroke of the piston - During the instroke of the piston (4 marks) |  |
|  | (d) | Suggest a pneumatic component that can regulate the speed of the barrier when it is lowered. Using annotated sketches, add the suggested pneumatic component to the answer of (b) and show all components in the diagram in the correct positions and conditions. (4 marks) |  |
|  | (e) | State two advantages of pneumatic control systems. (2 marks) |  |

| This question is about “Robotics” and has a total of 25 marks. Suggested completion time is 25 minutes. |  |  |
| --- | --- | --- |
| 5. | Figure 1 shows a painting robot arm. |  |
|  | Figure 1 |  |
|  | (a) | - Name the type of robot shown in Figure 1. (1 mark) |
|  |  | - Use annotated line diagrams, show the movements of the robotic arm’s axes/links.(excluding the end effector for holding the brushes) (6 marks) |
|  |  | - Use the robotic arm as an example, explain what is meant by “degrees of freedom” (DoF) and analyse its degrees of freedom (DoF). (3 marks) |
|  |  | - Compare pneumatic and electric drive systems and suggest a suitable drive system for the robotic arm. State three performance requirements that would explain why the drive system is suitable. (5 marks) |

|  | Figure 2 shows an industrial robot. |  |
| --- | --- | --- |
|  | Figure 2 |  |
|  | (b) | - Name the type of robot shown in Figure 2. (1 mark) |
|  |  | - Use the robot as an example, explain what is meant by “work envelope”. Using annotated line diagrams, show the movements of the robot’s axes (excluding the gripper) and its “work envelope”. (6 marks) |
|  |  | - State two advantages of this type of robot. (2 marks) |
|  |  | - Give one example of the application of this type of robot. (1 mark) |

(iii) Marking Scheme and Answering Guide

Marking Scheme

|  |  |  |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | (a) | (i) | Toy car’s control system: |  |  |
|  |  |  | - Open loop system (1M) - Reason: Output is fully controlled by system input, no feedback loop (1M) |  | (2) |
|  |  | (ii) | Timing diagram: |  |  |
|  |  |  | 6 operational procedures + annotations (@1×6 = 6M) |  |  |
|  |  | 2 4 0 |  |  | (6) |
|  |  |  | - Commanding toy car to move backward: After bumping into an obstacle, the left and right motors (wheels) pause, the left and right motors rotate in reverse direction at the same time for 2 seconds. - Commanding toy car to turn right: The left motor resumes forward rotation, the right motor continues to maintain the reverse rotation for 2 seconds. - Commanding toy car to resume forward movement: Left motor continues forward rotation, right motor resumes forward rotation. |  |  |
|  | (b) | (i) | System control diagram: (@1×3 =3M) |  |  |
|  |  |  |  |  | (3) |
|  | (b) | (ii) | Working principle of ultrasonic sensors: - The transmitter of the ultrasonic sensor generates sound waves (1M) and transmits through the air. When it reaches an obstacle, the signal is reflected back. The receiver receives the reflected wave (1M) and sends the signal to the microcontroller for processing. - The distance between the obstacle and the ultrasonic module is calculated by taking the time difference between the transmitted sound wave and the received return wave. (1M) |  | (6) |
|  |  |  | How the toy car navigates around the obstacle: - When the ultrasonic sensor of the toy car detects an obstacle within a predetermined distance (e.g., 20cm) ahead of its planned route, it will slow down and stop moving forward (1M), and turn left and right to detect that there is no obstacle in that direction (1M), and then update the path in real time, and continue to the side without obstacles. (1M) |  |  |
|  |  | (iii) | Working principle of infrared sensors: |  |  |
|  |  |  | - The infrared line tracking sensor will continuously emit infrared (1M) and receive the infrared signal reflected from the ground (1M) and send the signal back to the microcontroller for processing. - By measuring the strength of the reflected infrared signal, it is able to determine whether the ground is white (strong reflected signal) or black (weak reflected signal). (1M) |  | (6) |
|  |  |  | How the toy car follows the black line: - Situation (A) need to move straight: Under normal conditions, if no sensor detects the black line, and both sensors reflect strong signals, the motor drive module will instruct both motors to run, and the toy car will move forward. (1M) - Situation (B) need to turn left: Reflected signal from the left sensor is weak, but the signal from the right sensor is strong, in this case only the right motor runs, and the toy car will turn left. (1M) - Situation (C) need to turn right: The reflected signal of the left sensor is strong, but the signal of the right sensor is weak, in this case only the left motor runs, and the toy car will turn right. (1M) |  |  |
|  | (c) | Example of robots using microcontrollers: @1×2=2M |  |  |  |
|  |  | (i) | Obstacle avoiding robot: (Any one of the following) - Removing explosives in hazardous environment to avoid personal injury. - Automatically vacuum and sweep the floor in the home environment, to help deal with tedious and repetitive housework. |  | (2) |
|  |  | (ii) | Line-following robot: (Any one of the following) - Deliver food or daily necessities to the wards in the hospital, for sparing work of the medical staff. - Help deliver food in restaurants, for saving manpower. |  |  |
|  |  |  | Total： |  | 25 marks |

|  |  |  |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 2. | (a) | (i) | Complete the truth table to express the relationship between each input and output. |  |  |
|  |  |  | Input; Output. Driver seat sensor; Engine on/off sensor; Seat belt sensor; Warning device. 0; 0; 0; 0. 0; 0; 1; 0. 0; 1; 0; 0. 0; 1; 1; 0. 1; 0; 0; 0. 1; 0; 1; 0. 1; 1; 0; 1. 1; 1; 1; 0. (Each row of combination @1×8=8M) |  | (8) |
|  |  | (ii) | Logic circuit: |  |  |
|  |  |  | - Each logic gate symbol in the correct position with correct input/output connection line @1×3=3M |  |  |
|  |  |  |  |  | (3) |
|  | (b) | (i) | System control diagram: @1×4=4M |  |  |
|  |  |  |  |  | (4) |
|  |  | (ii) | Ladder diagram: |  |  |
|  |  |  | - Name of components + contact point conditions (@1×8=8M) |  |  |
|  |  |  |  |  | (8) |
|  |  | (iii) | Disadvantages of using infrared sensors to control automatic doors: (Any two of the following) (@1×2=2M) |  |  |
|  |  |  | - Susceptible to malfunction due to interference from other heat sources and light sources. - When the ambient temperature is close to the human body temperature, the sensitivity of the sensor will drop significantly and cause temporary failure. - Passive infrared penetration is relatively poor. When the infrared radiation of the human body is blocked by other objects, it is not easily received by the detector. |  | (2) |
|  |  |  | Total: |  | 25 marks |

|  |  |  |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 3. | (a) |  | Flow chart: @1×5 =5M |  |  |
|  |  |  | - Conveyor belt stops 1s - Full bottle detection sensor detects whether the bottle is full? - Yes - No - Filling equipment continues working |  | (5) |
|  | (b) | (i) | Clean and damage inspection system: (e.g.) |  |  |
|  |  |  | Major components: - High resolution camera, photoelectric sensor, computer, image analysis software. (@1×4=4M) Working principle: - Using visual inspection technology and several cameras to take pictures of moving empty bottles from multiple angles. (1M) - Input the collected images and videos into the computer, and use image analysis software for image recognition and analysis to detect in real time whether there are cracks, dirt and foreign objects on the inner wall of the bottle. (1M) |  | (6) |
|  |  | (ii) | Bottle rejection system: (e.g.) |  |  |
|  |  |  | Major components: - Computer/controller, solenoid valve, pneumatic rejector, collection box. (@1×4=4M) Working principle: - A row of bottles are transported by the conveyor belt to go through the inspection equipment. The inspection equipment sends a signal to the computer indicating that the bottle passed inspection, or to the controller if the bottle failed inspection. (1M) - When a bottle is rejected, the operating signal from the controller will instruct the rejector to push the bottle out of the conveyor belt and drop it into the collection box in front. (1M) |  | (6) |
|  |  | Schematic diagram of the two systems: |  |  |  |
|  | (c) |  | Pneumatic circuit diagram of the bottle rejection system: |  |  |
|  |  |  |  |  | (6) |
|  |  |  | - Appropriate components: (e.g.) double-acting cylinder, 5-way 2-position directional control valve, spring return valve, direct acting solenoid valve (@1×4=4M) - Correct piping lines (1M) - Inlet/exhaust (1M) |  |  |
|  | (d) |  | Compare the performance of pneumatic actuators and electric actuators in performing linear actuator work: (Any one of the following) (@2×1 =2M) |  | (2) |
|  |  |  | Precision: - Pneumatic actuators convert the energy formed by compressed air into linear motion. Since air is compressible, the positioning accuracy of pneumatic actuators is relatively low. (1M) - Electric actuators are powered by stepper motors to produce linear motion with high control and positioning accuracy. (1M) |  |  |
|  |  |  | Force: - Pneumatic actuators use the energy created by compressed air to generate considerable force from relatively little pressure. (1M) - The force of an electric actuator is generated by an electric motor, which converts electrical energy into mechanical torque. (1M) |  |  |
|  |  |  | Responsiveness: - Pneumatic actuators operate with fluid power. Air is compressible, and it takes time to build up pressure, causing a delay in reaction time. (1M) - Force of the electric actuator is generated almost instantaneously by the electric motor torque, so there is no reaction time delay. (1M) |  |  |
|  |  |  | Total: |  | 25 marks |

|  |  |  |  |  |  | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 4. | (a) | (i) | When the button is pressed: |  |  |  |
|  |  |  | - Valve supplies air to single-acting cylinder (1M) - Pushing out the piston rod (1M) - Barrier raised (1M) - Valve and piston in the pneumatic circuit diagram are in the correct state. (1M) |  |  | (4) |
|  |  | (ii) | When the button is released: |  |  |  |
|  |  |  | - Valve exhausts and returns to original position. (1M) - Piston returns to original position. (1M) - Barrier lowered to original position. (1M) - Valve and piston in the pneumatic circuit diagram are in the correct states. (1M) |  |  | (4) |
|  | (b) | Pneumatic system circuit diagram after adding the 5-way 2-position directional control valve: |  |  |  |  |
|  |  | - Correct symbol of the 5-way 2-position directional control valve (1M) - Corrrect piping lines (port to port) (2M) - All components in the figure are in the correct positions and states: including double-acting cylinder, 5-way 2-position directional control valve, valve A, and valve B. (@1×4 = 4M) |  |  |  | (7) |
|  | (c) | Operation of the newly added component: |  |  |  |  |
|  |  | (i) | Outstroke: |  |  | (4) |
|  |  |  |  | - In this state, the main air flows through the valve from port 1 to port 4. (1M) - All the air on the other side of the piston is able to exhaust through the valve from port 2 to port 3. (1M) - The 5/2 valve will cause the double-acting cylinder to outstroke or hold the piston in the positive position. |  |  |
|  |  | (ii) | Instroke: |  |  |  |
|  |  |  |  | - In this state, the main air flows through the valve from port 1 to port 2. (1M) - All the air within the cylinder is able to exhaust through the valve from port 4 to port 5. (1M) - The 5/2 valve will cause the double-acting cylinder to instroke or hold the piston in the negative position. |  |  |
|  | (d) | Suggest a pneumatic component that can regulate the speed of the barrier when it is lowered: |  |  |  |  |
|  |  |  | Name of the component: Flow control valve (1M) - Correct symbol of the flow control valve (1M) - Corrrect piping lines (2M) - All components in the figure are in the correct positions and states (1M) |  |  | (4) |
|  | (e) |  | Advantages of the pneumatic system: |  |  |  |
|  |  |  | (Any two of the following) (@1×2 =2M) |  |  |  |
|  |  |  | - High Efficiency: Compressed air can be used anywhere in the atmosphere without restriction; compressed air is not affected by distance and can be easily transported by pipelines; compressed air can be released in the atmosphere without recycling after use. - High reliability: Because of fewer moving parts. - Environmentally friendly and clean: Pneumatic system will not produce pollutants during operation; pneumatic systems can be operated in an operating environment that requires high cleanliness, such as: high-precision integrated circuit production lines and food production lines. - Safe: Pneumatic systems are safe compared with other systems. For example, we cannot use equipment with electronic components for paint spraying because many electronic components generate sparks and this could cause the paint to catch fire. - Simple in design and easy to adjust: Linear and rotary motion speeds of pneumatic systems can be easily adjusted with fewer restrictions. Suitable for automatic control systems with simple operation. |  |  | (2) |
|  |  |  | Total: |  |  | 25 marks |

|  |  |  |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 5. | (a) | (i) | Type of the robot arm: |  |  |
|  |  |  | - Articulated robot (1M) |  | (1) |
|  |  | (ii) | Line diagram and degrees of freedom of the robot arm: |  |  |
|  |  |  | - Showing movements of the four Joints / Axes (1 to 4) (4M) and Link (1). (1M) - Using appropriate notation. (1M) |  | (6) |
|  |  | (iii) | Meaning of degrees of freedom as related to the robot arm: |  |  |
|  |  |  | - Refers to the number of independently movable joints / links of the manipulator (1M), and types of movements, including rotational or translational (linear) types. (1M) |  | (3) |
|  |  |  | - Degrees of freedom: 4 or 5 (1M) - (Remarks: Joints (1 to 4) have 4 DoF, which can basically provide the flexibility of the required movement; and Link (1) is optional, the purpose is to expand the working range of the robot arm and to increase its DoF.) |  |  |
|  |  | (iv) | Suitable drive system for the robotic arm: |  |  |
|  |  |  | Name: Electrical (1M) Reason: (Any one of the following) (1M) - Pneumatic and hydraulic systems are susceptible to temperature changes in the environment, while electric systems are relatively stable. - Compared with pneumatic and hydraulic systems, electric systems require less maintenance. |  | (5) |
|  |  |  | Performance requirements when selecting driver for the robot arm: (Any three of the following) (@1×3=3M) |  |  |
|  |  |  | - Convenient energy - High control precision - Strong driving force - Low operation noise |  |  |
|  | (b) | (i) | Type of robot: |  |  |
|  |  |  | - Cartesian robot (1M) |  | (1) |
|  |  | (ii) | Meaning of work envelope: |  | (6) |
|  |  |  | - A Cartesian robot’s work envelope is defined by its rectangular structure. (1M) The robot works within the boundaries of the X-Y plane, and the actual operating height is limited by the ceiling and the height of the work envelope. (1M ) |  |  |
|  |  |  | Line diagram: |  |  |
|  |  |  | - Showing X, Y, Z axes and the directions of movements (@1×3=3M) - Work envelope: Cuboid / rectangular box shape (1M) |  |  |
|  |  | (iii) | Advantages of this type of robot: (Any TWO of the following) (@1×2=2M) |  |  |
|  |  |  | - Movements are highly accurate and repeatable. - Allow for long stroks (of approx 2m) - High load carrying capability |  | (2) |
|  |  | (v) | Application example: (Any one of the following) (1M) |  |  |
|  |  |  | - Computer Numerical Control (CNC) machine tooling - 3D printing - Material handling: e.g., pick and place, stacking |  | (1) |
|  |  |  | Total: |  | 25 marks |

Answering Guide

| Question No. |  |
| --- | --- |
| 1. | Timing Diagram |
|  | - Timing diagram focuses on showing the state or value changes of different objects over a period of time. - The series of vertical lines on the far left is called “lifeline”, showing from top to bottom the state of the objects changes over time in different phrases. - From left to right, the “timeline” describes the behaviour of objects and the dynamic collaboration between objects within a specific period time. |
| 2. | Logic Circuit Diagram and Truth Table |
|  | - A logic circuit diagram illustrates how different types of logic gates are connected to form a digital circuit to perform a particular task. - A truth table shows the final outputs that the devices produce when the signals are sent to their input points. |
| 2. | Ladder Diagram |
|  | - Ladder diagrams have two vertical “power rails” at both ends and a number of horizontal “rungs”. The “current” of the ladder diagram flows from the “positive power rail” on the left to the “negative power rail” on the right through the rungs to simulate the flow of current from the live wire to the ground wire in the circuit diagram. - Each “rung” defines the operation of the control process, including a series of logic input conditions (contacts) to trigger outputs (coils/function blocks). The number of “rungs” depends on the number of control circuits that is required. |
| 4. | Pneumatic Circuit Diagram |
|  | - A pneumatic circuit diagram can be divided into three levels according to their function: Power level, logic level and signal input level. When drawing a circuit diagram, pneumatic components with different functions should be placed at different levels and positions to clearly show the relationship between the components. |
| 5. | A Robot Arm’s Degree of Freedom (DoF) |
|  | - The degree of freedom of a robot arm represents the flexibility of its movement, but as the degree of freedom increases, its structure will become more complicated. - The best way to determine how many degrees of freedom a robotic arm should have, is to determine the minimum number of axes required for it to function adequately. |

(iv) Sources and Acknowledgment

Questions:

| Question | Figure | Source |
| --- | --- | --- |
| 1 | 1 - 3 | EDB S4-6 Design and Applied Technology - Learning and Teaching Resources Module 1 – Automation |
|  | 4, 6 & 7 | EDB Robotics Project Learning - Learning and Teaching Resources |
| 5 | 2 | https://grabcad.com/library/cartesian-arm-robot-1 |

Marking Scheme:

| Question |  | Source |
| --- | --- | --- |
| 1 | (b) (ii) & (iii) | EDB Robotics Project Learning - Learning and Teaching Resources |

We are grateful to organisations for permission to include the material in this assessment exercises. Every effort has been made to trace copyright. However, in the event of any inadvertent infringement due to errors or omissions, copyright owners are invited to contact us so that we can come to a suitable arrangement.