Fibres and Fabrics

- Materials for Clothing and Textiles

- clothes and textiles can be made from different materials

- most of them are made from fabrics which are composed of a variety of fibres

- generally, types of fibres are classified based on their sources

- there are three main groups of fibre: natural fibre, regenerated fibre and man-made fibre

- natural fibres

- plants

- cotton

- cotton fibres are seeds hair

- very fine fibre, flattened tube like thread

- Sea-Island cotton is the longest cotton fibre and Asian cotton is the shortest

| Fibre identification: burning test | - burns quickly with afterglow - smells like burnt paper - pale grey residue and powdery ash |
| --- | --- |
| Properties | - strong and durable - poor elasticity and creases easily - can be damaged by strong acids |
| Clothing comfort | - cool to wear - good moisture absorption - dries slowly |
| After care | - can be boiled and ironed at high temperature - may shrink |
| Typical fabric | - calico, corduroy, denim, gingham, drill, terry toweling |

- flax / linen

- fibres are extracted from the stalks

- coarse fibre bundle, cylindrical shape with nodes / joints

| Fibre identification: burning test | - burns quickly with afterglow - smells like burnt paper - pale grey residue with powdery ash |
| --- | --- |
| Properties | - very strong fibre and durable, even stronger when wet - brittle and poor in flexibility - creases badly - can be damaged by strong acids |
| Clothing comfort | - cool to wear - absorbs water quickly and fast drying |
| After care | - can be boiled and ironed at high temperature - may shrink |

- animals

- wool

- wool is a kind of animal fibre taken from fleece of sheep and goats

- wool fibre is light, ranges from fine to coarse, surface with overlapping scales and crimps

- the finest and softest wool comes from Merino sheep

| Fibre identification: burning test | - small and sputtering flame, self-extinguishing - smells like burning hair - black residue with friable cinder |
| --- | --- |
| Properties | - medium strength and not very durable - excellent resilience - felting occurred when affected by heat and moisture - weakened by alkalis and damaged by chlorine bleach |
| Clothing comfort | - warm to wear - good moisture absorption - water repellent |
| After care | - will shrink and felt - cannot be bleached by chlorine bleach - iron at moderate temperature - do not tumble dry or dry in direct sunlight |
| Typical fabric | - felt, tweed, gabardine - Woolmark, Woolmark Blend, Wool Blend |

- hair

- hair fibres possess similar structure and properties as wool

- the care of hair fibres is also very similar to that of wool

- hair fibres include camel hair, mohair, cashmere, alpaca, llama, vicuna and angora (rabbit hair)

- silk

- silk is obtained from cocoons of silkworm larvae

- silk is triangular in cross-section with round corners, smooth and rod like surface with some striations

| Fibre identification: burning test | - small flame, slowly self-extinguishing - smells like burning hair or horn - black residue with friable cinder |
| --- | --- |
| Properties | - strong, durable, light and elastic - weakened when wet - fairly crease-resistant - damaged by alkaline and chloride bleach |
| Clothing comfort | - both cool and warm - good moisture absorption |
| After care | - wash with gentle detergents with minimum agitation and cool rinse - cannot be bleached by chlorine bleach - do not tumble dry or dry in direct sunlight - iron with dry iron at low temperature |
| Typical fabric | - chiffon, crepe, satin, voile |

- regenerated fibres

- the basic principle of producing regenerated fibres is the dissolution of cellulose and the regeneration of fibres through fibre spinning

- the process produces filament yarns which can be chopped into staple to imitate natural fibres

- recently, renewable resources such as bamboo are used for regenerated fibre production

- acetate

- acetate is made from cotton linters

- acetate is white fibre with longitudinal, striations and irregular cross-section

| Fibre identification: burning test | - burns quickly and melts in a flame - with an acidic smell - with hard and black residue |
| --- | --- |
| Properties | - low strength, even weaker when wet - poor elasticity - creases and shrink easily - melts at high temperature |
| Clothing comfort | - not very warm - low moisture absorption - fast drying |
| After care | - sensitive to dry heat - do not bleach and tumble dry |
| Typical fabric | - brocade, satin, taffeta |

- viscose rayon

- viscose rayon is made from wood pulp

- viscose rayon is white, lustrous fibre with irregular cross section

| Fibre identification: burning test | - burns quickly with bright flame and afterglow - smells like burning paper - with pale grey powdery ash |
| --- | --- |
| Properties | - lower strength and durability than cotton - poor elasticity - shrink and creases easily - degraded by acids and alkalis - melts at high temperature |
| Clothing comfort | - cool to the touch - good moisture absorption |
| After care | - wash in warm water - iron at low temperature - do not bleach and tumble dry |
| Typical fabric | - lustrous and crepe fabric - cotton, linen, silk and wool-like fabric |

- synthetic fibres

- synthetic fibres refer to fibres obtained from petroleum

- new fibres have evolved based on chemicals extracted from corn, sugar beet, soya bean, etc

- polyester

- polyester is white fibre with different cross sections - round, tri-and multilobal, oval and hollow

| Fibre identification: burning test | - melts and shrinks from the flame - forms a brownish mass - hard and uncrushable residue |
| --- | --- |
| Properties | - good strength and durable - good elasticity and crease resistant - melt at high temperature |
| Clothing comfort | - poor moisture absorption - fast drying |
| After care | - wash in warm water - do not bleach - iron at low temperature |

- nylon

- nylon is transparent and resembles a glass rod with round cross section

| Fibre identification: burning test | - shrinks and melts away from flame with fibre-forming drips - hard and uncrushable residue |
| --- | --- |
| Properties | - very strong and durable - good elasticity and wrinkle resistant - melts at high temperature - affected by concentrated acids and sunlight |
| Clothing comfort | - poor moisture absorption - windproof and water repellent |
| After care | - wash in cool water - do not bleach and tumble dry - iron at low temperature |
| Typical fabric | - Nylon 6, Nylon 6,6 |

- acrylic

- acrylic fibre is commonly used as wool substitute as its resilient resembles to that of wool but without felting and shrinkage tendency

- acrylic fibres crimp with striations and may be smooth or twisted surface

| Fibre identification: burning test | - shrinks and burns with a sooty flame - with melting, dripping and forms black beads - pungent smell - hard and unbreakable residue |
| --- | --- |
| Properties | - strong and durable when compare with cotton and wool - moderate elasticity and wrinkle resistant - melts at high temperature |
| Clothing comfort | - warm, insulating qualities similar to wool - low absorbency - fast drying - prone to static |
| After care | - wash in cool water - iron at low temperature |
| Typical fabric | - usually blended with other fibres, such as cotton, linen, viscose rayon, wool and silk |

- Ways to Produce New Textile Materials – Blending

- blending is mix two or more fibres together

- the advantage of fibre blending is that poor properties of one component can be compensated by good ones of the other component

- e.g. some natural fibres shrink and wrinkle after laundering, blending with man-made fibres can reduce shrinkage and wrinkle of natural fibres

- Fabric Construction

- fabric is constructed from yarns

- yarns are threads which are made from smaller threads

- fabrics are readily made materials for the production of various kinds of textile products

- woven fabrics

- woven fabrics are made from interlacing two sets of yarns at a right angle

- the process is called weaving

- yarns parallel to length are called warps

- yarns parallel to width are called wefts

| Plain weave | 2/1 S-twill | 2/2 Z-twill |
| --- | --- | --- |
| Poplin | satin | sateen |

- knitted fabrics

- knitted fabrics are formed by interlocking yarn loops

- the process is called knitting

- the direction is called course

- the direction parallel to length is called wale

Front Side		Back Side

- non-woven fabrics

- the formation of non-woven fabrics is by compressing fibres together through:

- mechanical bonding

- glue

- heat fusion

4. 	Colouration

(a)	dyeing

(b)	printing

| Flatbed screen printing | Rotary screen printing |
| --- | --- |

5. 	Finishing

(a)	mechanical finishing

- raising

- calendaring

- embossing

- fulling

(b)	chemical finishing

- water repellency

- water proofing

- stain resistance

- antibacterial finishing

- antistatic finishing

- nanotechnology (e.g. stain proof finishing, antimicrobial finishing, odorless textile, fragrance release finishing, skin care finishing)