Preface

To support the implementation of the Enriched Technology Education Key Learning Area Curriculum (Enriched Curriculum) at junior secondary level, the Technology Education Section of Curriculum Development Institute, Education Bureau, has developed a set of learning and teaching resource materials covering technological subjects learning element modules for teachers’ reference and use.

The purpose of providing this set of teaching resource materials is to enable teachers to adopt the related technological subjects learning element modules under the Enriched Curriculum for students to acquire thorough understanding and mastery of the three key aspects in Technology Education Key Learning Area, viz. the technological understanding, technological capability and technological awareness, using a flexible approach with reference to the suggested learning progress.

The content of this learning and teaching resources was compiled with project approach.  It gives students a purposeful and meaningful learning context through a series of diversified activities such as design projects, case studies, technological exploration and simulation experiments, and thus arouses their interest in technology and devotion in learning, as well as nurtures their ability in problem-solving, realisation, innovation and spirit of entrepreneurship.

For comments and suggestions related to this set of learning and teaching resources, please send to:

Chief Curriculum Development Officer (Technology Education)

Technology Education Section

Curriculum Development Institute

Education Bureau

Room W101, West Block, 19 Suffolk Road

Kowloon Tong

Hong Kong

Compilation and Authoring:

Mr. LAU Kwok-kuen

Retired Design and Technology Panel Head

Chinese to English Translation and Typesetting:

Vocational Training Council

The copyright of this set of teaching resource materials belongs to the Education Bureau of the Government of the Hong Kong Special Administrative Region.

Schools and educational organisations are welcome to use the content of this set of teaching resource materials for non-profit educational purposes. Teachers can copy, enrich or delete the contents in order to meet the needs of teaching, but in all cases, acknowledgement must be clearly stated.

Otherwise, all rights are reserved, and no part of these materials may be reproduced, or transmitted in any form or by any means without the prior permission of the Education Bureau.           © Copyright 2014

Relevant Knowledge

I. Structure

1. Introduction to structure

A structure is created by combining many lightweight parts, which can be used to withstand a greater load.  The parts that make up the structure are called members. Many buildings and machinery are all structures, such as: cable towers, buildings, bridges and cranes.

(a) The cable tower		   (b) The crane

2. Forces on structure

When the structure is to withstand load, the nature of forces being applied to the members can be classified into compression, tension, bending, twist and shear, etc.

- Compression

Compression leads to the shortening of distance between members along the axis where the force is applied. For example, the pillar is being compressed and deformed.

Effort

Effort

Figure 1	 (a)	Compressive force

(b) Rubber being compressed

(b) Tension

Tension leads to prolonged distance between members along the axis where the forces are applied. For example, the forces exerted by the hangers/stringers of suspension bridges and the iron chains of the swings.

Effort

Effort

Figure 2  (a) Tensile force

(b) Rubber being elongated

(c) Bending

Member of structure would become bended when the force is applied at the lateral face perpendicularly. It is compressed at the side of the exerting force whereas there is tension on the other side. For example, the bookcase shelf is bended under pressure.

Force

Compression

Compression

Tension

Tension

(a) Bending force

(b) The bookcase shelf is bended by the weight of the books

(d) Twist

Figure 3 (a) Twisting forces

Twist is the effect produced by two equal forces in strength but opposite in direction, the function lines of two forces do not overlap. For example, the steering shaft turning the car steering wheel; manually unscrewing the cap of the bottle and so on.

Force

(b) The car steeling wheel being turned by force

(e) Shear

Shear is the result produced by a pair of forces equal in strength but opposite in directions being applied to the perpendicular axis of the member. For example, cutting papers by the scissors, riveting the steel plate, etc.

Force

Force

(a) Shearing force

(b) Cutting papers with the scissors

Force

3. Equilibrium of structure

| A structure in equilibrium is referred to as a stationary structure. This means that the forces applied to the structure eliminate each other without generating resultant force. In the diagram, the combined force exerted downward by the chair's own weight together with the weight of the person sitting on it must be equal to the balanced forces generated by the upward forces exerted by the floor, which is the reaction force. Chair at rest |  |  |
| --- | --- | --- |
| The equilibrium of a structure is stable if it returns to its original position after being interfered. Taking the chair as an example, when the weight acts within the area whose corners are the points where the legs touch the floor, this is the stable equilibrium state. Stable equilibrium |  |  |
| However, consider the person sitting on the chair leans backward until the weight is acting in the vertical plane beyond the points where the back legs touch on the floor. Although the reactions at these points can still balance the weight, the slightest interference can cause the chair to topple. In such case, the equilibrium is said to be unstable. Unstable equilibrium |  |  |
| Besides ensuring that the structure is in stable equilibrium, it also needs to have sufficiently strong elements to withstand the forces applied on it. Able to withstand tension Able to withstand pressure | Stable contact point |  |

4. Material and the shape of its cross section

(a) Properties of material

Appropriate materials must be chosen in the design and production of structures.  Wrong choice of materials can result in structural damages by external forces easily or can cause permanent deformation.  For example, cables can withstand greater tension but not any pressure. On the other hand, concrete can withstand extremely large compression force but it can easily be pulled off by tension.

Hangers of the Tsing Ma Bridge			Concrete blocks

Concrete blocks

As the different parts of a structure may be subject to different loads, they may need to be constructed by different materials.

- Tubes

Comparing with solid cylinders manufactured with the same material, hollow tubes have a larger diameter, and thus requiring a greater radial load to bend it. Long cylindrical structures such as street lamppost, school flagpoles are mostly manufactured with tubes.

(c) Different types of structure

(i) Arch

Arch is a structure purely subject to compressive loads; the downward force of the load is transferred through the members to the left and right sides. Arch does not need any member for joining, as long as the members are placed in the proper positions, a robust structure will be formed. Commonly used materials include stone, brick, concrete, etc. Arch is commonly seen in the foundation part of ancient Chinese stone bridges and also in ancient buildings in Europe.

Load

Force being

moved

Horizontal supporting

force

Horizontal supporting

force

(a) Arch structure

- Practical example of arch structure

(ii) Framed Structure

Framed structure is formed by linking individual rod or bar shape objects. The advantage of framed structure is simple and lightweight, which is generally used to carry load. Its main components are rods and their joints. There are many examples of framed structure as lots of buildings and machinery are assembled by it. For example: cable towers, buildings,1bridges1and1cranes,1etc.

Load

Load

Load

Supporting force

Supporting force

(a) Framed structure

(b) Practical examples of framed structure

(d) Beams of different cross-sections

Commonly seen footbridges and cargo frames all belong to beams.  Beams can withstand the pressure from top to bottom and the cross-sections shown below are the most common:

(i) I-beam

I-beam piles or crash barriers can evenly withstand the pressure.

- Piles used in construction

(b) Crash barriers at the roadside

(ii) Box

Box construction material can withstand greater twisting forces

Top down pressure

Horizontal pressure

The containers on a container truck is an example of square box structure

(iii) L-section beam

L-section beam is mainly used as the frame for holding air-conditioners or shelves on the wall for goods.

- L-section beam

- Frame for air conditioners

(e) Corrugated surfaces

The load that materials with flat surface can withstand is not as strong as the load intensity of the same materials with corrugated surface. Common examples are:

(a) Groove-shaped case of the container

Therefore, using the designs of different-cross sections can make the overall structure more compact and lighter, which can better withstand the load and not easily damaged or deformed.

b) Wave-shaped windscreen plastic sheet

(c) Corrugated cartons

II. Mechanism

1. Understand mechanisms with black box approach

To enhance the effectiveness of machinery, compound machinery is composed by a number of mechanical components, and the designers will regard them as a black box.

(a) Robots    	     (b) Walking robots

Below are commonly seen black boxes for mechanical movements

| - Rotary motion Rotary motion (opposite direction) | - Rotary motion Rotary motion (same direction) | (c) Rotary motion Reciprocating motion |
| --- | --- | --- |
| (d) Rotary motion Rotary motion (90 o) | (e) Reciprocating motion |  |

2. Properties of mechanical movements

The power of human body is limited, we need machinery to increase the strength, increase speed or apply forces conveniently.

(a) Increase the strength

For example a car jack can be used to increase the user's strength; we can use less force to lift a very heavy car.

Mechanics to increase the strength (car jack)

(b) Increase the speed

For example a bicycle can increase our speed and make us move faster than running.

Machine to increase the speed (bicycle)

(c) Apply forces conveniently

Although some mechanisms cannot save effort or increase speed, it can change the direction of force to facilitate our application of force, such as fixed pulley is a common example.

Fixed pulley

Force

Fixed pulley

Load

3. Application of Mechanism

A

ll machines are formed by a combination of different parts and these parts are called the mechanical components. Machine comprising of two or more simple machines is called compound machinery; they can drive and control movements.

- Simple mechanism

Output

The earliest and simplest mechanical components used by human probably be the lever. It is a hard and straight bar which can rotate at a fixed point; this fixed point is called the fulcrum / pivot. The location where the force is applied is called the effort, and the position of the loading is called the load. The function of the lever is to change the magnitude, speed and direction of power by changing the position and distance between effort, load and fulcrum.

It contains the three major parts of a typical system:

- Compound mechanism

In compound machinery, lever, cam, belt, gear, linkage, crank, etc. are mixed and used to change the various input movements to output movements of all kinds.

Output

Input - Movement of human force or machine 
Process - Cam, belt, gear, linkage, crank, etc. change the magnitude and direction of forces 
Output - Compound machine being driven

| Input |  | Process |  | Output |
| --- | --- | --- | --- | --- |

Belt

Electric motor

Input

Input - Push down movement by human force 
Process - Lever changes the magnitude and direction of the force
Output - The rock is being raised.

Input

(a) The belt and gear box of the machine tool

Input

Effort

Rock

Load

Fulcrum / Pivot

Gear box

Cam

Follower

Input

Output

- (b) Cam shaft of the car engine

Input

Crank

Chain

Gear

Input

- (c) The pedals, crank, chain and sprocket of a bicycle

III. Glossary of terms

| 結構 |  | Structure | 盒式 |  | Box |
| --- | --- | --- | --- | --- | --- |
| 壓縮 |  | Compression | 摺曲 |  | Corrugated |
| 拉張 |  | Tension | 黑盒 |  | Black box |
| 屈曲 |  | Bending | 機械結構 |  | Mechanism |
| 扭轉 |  | Twist | 齒輪 |  | Gear |
| 剪切 |  | Shear | 帶 |  | Belt |
| 平衡 |  | Balance | 鏈 |  | Chain |
| 拱架 |  | Arch | 凸輪 |  | Cam |
| 支架 |  | Frame | 連桿 |  | Linkage |
| 切面 |  | Cross section | 曲柄 |  | Crank |
| 機器 |  | Machine | 機械 |  | Machinery |

IV. Relevant Information

|  | Websites | Brief contents |
| --- | --- | --- |
| 1 | http://lizarum.com/assignments/physical_computing/2008/mechanisms/cams.html | Simple mechanism - cam |
| 2 | http://www.petervaldivia.com/technology/mechanisms/gears.php | Simple mechanism - gear |
| 3 | http://www.lizarum.com/assignments/physical_computing/2010/FPMechanisms/index.html | What’s inside the machine black box? |
| 4 | http://wsdt.office-on-the.net/resources/DTonCD1/school/cranks.html | Simple mechanism - crank |
| 5 | http://www3.ul.ie/~kirwanp/examplesofsystemsinuse.htm | Simple mechanism - cam |

Lesson Activities

(1) Divide students of the class into groups of eight, each person takes an A4 paper

(2) Two of them follow method (a), two follow method (b), two follow method (c), and the remaining two method (d) to fold the A4 paper

(a)	(b)

- Testing method for

a beam

(3) Use items such as the erasers and pens of students in the group as support and conduct loading test to compare the various corrugated methods and the loading strength that can withstand when used as beams and poles:

Apply force             Apply force

- Testing method

of a pole

- (d)

(4) Result records ( is the highest force，the lowest)

|  | (a) | (b) | (c) | (d) |
| --- | --- | --- | --- | --- |
| Withstanding loading strength when used as beams |  |  |  |  |
| Withstanding loading strength when used as poles |  |  |  |  |

Lesson Exercises

- Please give an example for each of the five properties of force structure (compression, tension, bending, twist and shear).

- List commonly used materials for the five types of load, and give some relevant examples.

| Nature of the force | Examples | Commonly used materials |
| --- | --- | --- |
| Compression |  |  |
| Tension |  |  |
| Bending |  |  |
| Twist |  |  |
| Shear |  |  |

Table 1

A

B

Siu Ming was led by teachers to visit a bridge being built. Siu Ming found that there were a lot of steel rods within the concrete but he did not understand the purposes of these rods. Please explain the reasons for using the steel rods?

- The right diagram is a common folding ladder used by decoration workers. Please point out that when the worker steps on the ladder:

(i)  What type of force is being exerted on the leg labeled as A?

(ii) 	What type of force is being exerted on the rope labeled as B?

(iii) 	What kind of accident may be caused if there is no rope used as labeled as B?

- (i) Based on the consideration of the structure’s strength, why tube is used to manufacture the school flagpole?

Force

Load

(ii) List two examples that increase the structure’s strength by changing the cross-section.

- Explain the properties of arch structure. (With annotated sketches)

- For the two machineries below, please point out their input-process-output

(a) Nail pulling hammer	          (b) Tool box

- The diagram below is a movable toy, briefly explain how it can be moved.

Cam

Driven member

Hand wheel

- 9. In a tourist district kiosk, the shop owner installs the following device in the show window in order to expel insects causing nuisance to the food placed there.

- Briefly describe the operation of this device.

- Suggest some methods to change the swing magnitude of the swinging sweeper.

Crank

Horizontal motion linkage

Swinging sweeper

Joining point

Motor driving wheel

Direction of

rotation

Project Activity - Structure

The watchtower

Guidelines for project activity

1. Objectives

Students get a basic understanding about design process and safety usage of hand tools. Use appropriate communication skills to express the design ideas and use appropriate hand tools to make the artefact.

- At completion of the project activity students should be able to master

- use of communication skills to express the design ideas;

- use of hand tools to make the artefact.

3. Situation

It is happy and refreshing for going up the hill or high areas and look over a long distance. In the old days towers were usually built for defensive purposes against thieves and enemies.

4. Requirements of the project activity

(a) Task brief

Students have to make a watchtower which can hold a soda can (~335ml)

1

(b) Grouping

Four persons in each group		    Name of the group: ____________________

Names of team members:

（1）：_______________________________ （   ）	（2）：______________________________（   ）

（3）：________________________________（   ）	（4）：______________________________（   ）

5. Recommended time

(3 lessons + 1 lesson for competition) x 40 minutes (total 160 minutes)

6. Required materials

(i) Two pieces of newspaper (Two other pieces of newspaper for testing)

(ii) One roll of 12mm width adhesive tape

7. Activity contents

(i) Group members discuss, test and modify:

(i) Different methods to increase the strength of the paper

(ii) Methods to build a higher watchtower with less material

(iii) Methods to carry the required weight with less material

2

(ii) Consideration factors

Only

2 pieces of newspaper and 1 roll of adhesive tape

Strength of the newspaper

Safety

The tower’s centre of gravity

Limited time

Weight of the soda can

The

watchtower

The following questions should be considered in designing and making the watchtower:

- Only two pieces of newspaper and one roll of   adhesive tape can be used

- How to increase the strength of the newspaper

- How to fully utilize the adhesive tape

- The weight of the soda can

- Soda can’s effect on the tower’s centre of gravity

- Time is limited

(iii) Making procedures

- Test the effects of reinforced newspaper

- Design the appearance of the watchtower

- Design how to place the soda can

- Making

- Testing

- Modify the design, re-work, test again

- Competition

(iv) Competition:

Students put the soda can on a high platform and then leave the can and the tower, timing for 10 seconds without collapsing.

The teacher will measure from the table top to the lowest point of the soda can. The group that can carry the can to the highest level will win. In case of same results then the one using lighter materials will win.

(v) Recording and reflections:

Our design of the watchtower:

Sketch or affix photos

Our results:

The total weight of our watchtower: ___________ g

Our watchtower raises the soda can by: _____mm

Efficiency of our watchtower: _____________ mm/g （Height raised ÷ Weight of the watchtower）

The best result in the class is to raise the soda can by: _________mm

3

Our modified version of the watchtower:

If we can make the watchtower again, it will be …… Sketch or affix photos

Use not less than 30 words to briefly introduce your aspects of design which require modifications:

________________________________________________

________________________________________________

________________________________________________

4