Relevant Knowledge

I	Using Integrated Circuit (IC) for Control Automation

1. Integrated Circuit (IC)

At the beginning of Teaching Material 7, we have introduced the development of electronics technology.  At the end of it, we mentioned integrated circuits, which developed from comprising only a few chips to very-large-scale integrations (VLSI).  Since most products nowadays use ICs to fabricate microcomputers, we have already entered the age of microcomputer.

As new ICs are being developed every day, it is likely that no one can actually point out the exact number of types of IC existing to date.  Compared to the home computer bought last year, one that possesses a much faster processing speed will be available in the market today.  When brand A introduced a smartphone last month, brand B may launch another one this month which is claimed to be faster and more powerful.  These suggest that new ICs have been given birth again.

Categorisation of Integrated Circuits

The number of chips in an IC can range from a few dozens to more than a million.  When assembling an IC, the chips can be

- connected together as a circuit to carry out a function;

- not connected together as a circuit to carry out different useful functions.  In such a case, the functions of an IC are programmed after it is being assembled.

ICs can be assembled using different methods, each with a different function.  We categorise ICs roughly in the following ways.

(a) Categorising ICs by their functions

Commercially available ICs can be assembled to perform the following functions.

Categorisation of ICs by their functions

| Type | Examples | Functions |
| --- | --- | --- |
| Logic gate | 7408, 7432, 7400 | AND, OR, NAND |
| Operational Amplifier | 741 | General-purpose operational amplifier |
| Timer | 555 | Keeping track of time under different modes |
| Analogue switch | 4016 | Behaving like a relay to allow electric current to pass through |
| Memory | Random-access memory (RAM), read-only memory (ROM) | Storing data |
| Microcontroller | 8051, PLC | Acting as a miniaturised computer |

(b) Categorising ICs by their complexity

As technology advances, engineers are able to assemble more electronic components on a single chip.  The complexity and level of integration are measured by the number of electronic components placed on a chip, which can be used as an indicator to categorise ICs.  The following table summarises the different levels of integration.

Categorisation of ICs by their complexity

| Type | Denote | Electronic components per chip | Examples |
| --- | --- | --- | --- |
| SSI | Small-scale integration | 100 | Logic gates |
| MSI | Medium-scale integration | 100 – 3,000 | Counters |
| LSI | Large-scale integration | 3,000 – 100,000 | Analogue-to-digital converters, digital-to-analogue converters |
| VLSI | Very-large-scale integration | 100,000 – 1,000,000 | Microcontrollers |
| ULSI | Ultra-large-scale integration | > 1 million | Microprocessors, dual-core microprocessors |

The term ULSI was proposed to be used to describe chips with a high level of complexity that they consist of more than a million electronic components but the term VLSI is often used instead.

(c) Categorising ICs by their packaging methods

Upon the completion of the assembly and testing of an IC, a wafer, on which the circuit is built, will be added to the package of the IC before sale.  The package prevents the wafer from damaging by external forces or moisture, and also prevents the silicon chip from erosion by any kinds of chemical.  There are two common types of IC packaging methods.

Categorisation of ICs by their packaging methods

| Packaging method | Examples | How it is mounted to the circuit board |
| --- | --- | --- |
| Through-hole package |  | Pins of an IC are inserted into the holes drilled on a printed circuit board and soldered |
| Surface-mount package |  | Directly attached to the surface of a printed circuit board, thus saving space and processes for drilling holes on the circuit board |

(d) Categorising ICs by the types of signals they process

Based on the types of signals they process, ICs can also be categorised into analogue (linear), digital and mixed-signal (both analogue and digital on the same chip).

Categorisation of ICs by the types of signals they process

| Type | Characteristics | Examples |
| --- | --- | --- |
| Digital IC | Typically works on two states only – LOW and HIGH. | Logic gates, flip-flops, memories and microcontrollers |
| Analogue IC | Can output variable and continuous signals which are proportional to the input signals | Operational amplifiers and sensors |

2 Examples of Commonly Used ICs

- 555 Timer IC

A number of examples of IC part number have been mentioned above, such as IC 7408, 8051, 741 and 555.  In which, IC 555 is a cheap and widely used timer IC.  Relevant data sheets and circuit diagram examples of its application can easily be found on the Internet.

Different time delays can be obtained when combining different capacitance and resistance

IC 555 in monostable mode

From the figure on the right above, we can observe that a basic timer circuit like the one shown on the right can be obtained with the addition of only a few electronic components.  Pressing the switch once, the light emitting diode (LED) will light up for a specific period of time and then go off automatically.  With the value settings of the electronic components as shown in the figure on the right, the time for the LED to light up is about five seconds.

Application of 555 Timer IC

The figure on the right shows a toy keyboard.  The circuit of the IC 555 in it works in astable mode instead.  Its output is changed from LED to _________________. To produce different sounds, it adds a series of ___________________________ with preset values.

Point out other electronic products that you think timer ICs exist in their circuits.  What functions can the ICs perform?

b.  IC 741 Operational Amplifier

IC 741, a type of operational amplifier (op-amp), is employed in a wide range of circuits for amplifying weak electric currents in them.  An op-amp is often included as one of the components in the circuits of radios, stereo systems, headphones, televisions and many other electrical products.

Op-amps are the most useful standalone devices in analogue electronic circuits.  Coupling with different combinations of external components, they can carry out a variety of analogue signal processing tasks.

8 	not connected

7 	+V

6	output

5 	offset null

offset null	1

inverting input	2

non-inverting input	3

–V	4

+

–

An ideal op-amp has characteristics such as (i) infinite input impedance and magnification, (ii) unrestrained bandwidth and (iii) zero output impedance.

The following figures show the basic operation of an op-amp.  Observe the variation in the output of the op-amp when VR1 is adjusted to different positions.

|  | VR1 is adjusted to a balanced position; the output voltage is zero. |
| --- | --- |
|  | VR1 is adjusted to a higher position; the output voltage becomes negative (－). |
|  | VR1 is adjusted to a lower position; the output voltage becomes positive (+). |

Applications of IC 741

The figure below shows a part of a circuit of an alarm system.  When the light sensor of the circuit detects any changes in light intensity, it will emit a signal to trigger the relay and set off the alarm system and the siren.  Without an op-amp, the signal would be too weak.  The op-amp amplifies the signal so as to make it strong enough to set off the alarm.  To work properly, sensors often have to work together with op-amps.

Alarm

Light dependent resistor

Op-amp

Relay

Diode

Transistor

Resistors

Infrared

Internal battery

Sensitivity adjustment

What are the uses of different ICs?

Find a daily electronic product and dismantle it (e.g. the soya milk maker shown below or the Wiimote of a Wii console).

Among the commonly used types of ICs found in electronic products that you encounter regularly, choose and name three of them.

What categories do the ICs you have chosen belong to?  Each of the ICs can be classified into more than one category.
[Note: You can search for the data sheets of the ICs you have chosen on the Internet and use them to describe the functions and characteristics of the ICs.]

Guess what IC category appears most on classmates’ IC lists and guess its function.

II	Using Electronics, Microprocessors and Computers for Control Automation

1. Electronic-control Circuit

In Material 7, we have studied an electronic-control circuit whose operation relies on light intensity.  In automation, controlling by light is frequently employed.  For example, the figure on the right shows a door of an early lift.  There is a light source on one side of the door.  The door will keep closing as long as the other side senses the light properly.  Operation procedure begins after the door is closed completely.

*What are the drawbacks of controlling by light only?  How can it be improved?

Washing machines like the one shown on the right is another example.  Temperature of the water for washing the clothes can be adjusted.  Apart from cold water, users can choose from 40℃, 60℃ or even 90℃ for washing clothes that are dirtier.  The temperature sensing device inside the machine will detect the temperature of the water being added.  The motor will be connected and the washing process will begin only when the water is heated up to the required temperature.

2. Microprocessor-control Circuit

The above washing machine, or some even more modern models, contains not only a temperature sensing device, but also other sensing devices for

- checking whether the door is closed;

- checking whether the water pressure is sufficient;

- detecting the weight of the clothes so as to estimate the amount of water needed;

- checking whether the amount of water added is enough;

- checking the turbidity of the water so as to determine the washing process;

- controlling the rotating speed of the motor and all kinds of washing processes;

- drying the clothes and controlling the related processes; and

- releasing the lock after the completion of the washing processes and reminding users to take the clothes out.

Each of the above processes requires the use of sensing devices.  Constructing a circuit to control the devices using electronic components such as transistors would be very complex and the resultant apparatus would be very bulky.  In this case, a microprocessor which has preset reactions and controls over the inputs from the various sensing devices will be more appropriate.

3. Computer-control Circuit

Some local schools might have bought the rapid prototyping machine shown in the figure on the right, which is equipped with a temperature sensing device.  When its internal temperature reaches the required temperature (about 300 ℃), the platform will be raised, the raw materials will be extruded through the nozzle and the production process will begin.  Nevertheless, the machine contains not only a temperature sensing device for controlling automation, but also other sensing devices for:

- detecting whether raw materials have been filled properly;

- checking whether the passage between the raw material storage cartridge and the extrusion nozzle is obstructed;

- detecting the internal temperature so as to start operation when it reaches the required temperature (about 300℃);

- checking if the platform has reached the necessary height before the production process begins;

- checking if the nozzle has been moved to the desired position before the production process begins;

- so on and so forth.

Besides requiring sensing devices and giving responses, the machine also needs to connect to a computer in order to perform automation operation.  In the past, computer-control machines often need special interface cards and cables but these newer and simpler machines can perform the control through connecting to the same network as the computer does.

The figure on the right shows the input of a design drawing of a product (a three-dimensional school badge), with appropriate units and scale, density of the finished product, number of copies required, etc.

The following figure shows the instantaneous, real-time responses, including the status of the rapid prototyping machine, the amount of materials left, and the amount of materials and time required for this production session, figured out by the software upon movement of the position of the product.  Commands such as the ones for starting and stopping the production process are also initiated by the computer.

From the above examples, we know that simple controls can be carried out by electronic components while more complex ones can be achieved by microprocessors.  When dealing with even more complex and interactive controls, computers have to be used.

Discussion

What examples of automated operation can you find at home and in schools?

III Recognising the Common Symbols for Representing Electronics Components and Drawing Simple Electronic Circuit Diagrams

In Teaching Material 7, we have introduced basic electronic and electrical components and their symbols.  Thus, relevant information will not be repeated here.  In the following, let’s try to draw simple electronic circuit diagrams from pictorial diagrams.

LDR

Variable resistor

Diode

Resistor

Transistor

Relay

Light bulb

Step 1:

Draw the symbol representing each electronic component in the following table.

| Battery |  |  |
| --- | --- | --- |
| Variable resistor |  |  |
| Light dependent resistor (LDR) |  |  |
| Resistor |  |  |
| Diode |  |  |
| Transistor |  |  |
| Relay |  |  |
| Light bulb |  |  |
| 220V power source |  |  |

Step 2:

According to the position of the real objects, draw the symbols of these electronic components.  Note the following key points when drawing:

- place the input components (e.g. power sources and switches) on one side, processing components in the middle and output components (e.g. LEDs, light bulbs and buzzers) on the other side;

- draw the positive terminal ‘+’ at the top and negative terminal ‘－’ at the bottom;

- connect the positive terminals ‘+’ and negative terminals ‘－’ of the electronic components to the upper side and the lower side respectively;

- draw each connecting wire using horizontal or vertical line;

- try to avoid the wires from intersecting;

- draw the electronic components on the same straight line;

- annotate each electronic component when necessary; and

- connect the circuit to the ground.

Exercise 1

The following figure shows a pictorial diagram of a flashing light circuit.  Try to draw its corresponding electronic circuit diagram.

Step 1:

Draw the symbol representing each electronic component in the following table.

| Object | Name | Symbol |
| --- | --- | --- |

Step 2:

According to the placement of the real objects, use the symbols of these electronic components to draw the electronic circuit diagram.

Suggested answer for the simple circuit diagram:

Exercise 2

The following figure shows an example of the design of a circuit diagram for a music box.  Draw in the spaces the appropriate circuit symbols.

(Refer to p. 10–13 of Teaching Material 7)

PNP-type triode

NPN-type triode

LDR

Resistor

Capacitor

Resistor

Earthing

Earthing

Resistor

Switch

Capacitor

Resistor

Resistor

Resistor

Resistor

Speaker

Earthing

Resistor

Resistor

Capacitor

Earthing

Capacitor

Earthing

Battery

IV	Using Construction Kits to Model Pneumatic-control Systems to Achieve Specified Functions

In S1 Teaching Material 6, System and Control, we have introduced the use of compressed air for transmitting and controlling energy in a system.

Input	－	Compressed air, switch, etc. 
Process	－	Transmit air to various pneumatic processing components via sealed pipes. Output	－	Work done by pneumatic components to complete designated tasks.

In the teaching material, there is also an activity to make a pneumatic toy.  In the following, we will introduce how to use construction kits to model pneumatic-control systems to achieve specified functions.

In Hong Kong, pneumatic components of different brands and designs are available for purchase.  Two major suppliers of such products are SMC and Festo.  In addition to sales of hardware, they also provide training courses and online learning materials.

Online learning materials in simplified Chinese provided by SMC: http://www.smchk.com.hk/smc_website/elearn/elearnmain.htm

English online learning materials provided by Festo: http://www.festo-didactic.com/int-en/ 
Students can also download the demo version of Fluidsim, a piece of software for the simulation of pneumatic systems: http://www.fluidsim.com/fluidsim/download/v4/demo001p4.exe.

For the above figure and the rest of the content, we have used the software from Festo for demonstration.  It must be noted that, however, there are choices of software and hardware from companies other than the two mentioned above.

Before the actual use of constructions kits or simulation software, let’s briefly introduce pneumatic systems, as well as some commonly used pneumatic components and their functions.

- 1 Various Parts of a Pneumatic System

The following shows the major parts of a pneumatic system.  The various parts are connected to each other with pipes, via which energy is transferred from the air supply unit to the drive component by the use of compressed air.

| Input |  |  |  | Process | Output |
| --- | --- | --- | --- | --- | --- |
| Air supply | Storage | Purification | Processing | Control | Driving |
| Compressor | Reservoir | Filter, Dryer | Regulator, Filter, Lubricator | Directional control valve, Speed regulating valve | Cylinder |

Compressor

A compressor can compress air to the desired operating pressure*.  It converts mechanical energy of a generator or an internal combustion engine to pressure energy of compressed air.  The compressed air required by every pneumatic drive or pneumatic control device is provided by a set of central air compression equipment.   Each pneumatic component can acquire the compressed air it needs from the compressed air reservoir via pipes.  Air compressors can be categorised into two main types, reciprocating and rotary.

(a) A small compressor used in schools	(b) A common industrial compressor

* Note: The pressure of compressed air used for pneumatic control is about 6 times the atmospheric pressure (6 bar or 0.6 MPa).

Pressure Regulating Unit

A pressure regulating unit for compressed air consists of the following components:

- Filter – removing impurities existing in the compressed air before being passed into pneumatic components.

- Pressure regulator – stabilising the pressure and regulating the operation of pneumatic components.

- Lubricator – providing suitable lubrication to pneumatic parts.

Pressure regulating unit

Lubricator

Pressure regulator

Filter

Filter

| (a) Picture of a pressure regulating unit | (b) Symbols of the pneumatic components of a pressure regulating unit |
| --- | --- |

Connecting Pipe

The requirements of a connecting pipe are that it is able to withstand the required pressure and it can be connected appropriately.

Picture of connecting pipes

Safety Precautions When Using Compressed Air

1.	Compressed air can enter human ducts such as oral cavities and ears, and causes serious injury.  Never spray compressed air onto anyone.  At high temperatures, compressed air can even penetrate human skin.

2.	Compressed air released from the exhaust contains particles and oil droplets, which can cause damage to eyes.

3.	Even though the pressure of the compressed air is relatively lower in pipes and reservoirs, violent explosions may still occur when a container has been damaged.

4.	Before switching on the compressed air supply unit, one must thoroughly inspect the entire system arrangement.

- 2. Pneumatic Components

A wide variety of gas valves are used in pneumatic systems to control the operations of drive components (cylinders).  Examples include shuttle valves, flow control valves, quick exhaust valves, logic valves, one-way valves, speed control valves and directional control valves.  Among the different kinds of gas valves, directional control valves are the most commonly used.

- Directional Control Valve

The major function of a 2 ports 2 positions (abbreviated as 2/2) directional control valve is to force the valve open or close through the thrust of a spring, thus stopping compressed air from flowing towards working vent A from air inlet P.  When a force is applied to the control axis, the valve is pushed open, connecting P with A.

The force applied to the control axis must overcome both the repulsive force of the spring and the air pressure.  The control valve can be driven manually or mechanically, and is returned to its original position by the spring.  This kind of valves is mainly used to control gas motors and pneumatic tools.

Working vent A

Air inlet P

| - Picture of a 2/2 directional control valve | (b) Cross-sectional diagram | (c) Pneumatic symbol |
| --- | --- | --- |

A 3 ports 2 positions (abbreviated as 3/2) directional control valve can be used to control a single acting cylinder.  The valves opening in the middle is closed until P and A are connected and push downwards.  Then, another valve will open the sealed base between A and R (the exhaust).  The valves can be driven manually, mechanically, electrically or pneumatically.  This type of valves is mainly used to control single acting cylinders (push type) and pneumatic signals.

Working vent A

Air inlet P

(a) Pictures of 3/2 directional control valves			(b) Cross-sectional diagram

3/2 directional control valves can further be categorised into two types: normally open (N.O.) and normally closed (N.C.).  When not under operation, valves in a normally closed valve remain closed such that air cannot pass through them; under the same condition, valves in a normally open valve remain open such that air can pass through them freely.  The following figures show the pneumatic symbols of the two kinds of control valves.

| Pneumatic symbols of 3/2 directional control valves | (a) normally closed type | (b) normally open type |
| --- | --- | --- |

5 ports 2 positions (abbreviated as 5/2) directional control valves

| (i) Pictures of 5/2 directional control valves | (ii) Pneumatic symbol |
| --- | --- |

Depending on the functions, other pneumatic symbols may be added on the symbols of directional control valves.  The following are some examples.

| Air inlet | Air outlet | Directions of gas flow | Indicator light | Manual operation |
| --- | --- | --- | --- | --- |
| or |  |  |  |  |
| Flow restrictor | Spring return |

Some directional control valves can also be operated by using compressed air, mechanical or electronic systems.

Other Control Valves

Control valves are valves that control the flow of air.  Examples of control valves include one-way valves, flow control valves and shuttle valves.

- One-way Valve

A one-way valve allows air to flow in only one direction.  When air flows in the opposite direction, the valve closes.  One-way valves are also known as non-return valves.

(a) Picture of a one-way valve	(b) Pneumatic symbol

- Flow Control Valve

Flow control valves are also known as speed control valves.  Their structure is composed of a one-way valve and a variable throttle.

| (a) Pictures of flow control valves | (b) Pneumatic symbol |
| --- | --- |

- Shuttle Valve

Shuttle valves are also known as double control or single control non-return valves.  A shuttle valve has two air inlets P1 and P2, and one air outlet A.  When compressed air enters through P1, the sphere will seal and block the other inlet P2.  Air can then flow from P1 to A.  When the contrary happens, the sphere will block inlet P1, allowing compressed air to flow from P2 to A only.

A

P1

P2

| (a) Pictures of shuttle valves | (b) Pneumatic symbol |
| --- | --- |

Cylinder

Cylinders are the driving components of pneumatic systems, which are used to drive other machinery.  The components inside a cylinder move with a high speed, thus causing great impact force at the end of their movement paths.  To prevent the inside of a cylinder from malfunctioning, buffering devices are usually installed in it, such as rubber cushions, to absorb the impact force.

- Single Acting Cylinder (Spring Return)

A single acting cylinder has only one inlet for compressed air and thus can only produce thrust in one direction.  The piston rod is propelled in the opposite direction (retracting) by an internal spring or by external force supplied by the weight of a load, mechanical movement, etc.

| (a) Picture of a single acting cylinder | (b) Pneumatic symbol |
| --- | --- |

As the piston rod of a single acting cylinder needs to overcome the force from the spring, its thrust is reduced.  Therefore, in order to provide sufficient driving force for machinery employing single acting cylinders, the cylinders need to have larger diameters.  Besides, due to the lengths of the springs, the lengths of the cylinders have to be increased, thus limiting the lengths of the paths of the piston rods.  Single acting cylinders are used in processes such as stamping, printing and material feeding.

- Double Acting Cylinder

In a double acting cylinder, air pressure acts alternately on both sides of the piston, producing propelling and retracting force.  Since the effective area of the piston is small, the thrust produced during retraction is relatively weaker.  The seamless barrels of double acting cylinders are usually made of steel, whose working surfaces are much polished and are plated with hard chromium to reduce friction.

| (a) Picture of a double acting cylinder | (b) Pneumatic symbol |
| --- | --- |

- 3.	Pneumatic Circuit

Pneumatic control systems can be designed in the form of a variety of pneumatic circuits to perform specific tasks.  Pneumatic circuits are made up of a combination of pneumatic components, including cylinders, directional control valves and flow control valves.  They have the following functions:

- Control the injection / release of compressed air into / from the cylinders.

- Use one valve to control another valve.

- Basic Rules

- A pneumatic circuit diagram describes a static system and assumes there is no supply of pressure.  We should determine the positions of pneumatic components on a circuit based on this idea.

- The basic pneumatic symbol for a directional control valve is formed by one or more squares.  The inlet and exhaust of the valve are drawn underneath the squares, while the outlet is drawn at the top.  Each function of the valve (the position of the valve) is represented by a square.  If a valve has two or more functions, the squares should be arranged horizontally.

Outlet

Inlet and exhaust

Air Input

Exhaust

Pneumatic symbols of 3/2 directional control valves (normally closed type)

- Arrows (↓↖) are used to indicate the directions of air flow inside a directional control valve.  When an external port and the internal parts of a valve are not interconnected, the symbol ┬ is used.  The symbols ⊙ and ▽ underneath the squares represent an air input and an exhaust respectively.  The figure on the right-hand side above shows an example of a typical pneumatic valve.

- Drawn on the outside of the squares are pneumatic symbols of operation components.  They can be categorised into two main types: mechanically operated and manually operated.

| Mechanically operated | - Straight plunger lever | - Roller lever | - One-way roller lever |
| --- | --- | --- | --- |

| Manually operated | - Standard | - Lever | - Push button | - Pull/push button |
| --- | --- | --- | --- | --- |

- A pneumatic operation signal pressure line should be drawn on one side of the squares, on which a triangle is drawn to represent the direction of air flow.

Pneumatic operation signal pressure line

- Basic Principles

The figure on the right-hand side below shows the basic principles for drawing pneumatic circuit diagrams.  The numbers in the figure correspond to the following points:

1.	When the manual switch is not pushed, the spring will return the valve to its original position.

2.	From the position of the spring, one can deduce that this block is operating.  The other block will not operate until the switch is pushed.

3.	Due to the pressure from the spring, the piston rod is returned to its retracted position.

4.	The rear cylinder cavity and this line represent that exhaust is being discharged.  (When under manual operation, this will change to air being input.)

Connecting to an air supply unit

1

2

3

4

Manual operation

| (a) Pictorial diagram of a pneumatic circuit diagram | (b) Basic principles for drawing a pneumatic circuit diagram |
| --- | --- |

| When the manual switch is not pushed. | When the manual switch is pushed, the piston rod extends. |
| --- | --- |

- Setting of Pneumatic Circuit Diagrams

When drawing a complete pneumatic circuit diagram, pneumatic components of different functions should be placed at different levels and positions, so as to express the relations between components clearly.  This is called the setting of pneumatic circuit diagrams.  A pneumatic circuit diagram is usually divided into three levels: power level, logic level and signal input level.

Power level

Logic level ( Functions: memory, AND gate, OR gate, delay, etc.)

Signal Input level (Air supply unit)

Power level, logic level and signal input level

- The steps for planning a pneumatic circuit diagram setting are as follows:

| 1. |  | 2. |
| --- | --- | --- |
| In a pneumatic circuit diagram, the flow of energy is drawn from the bottom to the top. Therefore, air supply units should be placed in the bottom left corner. |  | The work cycle is drawn from left to right, so the first operating cylinder should be placed in the upper left corner. |
| 3. Power control valves should be drawn directly below the cylinders controlled by them, forming a power unit. |  | 4. Control cylinders or operation valves (signal components) driven by power control valves should be placed at lower levels of a pneumatic circuit diagram. |
| 5. |  | 6. |
| Auxiliary valves, or those with logic functions (e.g. memory, AND, OR, NOT and delay), should be placed between operation (signal) components and power control valves. |  | Use solid lines, which represent pipes, to connect all air supply units and pneumatic components to complete the pneumatic circuit diagram. Check carefully the circuit and its logic of operation before use to avoid any accident. |

- 4	Basic Types of Circuits

From this page on, the circuit of each pneumatic control system can be simulated by construction kits.  The use of simulation software for test running is also encouraged.

A basic circuit is a pneumatic circuit designed to perform a specific basic function, such as flow amplification, signal inversion, memory, delay, single acting cylinder control and double acting cylinder control.

- Flow Amplification

A cylinder with a large capacity requires a larger flow of air, which might accidentally injure the operators.  Thus, it is not recommended to manually operate a pneumatic directional control valve with a large flow capacity.  Instead, we should use a small manually operated control valve to manoeuvre a pneumatic control system with a large flow capacity.  This operation is called flow amplification, which can ensure the safety of operators.  During operation, the valve with a large flow capacity should be placed closed to the cylinder, while the valve with a smaller flow capacity should be set on top of an appropriate control panel in the distance.  The following figure shows a basic flow amplification circuit.  Notice that in the circuit diagram setting, different components have already been placed at different levels.

Output to cylinder with a large flow capacity

Signal pressure line

Directional control valve with a large flow capacity

Directional control valve with a small flow capacity

Compressed air supply unit with a large flow capacity

Manual operation

Compressed air supply unit with a small flow capacity

Flow amplification system

- Signal Inversion

The pneumatic diagram below shows how the output of a directional control valve can be switched.  When control valve  is operating, control valve  will stop producing pressure output.  When control valve  ceases operation and is returned to its original position, control valve  will resume its output.  Therefore, at any given time, the pressure output of control valve  and control valve  is always the exact opposite of each other.

Output

Signal inversion system

- Memory Function

Memory is a common basic function.  It causes a component to stay at a certain state permanently until a signal change happens.  The following figure shows a circuit with a memory function.  When control valve  is operated momentarily (i.e. being pressed once), the output signal of the 5/2 directional control valve  will be set as ON.  The signal will stay unchanged until control valve  is being operated momentarily, instantaneously generating another signal switch and causing the signal to change to OFF permanently.

The operation of a double acting cylinder will be introduced later.  We will see that when control valve  is being pressed, the piston rod of the cylinder will extend.  The piston rod will retract only when control valve  is being pressed.

Output OFF

Output ON

Memory function circuit

- Delay Function

The function of a pneumatic delay circuit is to delay the starting time of the operation of the next control valve.  Its working principle involves the use of a flow restriction valve to slow down the flow of air, so as to control the time of the pneumatic operation.  Delay functions can be categorised into ON-signal delay and OFF-signal delay.

(i) ON-signal Delay

The following figure shows an ON-signal delay circuit diagram.  It delays the output of air pressure (ON) of the next control valve.  When control valve  is being operated, the one-way flow restriction valve will slow down the flow of air, thus delaying the signal output at the outlet of control valve  at (A).  This process will bring about a time delay in the output of an ON signal.  The time for contr ol valve  to return to its original position is not affected.

Output

One-way flow restriction valve

A

ON-signal delay circuit

(ii) OFF-signal Delay

The following figure shows an OFF-signal delay circuit diagram.  It delays the release of air (OFF) of the next control valve.  This circuit is similar to that of an ON-signal delay, only that the one-way flow control valve is connected in a reversed direction.  Therefore, when control valve  is being operated, the outlet of control valve  at (A) will continue to output signals as normal.  However, when control valve  is about to return to its original position, its release of air will be slowed down by the one-way flow restriction valve, thus causing a time delay in the output of an OFF-signal.

Output

One-way flow restriction valve

OFF-signal delay circuit

- Control of Single Acting Cylinder

Single acting cylinders can be controlled manually.  Nevertheless, they can also be controlled by two or more valves.  This method is known as logic control.  Examples of logic control include OR function, AND function and NOT function.

(i) Direct Control and Speed Control

If a single acting cylinder is connected to a manual 3/2 directional control valve, the operation of this control valve will trigger the operation of the cylinder.  Hence, this circuit allows the cylinder to be controlled manually.

One-way flow restriction valve

Manual operation

Direct control of a single acting cylinder

Apart from controlling the cylinder to operate, if we want to alter the extension speed of the piston rod of this single acting cylinder, the only way is to restrict the flow of air at the inlet and use the spring to determine the retraction speed.  Therefore, a one-way flow restriction valve is placed in the circuit to control the speed.

(ii) OR Function

The single acting cylinder in the following figure can be operated by two different pathways – manual operation or relying on automatic circuit signals.  When either control valve  or control valve  is operated, the cylinder will be actuated.  Therefore, the circuit in the figure possesses an OR function.  If the output ports of the two 3/2 directional control valves are connected through a three-way connector, the air from one control valve will escape through the exhaust pipe of the other control valve that is not operating, so that the cylinder will not function.  This problem can be solved by connecting a shuttle valve to the three-way connector.

Shuttle valve

OR function circuit

(iii) AND Function

AND function is also called interlock control.  It refers to a pneumatic operation that will only be actuated when the specified conditions are satisfied.  A typical example is that a pneumatic press will start to work only when its manual control valve is being operated while its safety door is closed.  The flow passage will open only when both control valves are being operated.  The following figure shows the circuit diagram of an AND function.  Only when both valves  and  are operated will the cylinder begin to function.

AND function circuit

(iv) NOT Function

NOT function is also called inverse operation.  For example, for a conveyor or other machines to stop and become locked during operation, the cylinder must be locked until a signal for releasing the lock is received.  Therefore, the signal for releasing the lock should be operated by a normally open (N.O.) control valve.  When releasing the lock, the same signal must also release the locks on other devices such as indication signal  in the following figure.  The following figure shows how to use normally closed (N.C.) control valve  to cut off N.O. control valve  for the purpose of signal switching.

NOT function circuit

(f) Control of Double Acting Cylinder

(i) Direct Control

The only difference between a double acting cylinder and a single acting cylinder is that the former uses a 5/2 directional control valve instead of a 3/2 one.  When a double acting cylinder is not operating, port B and inlet P are usually interconnected.  In this circuit, whenever the manual operation button is pushed, the piston rod of the double acting cylinder will move back and forth once.

Direct control of a double acting cylinder

In order to control the movement speeds in both directions independently, flow restriction valves can be connected to the inlets on both sides of the cylinder.  The directions of the flow restriction valves are opposite to that of the release of air by the flow restriction valve of a single acting cylinder.  Compared to a throttling air supply, a flow restriction valve is more rigid and stable.   Connecting a circuit in this way can provide sufficient air pressure and energy to drive the piston.

(ii) Independent Control of Speed (Bi-stable Control)

A cylinder has to maintain its position in a lot of circumstances, even after the operation signal has disappeared.  To achieve this purpose, a circuit with a memory function can be employed.  In the following figure, the extension of the piston rod of the double acting cylinder is actuated by control valve  and is brought back to its retracted position by control valve .  Control valve , maintains the position of the piston rod by keeping its own position.  The status of control valve  will only be changed when either of its manual control valves, control valves  and , is pushed.  If both of them are operated at the same time, control valve  will remain in its original position as same amounts of pressure are exerting on it.  The speed of extension and retraction can be regulated by two flow restriction valves.

Circuit of a double acting cylinder for maintaining the position

The following is a more complex design

(iii)  Automatic Return of Double Acting Cylinder

One of the two manually operated 3/2 N.C. valves can be replaced by a roller actuated valve.  It is placed at the position where the outstroke of the piston rod of the cylinder ends.  When valve 1 is pressed momentarily, the piston rod of the cylinder will extend.  At the end of its outstroke, the piston rod will touch and trigger valve 2, and will then retract automatically.

Valve 1

Valve 2

Automatic control of a double acting cylinder

- 5 Examples of Application of Pneumatic Systems

Pneumatic systems can be applied to a wide range of areas.  The following are some industrial and everyday life examples.

(a) Feeding System

The figure on the left-hand side below shows a simplified industrial pneumatic feeding system.  When the switch is pushed, the feed cylinder will transfer one workpiece from the feeding rack onto the conveyer belt.  When the switch is released, the cylinder will retract automatically.  The figure on the right-hand side below shows the circuit diagram of this pneumatic system.

Material

Retractable arm

Cylinder

| - Operation of a pneumatic feeding system | - Pneumatic circuit diagram |
| --- | --- |

(b) Door Control System

The pneumatic system shown in the figure on the left-hand side below can be used to control the opening and closing of the doors of public vehicles.  Assume that the opening and closing of the doors are controlled by an ON button and an OFF button separately.  Pressing the ON button will open the doors; pressing the OFF button will shut the doors.  The figure on the right below shows a pneumatic system that can be used to control the doors of vehicles.

Control the opening of the doors (ON)

Control the closing of the doors (OFF)

| - Operation of a pneumatic door control system | - Pneumatic circuit diagram |
| --- | --- |

- 6	Safety Precautions When Using Pneumatic Control Systems

- Compressed air can enter human ducts such as oral cavities and ears, and causes serious injury.

- Never spray compressed air onto anyone.

- At high temperatures, compressed air can even penetrate human skin.

- Compressed air released from the exhaust contains particles and oil droplets, which can cause damage to eyes.

- Even though the pressure of the compressed air is relatively lower in pipes and reservoirs, violent explosions may still occur when a container has been damaged.

- Before switching on the compressed air supply unit, one must thoroughly inspect the entire circuit to check for loose parts, inappropriate pressure, damaged pipes, etc.

- A loose pipe (especially a long one) will swing violently due to the high-pressure air inside it and is thus dangerous.  Therefore, before increasing the system pressure, the entire circuit should be inspected thoroughly to prevent accidents.

- Pneumatic cylinders generate relatively large force and fast movement.  Getting hit by the moving parts of the cylinders may result in serious injuries.

- On/Off valves should be installed on compressed air supply units to allow easy and speedy control of air flow.

- In case a leakage happens, the compressed air supply unit should be shut off immediately.

- The compressed air supply unit must be shut off before making any change to a circuit.

- Stay clear of the moving parts of a system.  (Never try to move the driving parts in mechanically operated valves with your hands.)

- 7 Glossary of Terms

| Logic gate |  | 邏輯門 | Pneumatic control |  | 氣動控制 |
| --- | --- | --- | --- | --- | --- |
| Operational amplifier |  | 運算放大器 | Pneumatic circuit |  | 氣動迴路 |
| Timer |  | 計時器 | Compressor |  | 壓縮機 |
| Analogue switch |  | 模擬開關 | Regulator |  | 調壓器 |
| Memory |  | 記憶體 | Control valve |  | 控制閥 |
| Microcontroller |  | 微控制器 | One-way valve |  | 單向閥 |
| Wafer |  | 晶圓 | Flow restriction valve |  | 流量控制閥 |
| Through-hole package |  | 通孔封裝 | Shuttle valve |  | - 梭動閥 |
| Surface-mount package |  | 表面裝配封裝 | Single acting cylinder |  | 單作用氣缸 |
| Monostable mode |  | 單穩態模式 | Double acting cylinder |  | 雙作用氣缸 |
| Astable mode |  | 非穩態模式 |  |  |  |

- 8 Interactive Information

|  | Website | Content |
| --- | --- | --- |
| 1. | http://www.ti.com/lit/ds/symlink/lm555.pdf | 555 timer data sheet |
| 2 | http://eedept.ncue.edu.tw/study/labs/semiLab/Experiment_Data/Electron_Experiment_handouts/Unit_09.pdf | 555 timer experiment |
| 3. | http://web.nuu.edu.tw/~tzen/99ECLAB202_opamp.pdf | Introduction to 741 operational amplifier |
| 4 | http://www.technologystudent.com/pics/picdex1.htm | Learning material on microcomputer control |
| 5 | http://www.technologystudent.com/comps/compdex.htm | Learning material on computer control |
| 6 | http://www.smchk.com.hk/smc_website/elearn/elearnmain.htm | Learning materials on pneumatics by SMC |
| 7 | http://www.festo-didactic.com/int-en/ | Learning materials on pneumatics by Festo |
| 8 | http://www.fischertechnik.de/en/PortalData/1/Resources/didactic/documents/activity-booklet/ROBOPneuVac/Chinese.pdf | Pneumatic kits by Fischtechnik |
| 9 | http://www.spiraxsarco.com/tw/navigation/prod_overview.asp?grp_ref=4&sec_ref=1 | Spiraxsarco – a pneumatic product company |

Exercise

Recommended steps for the following exercise:

- Realise the needs.

- Figure out the logic of the pneumatic circuit.

- Determine the components required.

- Design the pneumatic circuit diagram (speed control of the piston rod can be added during the revision of the design).

- Use software to simulate the circuit.

- Use construction kits to simulate the pneumatic control.

- Draw the pneumatic circuit diagram that has passed the test.

- The figure on the right shows an automatic stamping machine.  It comprises mainly a single acting cylinder and a manual push button.  When the button is pressed, the piston rod of the cylinder will extend and the stamp will be driven downwards, stamping on the letter inserted.  Releasing the button will get the piston rod returned to its original position to prepare for the next use.  Design a pneumatic circuit diagram based on the above requirements.

- The figure on the right shows a pneumatic jaw device.  It comprises mainly a double acting cylinder, two manual push buttons and other necessary valves.  When button  is pressed, the piston rod of the cylinder will extend and the jaws will open quickly.  The mouth is left open until button  is pushed, when the piston rod of the cylinder will retract slowly and the jaws will close gradually until the piston rod returns to its original position.  Then the device is ready for the next use.  Design a pneumatic circuit diagram based on the above requirements.

A pneumatic exercise carried out by YLPMSAA Tang Siu Tong Secondary School students